3GPP TSG-RAN WG4 meeting #81  	R4-1610464

# Reno, Nevada, 14 – 18 November, 2016

Source: 	Ericsson

Title:  	TP for TR 36.789 v0.0.2: Tools and approach for Multi-node tests

Agenda Item:	7.1.2

Document for:	Approval

# Background

During RAN plenary in June 2016, a new study item has been approved in [1]. A new TR number is assigned for this SI, which is TR 36.789.

A skeleton TR is approved in RAN4#80 in [2]. In this update, two approved TPs from RAN4#80 was incorporated, as listed in [3] and [4]. TR36.789v0.0.2 was approved in [5], which includes [3] and [4].

In this TP, we propose to several changes in Section 5, related to tools and approaches for the tests.

# TP for TR38.803

We propose to remove Section 6.3, as shown below:

<<<<<<< Start of TP >>>>>>>>>

# 5	Tools and approach for Multi-node tests

<Editor’s note: Briefly describe the scope, approach and methodologies for multi-node tests design, including basic test design principles >

## 5.1	Basic principles

<Editor’s note: Include contents related to test setup, connections and interfaces >

Some basic principles to design the tests are listed as below:

- Tests should be conductive tests to easily control and repeat the tests considering the test feasibility.

- Performance of single 20MHz channel should be tested in this version of the technical report.

- Only channel access priority class capabilities which are declared by the LAA node should be tested for the LAA node. Maximum of two representative priority classes, e.g. Priority class 3 and Priority class 1 should be tested.

- The metrics to be used are FFS.

## 5.2	Test setup

### 5.2.1	Test setup

Test setup is given in Figure 5.2.1-1, in which

•	Node A can be bidirectional and B is downlink only

•	Node C and D are the companion devices attached to Node A and B, respectively

•	Link A-C and Link B-D represent the wanted signals

•	Link A-D, B-C and Link A-B represent interfering links

Node A and B can be IEEE 802.11 AP or LAA eNodeB. Node C and D can be IEEE 802.11 STA or LAA UE.

All the links are assumed to be connected at the antenna port, i.e. the multi-node test described in this TR are conductive tests.

Note that for LAA, both licensed and unlicensed band should be connected in the test setup for link B-D.

ATT1

Companion victim device, or Node C

TX/RX

TX/RX

Aggressor device (DUT), or Node B

TX/RX

Victim device, or Node A

Iagg-vic

Ivic-agg

ATT2

ATT3

Companion aggressor device, or Node D

TX/RX

SIRvic

SIRagg

ATT4

ATT5

ATT6

Figure 5.2.1-1 Test setup for the multi-node test

Test scenarios and different traffic combinations are as shown in Table 5.2.1-1 .

Table 5.2.1-1 Test scenarios for Rel-13

| Scenario | Case | Victim DUT | Companion victim DUT | Aggressor DUT | Companion aggressor DUT | Traffic in victim link | Traffic in aggressor link |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Baseline (1a) | 802.11 AP | 802.11 STA | 802.11 AP | 802.11 STA | Best effort | Best effort |
|  | Test (1a) | 802.11 AP | 802.11 STA | LAA eNB | LAA UE | Best effort | Best effort |
|  | Baseline (1b) | 802.11 AP | 802.11 STA | 802.11 AP | 802.11 STA | Voice | Best effort |
|  | Test (1b) | 802.11 AP | 802.11 STA | LAA eNB | LAA UE | Voice | Best effort |
| 2 | Baseline (2a) | 802.11 AP | 802.11 STA | 802.11 AP | 802.11 STA | Voice | Voice |
|  | Test (2a) | 802.11 AP | 802.11 STA | LAA eNB | LAA UE | Voice | Voice |
|  | Baseline (2b) | 802.11 AP | 802.11 STA | 802.11 AP | 802.11 STA | Best effort | Voice |
|  | Test (2b) | 802.11 AP | 802.11 STA | LAA eNB | LAA UE | Best effort | Voice |
| 3 | Baseline (3a) | LAA eNB | LAA UE | LAA eNB | LAA UE | Best effort | Best effort |
|  | Test (3a) | LAA eNB | LAA UE | 802.11 AP | 802.11 STA | Best effort | Best effort |
|  | Baseline (3b) | LAA eNB | LAA UE | LAA eNB | LAA UE | Voice | Best effort |
|  | Test (3b) | LAA eNB | LAA UE | 802.11 AP | 802.11 STA | Voice | Best effort |
| 4 | Baseline (4a) | LAA eNB | LAA UE | LAA eNB | LAA UE | Voice | Voice |
|  | Test (4a) | LAA eNB | LAA UE | 802.11 AP | 802.11 STA | Voice | Voice |
|  | Baseline (4b) | LAA eNB | LAA UE | LAA eNB | LAA UE | Best effort | Voice |
|  | Test (4b) | LAA eNB | LAA UE | 802.11 AP | 802.11 STA | Best effort | Voice |

### 5.2.2	Test approach

To verify co-existence performance between different systems, e.g. LAA to IEEE 802.11 and IEEE 802.11 to LAA, each test consists of two steps:

- Baseline test: The performance between the same systems should be tested first and recorded as the baseline.

- Coexistence test: Replace one IEEE 802.11 AP/IEEE 802.11 STA with an LAA BS/LAA UE respectively if IEEE 802.11 is a victim or one LAA BS/LAA UE with a IEEE 802.11 AP/IEEE 802.11 STA respectively if LAA is a victim and redo the test and record the performance. That means victim IEEE 802.11 metrics in the scenario of IEEE 802.11 to IEEE 802.11 should be the baseline for the scenario of LAA to IEEE 802.11; while victim LAA metrics t in the scenario of LAA to LAA should be the baseline for the scenario of IEEE 802.11 to LAA.

The set of devices which can be used to define the baseline in step 1. is described in section 5.2.4.

### 5.2.3	Number of nodes

One DL/UL IEEE 802.11 AP and one DL LAA BS should be considered in a test. Each node (LAA BS/IEEE 802.11 AP) connects to only one client node (UE/STA) in the test setup.

### 5.2.4	Device selection for baseline performance determination

For scenario 1, the baseline is created by selecting a specific number of APs and STAs.

- The tests shall ensure repeatability and representative operational behavior.

- The starting point of the configuration of both 802.11 and LAA equipment shall be based on the default settings.

- In order to achieve the test objectives of repeatability and representative operational behavior, modifications to the default settings might or might not be needed

- The above modifications, if needed, are FFS

- Modifications compared to the defaults settings, if any, shall be documented

The list of allowed devices which shall be used to create the baseline in scenario 1 is FFS.

### 5.2.5	Test signal levels

The test signal level to be adopted in the test shall be specified as follow:

The coupling losses in Link A-D, Link B-C and Link A-B in Figure 5.2.1-1 shall be set in such a way that:

- The received signal level from Node A to Node D is X dBm (interfering link)

- The received signal level from Node B to Node C is X dBm (interfering link)

- The received signal level from Node A to Node B is X dBm (interfering link)

The coupling losses for Link A-C and Link B-D shall be set in such a way that:

- The received signal level from Node A to Node C is X+Y dBm (wanted signal)

- The received signal level from Node B to Node D is X+Y dBm (wanted signal)

Y is the ratio between the wanted signal and the interfering signal. Y shall be set to [15] dB.

The Received Signal levels to be used in the tests are  as follows:

- -67dBm/20MHz

- -82dBm/20MHz

## 5.3	Test equipment

<Editor’s note: Include contents related to parameters and descriptions for devices under tests >

### 5.3.1	Measurement equipment

### 5.3.2	LAA eNB and UE device configurations

### 5.3.3	WiFi AP and STA device configurations

<<<<<<< Start of TP >>>>>>>>>

# Proposal

We propose to accept the above TP for TR38.803.

# References

RP-161197, New Study Item proposal: Multi-node tests for LAA, Huawei, HiSilicon, Ericsson, Qualcomm, Nokia

R4-166543, "Skeleton TR 36.789 for multi-node tests", Ericsson.

R4-166969, TP on Scope of TR 36.789: multi-node coexistence test, Source: Ericsson

R4-166970, TP for multi-node testing TR: Section 4 and 5, Source: Huawei, Ericsson

R4-168544, Updated TR TR 36.783 v0.0.2: Multi-node tests for LAA, Source: Ericsson