3GPP TSG-RAN WG4 Meeting #81	R4-1610509

# Reno, Nevada, USA, 14 - 18 November, 2016

Source:	KDDI Corporation

Title:	TP for TR36.714-02-02: UE co-existence table for eLAA example band combinations

Agenda item:	8.18.2

Document for:	Approval

Introduction

In RAN4#79, [1] was approved as way forward of eLAA work.  One of proposals in [1] includes;

Proposal#3: UE co-existence table for 2UL CA including Band 46 should be also captured into TR36.714-02-02.

UE co-existence table for 2UL CA is well-known as complex table.  Therefore, initial specification sometimes includes errors.  Then, RAN4 and RAN need additional CRs for bug fix.  In order to reduce this kind of editorial workload, Proposal#3 in [1] was made.  This contribution provides TP for TR36.714-02-02 to capture UE co-existence table for 2UL CA including Band 46.

Proposal

We propose to approve TP described in section 4.

Reference

- 1. 	R4-164676 “How to handle eLAA example combinations”, KDDI Corporation

TP for TR36.714-02-02

<Start of TP>

### X.Y	UE co-existence table for 2UL CA including Band 46

This clause specifies the requirements for the specified carrier aggregation configurations including Band 46 for coexistence with protected bands.

Table X.Y-1: Requirements for uplink inter-band carrier aggregation (including band 46)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_1A-46A | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 65, 67, 68, 69 |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | Frequency range |  | 1839.9 | - | 1879.9 | -50 | 1 | 3 |
|  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 3, 6 |
|  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 3, 6, 7 |
|  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 3, 6, 7 |
| CA_2A-46A | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 66, 70 |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_3A-46A | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 65, 67, 68, 69 |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 11, 18, 19, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 22, 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_7A-46A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 65, 66, 67, 68 |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 3, 7, 8 |
|  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 3, 7, 8 |
|  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 3, 8 |
| CA 41A-46A | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 65, 66, 70 |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9 |
|  | Frequency range |  | 1839.9 |  | 1879.9 | -50 | 1 | 9 |
|  | Frequency range |  | 1884.5 |  | 1915.7 | -41 | 0.3 | 4, 9 |
| CA_42A-46A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 65, 66, 67, 69 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
| CA_46A-66A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 66, 70 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 5: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 9: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz |  |  |  |  |  |  |  |  |

<End of TP>