3GPP TSG-RAN WG4 Meeting #81 	R4-1610628

# RENO, NEVADA, USA NOVEMBER 14 – 18, 2016

Agenda item:	11.1

Source: 	NTT DOCOMO, INC.

Title: 	Evening AH minutes for NR on Wednesday

Document for:	Approval

Introduction

The contribution summarizes minutes of evening AH meeting where the following topics are handled.

- Co-existence simulation study (7:00 ~ 8:00 PM)

  - Simulation assumptions

  - How to derive ACLR/ACS from ACIR

- RF parameters not related to Co-existence simulation (8:00 ~ 9:00 PM)

Discussion

Simulation assumptions

Discussion:

Simulation assumptions for urban macro

- Assumptions and parameters:

  - 200 MHz B/W

  - 300 m ISD

  - 0% indoor user ratio (assuming indoor users are served by small cells at 30 GHz)

- Alternative

  - DL: ISD=200m, CBW=200MHz, indoor UE ratio =20%

  - UL: ISD=200m, CBW=200MHz, indoor UE ratio =20%,

Samsung: Question to Nokia, Do you have any ACIR output?

Nokia: Try to avoid doing this way. You can do the simulation.

Qualcomm: We have concern on 0%. Need offline discussion based on alternative.

Nokia: Can Qualcomm study again?

QC: This is not my proposal but from operator. If operator prefers 0%, then no strong opinion.

Huawei: We should keep RAN1 assumption. We are OK with smaller parameter. But we cannot remove simulation parameters.

Nokia: Fine with alternative.

Samsung: Support alternative. Alternative assumption can achieve realistic ACIR value.

Nokia: if operators are OK, we are OK. In alternative, 5%-til UE is still outage.

Samsung: in our simulation results, 5%-tile UE is not outage.

Ericsson: if there are no objection, alternative 2 should be ageed.

Huawei: in Alternative, ISD should be 300m. 200m is not urban macro.

Nokia: there is only one company having objection. We don’t want to do same discussion tommowr

Huawei: still have concern.

In Urban macro, coordinated or not (100% grid shift)?

Samsung: OK

Status:  need further discussion.

How to determine ACLR/ACS from ACIR

Discussion:

Ericsson: agree with the WF

Qualcomm: Generally OK, What do you intend to “equal or less than”.

Intel: Average among companies?

Verizon: “less than” should be removed.

Status:  need further discussion.

RF parameters not related to co-existence

Duplex Method

Session chair memo

Option 1: TDD

Option 2: TDD. FDD is FFS

Option 3: no answer (up to RAN plenary)

Discussion:

Possible WF: FFS

Ericsson: Try to capture answer in WF in this meeting. For ITU, Op1, for RAN4, Op2.

DOCOMO: can we just go to Op2? This is middle way. FFS in LS should be avoided. FDD can be discussed further in RAN4.

Chair: can we agree Op2?

Ericsson: No

DOCOMO: let us clarify my intention. TDD is capture in LS. In RAN4 WF, it is captured that FDD is FSS.

Verizon: in mmWave, are there any requests for FDD?

Ericsson: So far, not available

Agreement:

Answer to ITU: TDD

WF in RAN4: FDD and SDL can be further studied.

Status:

Channel bandwidth (MHz)

Session chair memo

It was discussed whether we should answer single channel bandwidth or not.

It seems that most companies are OK with single channel bandwidth.

200 MHz and […, 80MHz, …] were proposed.

Discussion:

Possible WF: channel bandwidth = 200MHz

Huawei: clarify whether single carrier or single value. We don’t agree channel bandwidth. CA can be used.

Ericsson: could be CA, but need clear view.

Samsung: need clear view. If we discuss CA case, more complicated. CBW should be single carrier.

Huawei: if follow Ericsson’s comment, need to change terminology.

Vodafone: bandwidth used by system

Samsung: our preference is 200M in single carrier. And currently we use 200MHz single carrier in our co-existence study.

Ericsson: whether system BW or not does not have impact on ITU-R study

DOCOMO: Response to ITU is 200MHz only. And we clearly mention “how to realize BW is further study” in RAN4 WF

Intel: similar view with DCM.  In RAN4 WF, it mention “CBW is further study”

Huawei: ITU need only BW. Should separate BW to ITU and single carrier discussion.

Huawei: need to put note for information that RAN4 have not finished yet.

KT: has concern about “system band width”.

DOCOMO: alternative from HW: “note 200M is assumption for RAN4 co-ex study”

Nokia: ITU does not need note.

Docomo: in general, there parameters are the same situation. Then general note for all parameters. We can share our status to WP5D.

Huawei: not generic note but specific note.

WF:

Each company need to bring a generic note to share RAN4 status to ITU-R.

Then if we agree the generic note, then we agree 200MHz as ITU-R response.

-----

Response to ITU-R: 200MHz

WF in RAN4: Single carrier channel BW is FFS

Status:

Signal bandwidth (MHz)

Session chair memo

Followings were proposed.

[90% of channel bandwidth]

>90 % of channel bandwidth

Equal to channel bandwidth.

Discussion:

Possible WF: >90 % of channel bandwidth. Details value will be updated based on outcome of spectrum utilization discussion in AH meeting.

Status:

BS side

Power dynamic range (dB)

Session chair memo

It seems that most companies are OK with no DL power control

Since ITU-R has their own antenna configuration assumptions, we can answer conducted based power rather than OTA based one.

Discussion:

Possible WF: 0 dB

Nokia: Don’t disagree.  We don’t need to define minimum transmit power.

Intel: firstly we should need definition.

Ericsson: This is not for requirement but for sharing study. From BS side, always same power.

Intel: is this conducted power or OTA?

Ericsson: in this case, conducted power.

Agreement

we answer conducted based power based on ITU-R assumption

we don’t need power dynamic range in BS side

Status:

Spectral mask, ACLR, Spurious emissions

Session chair memo

Under discussion in RF session (SEM and spurious emission) and Co-existence session (ACLR)

Should we answer these parameters based on the specific CBW (e.g 200M)?

Discussion:

Status:

Noise figure

Session chair memo

30GHz: 9 or 11 dB

45GHz: 11 or 13 dB

70GHz: 13 or 15 dB

Discussion:

Status:

Sensitivity

Session chair memo

It was discussed that ITU-R may not use this value for their sharing study.

Discussion:

Possible WF: In Jan. AH meeting, this parameter is low priority

Status:

Blocking response

Session chair memo

In this meeting, no companies proposed blocking response.

Discussion:

Possible WF: In Jan. AH meeting, this parameter is low priority

Status:

ACS

Session chair memo

ACS is derived from simulation so we will skip in this ad hoc meeting.

SINR operating range

Session chair memo

Regarding the question from RAN4 “that the possible SINR range is very large and it is not clear what aspect of SINR that is requested”, the answer of WP5D is that the meaning of SINR range is in fact a mapping table between throughput and SINR for IMT-2020 in order to simulate IMT-2020 throughput loss due to external interference, in a form similar to Tables A.6 and A.7 found in TR 36.942. In case this type of mapping table would be available, WP5D would welcome that information.

Table in TR 36.942

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SINRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SINRMAX, dB | 22 | 15 | Based on 64QAM 4/5 (DL) & 16QAM 3/4 (UL) |

For NR, following SINR vs throughput model is assumed for WP5D simulation in RAN4.

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SINRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SINRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Discussion:

Possible WF: -10 ~ 30 (for DL)

UE side

Power dynamic range (dB)

Session chair memo

Since ITU-R has their own antenna configuration assumptions, we can answer conducted based power rather than OTA based one.

Discussion:

Possible WF: 63 dB

Status:

Spectral mask, ACLR, Spurious emissions

Session chair memo

Under discussion in RF session (SEM and spurious emission) and Co-existence session (ACLR)

Should we answer these parameters based on the specific CBW (e.g 200M)?

Discussion:

Status:

Noise figure

Session chair memo

30GHz: 9 or 11 dB

45GHz: 11 or 13 dB

70GHz: 13 or 15 dB

Sensitivity

Session chair memo

It was discussed that ITU-R may not use this value for their sharing study.

Discussion:

Possible WF: In Jan. AH meeting, this parameter is low priority

Status:

Blocking response

Session chair memo

In this meeting, no companies proposed blocking response.

Discussion:

Possible WF: In Jan. AH meeting, this parameter is low priority

Status:

ACS

Session chair memo

ACLR is derived from simulation so we will skip in this ad hoc meeting.

SINR operating range

Session chair memo

Regarding the question from RAN4 “that the possible SINR range is very large and it is not clear what aspect of SINR that is requested”, the answer of WP5D is that the meaning of SINR range is in fact a mapping table between throughput and SINR for IMT-2020 in order to simulate IMT-2020 throughput loss due to external interference, in a form similar to Tables A.6 and A.7 found in TR 36.942. In case this type of mapping table would be available, WP5D would welcome that information.

Table in TR 36.942

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SINRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SINRMAX, dB | 22 | 15 | Based on 64QAM 4/5 (DL) & 16QAM 3/4 (UL) |

For NR, following SINR vs throughput model is assumed for WP5D simulation in RAN4.

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SINRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SINRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Discussion:

Possible WF: -10 ~ 22 (for DL)

Status: