Source:		Apple, Intel, Oppo, Xiaomi

Title:	Input for defining a suitable framework for setting TRP/TIS (TRS) limits

Agenda Item:	7.14

Document for:	Approval

# Summary

This contribution will try to discuss the following with regards to setting limits for LTE TRP/TRS:

- Problem statement driving setting of the limits for LTE TRP/TRS

- Metrics which will be used to understand whether the defined LTE TRP/TRS addresses the problem

- Factors which influence choice of limits for LTE TRP/TRS

Some of the arguments mentioned in this contribution have also been raised in previous contributions; [1-6]

Problem Statement

A contribution by the North American operators [9] lists the following reasons as justification for setting the LTE TRP/TRS limits.

“US operators have an established a minimum set of TRP and TRS criteria used on their network deployment needs. These criteria apply to phones distributed by the North American carriers.”

We believe that the committee needs to consider the following to fully understand the scope of limit definition.

- Understand if the guidance is regarding existing LTE devices to improve shortage of performance or to set a guideline for future devices?

- The proposed LTE radiated performance should ensure a satisfactory field performance of a device

- The TRP/TRS requirements agreed by RAN4 should conform with the scope of OTA requirements defined in TS 37.144 [8]:

Handheld UE requirements are defined for roaming bands for the speech position (beside the head and beside the head and hand) and hand phantom browsing mode position. Laptop mounted equipment requirements are defined for roaming bands for the data transfer position (laptop ground plane phantom). Laptop embedded equipment requirements are defined for roaming bands for the data transfer position (free space).

All bands are potential roaming bands, and the requirements for roaming bands shall therefore be fulfilled for all bands supported by a UE/MS.

Requirements for operating bands are dependent on how the network has been built and are thus operator specific and cannot be specified here. Recommended performance values for operating bands (Annex B) are however included in this specification for information. It should be recognized that the ability to meet the recommended performance values depends on the number of frequency bands supported by the UE/MS.

Discussion on Metrics

The usual process that is followed before a device is introduced on a carrier’s network can be used to guide the metrics that need to be scoped to understand if the above problem statements have been addressed.

Phantoms vs. User Performance

We understand that the CTIA Phantom grip represents a repeatable test condition for OTA but real user interaction with real devices may span a wide range of actual conditions for the device. Therefore, we should avoid stringent requirement for the standard test condition so as not to compromise performance of actual scenarios.

Roaming vs. Specific Market Requirements

The scope of radiated requirements in 3GPP RAN4 is clearly defined in [8].  Thus, the discussion should focus on defining minimum roaming requirements with the understanding that a UE undergoing OTA conformance testing per GCF shall obtain certification if the roaming requirements are satisfied for all applicable bands.

Factors which influence choice of LTE TRP/TRS

Drawback of Using One Grip for Limit Definition

Several standard bodies are currently considering using CTIA phantoms to come up with radiated performance limits, which can be used to gauge the wireless system performance of a device.  Based on our experience, we have seen that sometimes CTIA phantom performance doesn’t convey the entire picture. One of the challenges that we have seen is illustrated in Fig 1.  In this figure, we present the BHHR TRP for 4 devices (all shipping) across multiple grips taken with real users. Each user is represented by a unique grip. On the ‘x’ axis each tick mark is 1dB. We see from the mean TRP values that the 4 devices perform very similar to each other. But, if we compare the CTIA BHHR (blue dot and red diamond) the values are different by ~3.5dB. So, if we would consider the CTIA BHHR values only, we would not get the correct representation of the comparative wireless system performance of Device B and D across different users grips.

The point of raising this limitation is twofold:

- Having a stringent specification for one grip could potentially impact the performance of the device across other grips since the design might need to be tailored to address one grip while becoming overly sensitive to other use cases.

- We understand that a certification test can’t be necessarily done over multiple grips possibly due to logistical issues. But, the group should consider this limitation while deciding on a limit so that we don’t penalize wireless system design for no obvious improvement in performance.

Fig 1: Example performance for 4 shipping devices for different grips

Carrier Aggregation

We understand that the limits under consideration are for non-CA devices but a discussion for CA devices is essential for the following reasons:

- Majority of devices currently support CA and mostly the data that the ODMs might have readily available for discussion is for CA devices

- Maximum delta from non-CA limits would be different based on support of downlink or uplink CA, 2CA/3CA/4CA, inter-band CA, intra-band contiguous CA and intra-band non-contiguous CA. The team needs to at least decide on a maximum delta that they would consider from the non-CA spec.

Frequency Band Coverage

The global frequency spectrum allocated for LTE is much different than the ones used by 2G/3G terminals. Supporting bands from 700MHz to 3700MHz either for primary or roaming purpose is already very challenging considering the other HW designs (e.g., choice of material, battery size, acoustic features, camera, etc.) that need to be made for contemporary handsets.

We understand that some of the new bands are not necessarily for the North American market but having a realistic spec for a “world” device would be the best choice for the consumer. With that in mind, we would encourage the committee to consider the impact of supporting multiple bands and incorporate required relaxation for such devices.

4 X 4 MIMO

It is widely expected that carriers will be rolling out networks supporting 4 X 4 MIMO. To make the specification future proof, we should consider the effect of supporting 4 X 4 MIMO on the radiated performance.  Even though the 4 X 4 MIMO is a DL feature, supporting two additional antennas and additional RF front end in a contemporary handset will come at a cost of antenna volume and insertion loss.

The committee needs to consider this also while deciding on limits for applicable to near future devices.

LTE in Unlicensed Bands

There is considerable interest in the wireless community to support LTE in unlicensed bands (LAA/LWA/LTE-U). This could potentially have impact to RF front-end designs depending on the implementations and needs to be considered in setting of the OTA performance limits.

RF Exposure (SAR)

There are several on-going discussions in different regulatory domains on changing test methodologies for measuring RF exposure (SAR) for smartphones. For instance, in Europe, the test distance at which Body-worn SAR is measured is being changed to 5mm.Previously, the test distance could be anywhere from 10 – 25mm. These changes could potentially affect the maximum TRP that the device might be able to output to ensure that SAR limit is met.

In RAN4 #80 meeting, a WF [1] has been agreed as the starting point of TRP/TIS framework improvement studying. Among the factors mentioned on the above, some (such as CA and frequency band coverage) have been collected in [1], others (such as MIMO, unlicensed band and SAR) have not yet. Thus it is important to continue the framework improvement to collect more factors which can have impacts on the TRP/TIS limits setting. From RAN4 progress point of view, a step-by-step approach can be considered: 1st step, agree on which of factors will be considered in the initial limits proposal; 2nd step, then a way forward plan for the remaining factors.

Proposals

The following proposals are submitted for approval:

- Agreement on clear understanding of the problem statement behind the proposed LTE TRP/TRS limits.

- Agree on a common set of metrics which will be used to confirm that the problem has been addressed

- Continue framework improvement with 2-step approach: agree on which of proposed factors will be considered in the initial limits proposal and then a way forward plan for integrating the other factors

4.  Update the framework in [1] to introduce a joint band passing rate analysis of
     all proposed OTA requirements

References:

[1]	R4-167201; Way forward on LTE TRP TRS framework improvement and work plan

[2]	R4-168617; Methodology for deriving E-UTRA BHH TRP/TRS requirements.

[3] 	R4-166436; CR to TS 37.144 Applicability of UTRA Requirements

[4]	R4-166256; OTA TRP/TRS framework improvements

[5]	R4-165242; The way to define LTE TRP/TRS requirements

[6]	R4-165206; OTA TRP/TRS framework improvements

[7]    RP-161323, “LTE UE TRP and TRS and UTRA Hand Phantom related UE TRP and TRS Requirements,” Telecom Italia, Vodafone, 3GPP RAN #72, June 2016

[8]    TS 37.144, “UE and MS GSM, UTRA and E-UTRA over the air        performance requirements,” V14.2.0, December 2016

[9]    R4-1610787, “LTE UE TRP and TRS Requirements for North American Bands,” Sprint,AT&T, Verizon, SouthernLINC, Bell Canada, Dish, 3GPP RAN4 #81, November 2016