3GPP TSG-RAN WG4 Meeting RAN4#82	    R4-1700535

Athens, Greece, 13 - 17 February, 2017

Agenda item:	10.4.1.1- Spectrum utilization

Source: 	Intel Corporation

Title: 	Aspects of spectral efficiency for UE

Document for:	Discussion

# Introduction

RAN 1 sent a LS on NR waveform to RAN 4 [1] which discusses about spectrum utilization. It was agreed that RAN 4 should consider dependencies for achieving Y > 90% for NR (in LTE, Y = 90%) where Y was defined as:

Y (%) = transmission bandwidth configuration / channel bandwidth * 100%

Several studies are carried out in RAN 4 accordingly. For example, the WF in [2] concerning NR spectrum utilization. In this paper, we would like to discuss 2 major techniques concerning maximum spectrum utilization:

- Partial resource block allocation as discussed in [3] – [4]

- Spectral confinement techniques:

- Filtering (f-OFDM) as discussed in [5] – [10]

- Windowing (w-OFDM) as discussed in [11]

This paper would also like to clarify on the proposal from [9] regarding:

- UE reporting Y to network and different value of Y for different UE/BS

# Discussion on partial RB allocation

The following table was proposed in [3] to achieve higher spectrum utilization.

| SCS Bandwidth |  | 3.75kHz | 7.5kHz | 15kHz | 30kHz | 60kHz | 120kHz | 240kHz | 480kHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5MHz | NRB | 111 | 55 | 27 | 13 | 6 | 3 | 1 | - |
|  | MSU | 99.9% | 99% | 97.2% | 93.6% | 86.4% | 86.4% | 57.6% | - |
| 10MHz | NRB | 222 | 111 | 55 | 27 | 13 | 6 | 3 | 1 |
|  | MSU | 99.9% | 99.9% | 99% | 97.2% | 93.6% | 86.4% | 86.4% | 57.6% |
| 15MHz | NRB | 333 | 166 | 83 | 41 | 20 | 10 | 5 | 2 |
|  | MSU | 99.9% | 99.6% | 99.6% | 98.4% | 96% | 96% | 96% | 76.8% |
| 20MHz | NRB | 444 | 222 | 111 | 55 | 27 | 13 | 6 | 3 |
|  | MSU | 99.9% | 99.9% | 99.9% | 99% | 97.2% | 93.6% | 86.4% | 86.4% |
| 40MHz | NRB | 888 | 444 | 222 | 111 | 55 | 27 | 13 | 6 |
|  | MSU | 99.9% | 99.9% | 99.9% | 99.9% | 99% | 97.2% | 93.6 | 86.4% |
| 80MHz | NRB | 1777 | 888 | 444 | 222 | 111 | 55 | 27 | 13 |
|  | MSU | 99.96% | 99.9% | 99.9% | 99.9% | 99.9% | 99% | 97.2% | 93.6% |
| 100MHz | NRB | 2222 | 1111 | 555 | 277 | 138 | 69 | 34 | 17 |
|  | MSU | 99.99% | 99.99% | 99.9% | 99.72% | 99.36% | 99.36% | 97.92% | 97.92% |
| 200MHz | NRB | 4444 | 2222 | 1111 | 555 | 277 | 138 | 69 | 34 |
|  | MSU | 99.99% | 99.99% | 99.99% | 99.9% | 99.72% | 99.36% | 99.36% | 97.92% |

From the table above, it can inferred that for channel bandwidths (CBW) of 200 MHz (or higher) in NR frequency bands above 6 GHz (24.25-29.5 GHz) with smaller SCS (like 15 KHz), the resulting FFT size would be greater than 12000 which is un-realistic from UE perspective. Also, flicker noise (close to DC) & phase noise degradation will affect performance.

Similarly, for small CBW with large SCS, as seen from table above, BW efficiency is less than 90%, which is attributed to increase in relative size of CP. Also small CBW with large SCS is not a system level requirement.

Observation 1: Partial RB allocation is not recommended, both for small CBW with large SCS and large CBW with small SCS.

Proposal 1:  To limit burden on UE*, in context of maximum spectrum utilization, the following limits are proposed:

- For NR bands in sub-6 GHz: channel bandwidth range should be 20 MHz - 100 MHz with SCS options of 15 KHz, 30 KHz, 60 KHz

- For NR bands above 6 GHz: channel bandwidth range should be 100 MHz - 800 MHz with SCS options of 60 KHz, 120 KHz

*Over all aim must be to limit the number of numerologies for both sub-6 GHz and above 6 GHz NR bands but still achieve higher spectrum utilization. Implementing multiple SCS for a fixed CBW should be jointly decided with RAN 1 by allocating first priority to UE base-band implementation complexity.

# Discussion on spectral confinement techniques

For achieving parameter Y >90 (in %), in RAN 1 and RAN 4, two major spectral confinement techniques are discussed: filtered OFDM and windowed OFDM.

Extensive study has been made in [5] – [10] to propose f-OFDM technique as a part of NR uplink and downlink procedures. But, from implementation point of view, f-OFDM has higher complexity and introduces delay during spectral shaping to meet requirements like SEM, ACLR and EVM. Truncation method proposed in [7] to counter the delay overhead adds to complexity. From implementation complexity, w-OFDM could be less critical than f-OFDM [11]. As mentioned in [12]:

- Filtered-OFDM may achieve higher Y% than w-OFDM only in the case of narrow contiguous RB allocations. For wider contiguous RB allocations, both methods achieve similar Y%.

- Filtering length should at-least be 40% of symbol length to meet EVM requirements but windowing length has minimal impact to EVM.

In [12], table 1 compares f-OFDM and w-OFDM from 5 MHz until 20 MHz CBW, but for NR it is important to study CBWs up to 800 MHz.

Observation 2: Windowing performs better in terms of EVM, which makes it an attractive candidate for higher order data modulation schemes. Further study on filtering techniques, with focus on NR, is required [13].

Proposal 2: RAN 4 should not specify any spectral confinement technique within UE (or) BS to achieve higher spectrum utilization. The methods used for uplink (or) downlink should be kept transparent in order to meet SEM, EVM and ACLR requirements.

# Discussion on UE reporting Y to network and different Y for different UE/BS

The following excerpts are taken from [9]:

- “RAN4 should allow different value of Y for different BS/UE capabilities with compliance of related RF requirements, e.g. EVM, ACLR, SEM, etc.”

- “UE capability in terms of Y should be reported to network when it accesses network”

First problem evident with such a technique is the signalling overhead in the system & BS/UE processing complexity. RAN 4 should discuss and study further the merits and de-merits of such an approach.

Proposal 3: RAN 4 should try to avoid additional reporting mechanism from UE/BS with respect to Y parameter.

# Conclusion

Observation 1: Partial RB allocation is not recommended, both for small CBW with large SCS and large CBW with small SCS.

Proposal 1:  To limit burden on UE, in context of maximum spectrum utilization, the following limits are proposed:

- For NR bands in sub-6 GHz: channel bandwidth range should be 20 MHz - 100 MHz with SCS options of 15 KHz, 30 KHz, 60 KHz

- For NR bands above 6 GHz: channel bandwidth range should be 100 MHz - 800 MHz with SCS options of 60 KHz, 120 KHz

Observation 2: Windowing performs better in terms of EVM, which makes it an attractive candidate for higher order data modulation schemes. Further study on filtering techniques, with focus on NR, is required [13].

Proposal 2: RAN 4 should not specify any spectral confinement technique within UE (or) BS to achieve higher spectrum utilization. The methods used for uplink (or) downlink should be kept transparent in order to meet SEM, EVM and ACLR requirements.

Proposal 3: RAN 4 should try to avoid additional reporting mechanism from UE/BS with respect to Y parameter.

# References

- R4-167218, “LS on NR waveform”, RAN 1

- R4-1610922, “Way forward on NR spectrum utilization”, Huawei, Hisilicon

- R4-1609662, “Considerations on NR spectrum utilization and usage of fractional RB”, ZTE

- R4-1700009, “Considerations on NR spectrum utilization and guard band using fractional PRB”, ZTE

- R1-165425, “f-OFDM scheme and filter design”, Huawei

- R4-1700132, “Evaluation on the complexity of NR waveforms”, Huawei

- R4-1700133, “Evaluation on the delay overhead analysis for NR waveforms”, Huawei

- R1-166093, “Waveform evaluation updates for case 1a and case 1b”, Huawei

- R4-1609411, “Further discussion on spectrum utilization”, Huawei

- R4-1609412, “Considerations on different guard band for mixed numerologies”, Huawei

- R1-163224, “Feasibility of Mixing Numerology in an OFDM System”, Ericsson

- R4-1700099, “Spectral Utilization Analysis for NR”, Qualcomm

- R1-1612167, “Further considerations on NR waveform with improved spectral containment”, Intel Corporation.