3GPP TSG-RAN4#82	R4- 1700604

Athens, Greece, 13 – 17 Feb, 2017

# Source:	Intel Corporation

# Title:	UE reference architecture in mmWave

# Agenda Item:	10.4.2.1

# Document for:	Discussion

# 1	Introduction

In the previous meetings there have been discussions about the suitable UE reference architecture for NR. For high frequency broadband systems - especially at mmWave range-, the nonlinearity of analogue components used in RF front ends gives increased challenges in the implementation. The principal impairments at RF front end of the transceiver architecture and the key requirements need to be analysed for achieving high performance, which will be outlined in this contribution.

# 2	Discussion

The RF Frontend consist of all components between the antenna and the digital baseband system of a transceiver, which is divided mainly in mixers, RF Phase shifter and power amplifier. The demand of high data rate increases the research on conflicting requirements. Furthermore, high bandwidth requires operation in high frequency RF band and this creates big challenges to achieve high efficiency and good performance. The balance that must be found in size, capacity and cost should be taken into account in NR.  Therefore, it will be beneficial to define a UE reference architecture to carry out the RF specification discussions, since different architectures determine efficiency, gain and linearity of the transceiver.

## 2.1 Transceiver architectures

Wireless communication requires several stages of signal processing in digital and analogue domain. Once the signal is converted from digital to analogue, it is expose to several analogue impairment sources. Therefore, transceiver architectures selection can influence differently to the performance of the system. The transceiver architecture can be either direct (Homodyne) conversion, as shown in Figure 1 or IF (Heterodyne) conversion, as shown in Figure 2. [1]

Figure 1: Simplified architecture Direct Conversion

Figure 2: Simplified architecture one stage IF conversion

- Homodyne transceiver architecture

In a direct conversion architecture, the I and Q modulation is carried out in the analogue domain, having as a drawback the deficiency of the modulator; as for example the IQ-imbalance, DC offset and degradation of transmitted signal resulting in degradation of EVM. However, the major advantage of direct conversion architecture is that the image problem is avoided because the IF is zero, and therefore the requirement of a single LO realization is fulfilled. Another disadvantage of this architecture is that there are no other filters than RF-band selection filter, which means that the creation of a DC signal can contribute directly as interference in the band of interest.

- Heterodyne transceiver architecture

In the heterodyne architecture, the IF-sampling architecture has the modulation in the digital domain, thus becomes immune to the problems related to the I and Q modulator. Nevertheless, it requires a wide bandwidth DAC, which is more sensible to errors and require more stages of LOs and filters to eliminate the IF spectral images. The heterodyne architecture has an advantage in the phase noise requirements compared to homodyne architecture, since the phase noise is logarithmically proportional to the LO frequency having two sources of LO reduce the phase noise of the system.

The UE reference architecture in Figure 3 with IF conversion is proposed in mmWave, this does not preclude from using direct conversion at a more advance stage in NR.

Observation 1: It will be beneficial to define a UE reference architecture to carry out the RF specification discussions, since different architectures determine efficiency, gain and linearity of the transceiver.

## 2.2 UE reference architecture

Figure 3: UE Reference architecture

Besides the RF front end, it must be considered that mixers, oscillators, quadrature modulators are also sources of impairments, due to low port isolation, generating imperfections in the output signal.

Mixing the RF signal with the LO realizes frequency conversion within the RF transceivers. The phase of the LO can be non-stationary as a free running oscillator or time varying modelled as a stationary process of a PLL synthesizer. Ideally, the outcome of an LO is a single tone in the frequency domain. However, in reality, the outcome is a modulated tone with a phase shift. Phase noise – which increases with LO frequency - causes significant degradation in the performance and reduces the effective SNR at the receiver, limiting the BER and data rate. The phase noise gives several constraints on the design of oscillators. These are further constraints besides the limitations at the RF frond end – explained in the next subsections - that make the choice of a UE transceiver architecture important.

Observation 2: The UE reference architecture in Figure 3 with IF conversion is proposed for mmWave. This does not preclude from using direct conversion at a more advance stage in NR.

### 2.2.1 TX Chain

This subsection discusses the most relevant components in the RF front of end in the transmitter chain, which should be considered as key building blocks for further study in NR. The transmitter is less complex than the receiver, thus a less variety of implementations can be studied.

Power Amplifier

For transmitting large amount of data modulated in complex waveforms, a high linear amplification stage is required to minimize distortion. The drawback of high linear PA is the lack of efficiency. At higher frequency the efficiency and output power decreases. This means that for transmitting a certain amount of signal power, more power is required by the PA compared to the systems operating at carrier frequency under 6 GHz. The major imperfection of the PA is its nonlinear response and memory effects [2]. The nonlinearity problem of the PA becomes evident as requirement of larger bandwidth and higher order modulation schemes are needed to achieve high data rate.

Phase Shifter

Phase shifters are source of imbalance and they can be implemented by power divisor and all – pass filters producing 90 degree phase shift. The accuracy depends on the components so that the 90 degree phase shift is not really exact. There are different phased-array architectures (i.e., RF phase shifters, LO phase shifters) which differ in functionality of phase shifting and signal combining and have as critical factors power consumption, losses and bandwidth.

Figure 4: RF and LO Phase Shifters

LO phase shifters - compared to RF phase shifter - has a LO path, which is less sensitive to bandwidth because single tone LO is usually delivered. Bandwidth only translates to conversion gain difference in the mixer while using different LO frequencies and can be compensated pushing up the gain in the LO driver. The penalty for this is the higher power consumption, LO phase shifter has as drawback the high number of mixers. Every element needs a mixer to down convert before phase shifting.

In the case of the RF phase shifter, it needs an RF gain stage prior signal combining for noise reasons due to signal insertion loss. Additionally, it should meet tougher linearity specifications, which increases the losses of RF phase shifter. A way to cancel out the losses is to use an amplifier stage. The bandwidth of RF phase shifter is also challenging and demand more power consumption. The advantage is that requires one single mixer. An example of both RF and LO phase shifters can be seen in Figure 4.

Observation 3:  The most relevant components in the RF front of end in the transmitter chain should be considered as key building blocks for further study in NR

- At higher frequency the efficiency and output power decreases. This means that for transmitting a certain amount of signal power, more power is required by the PA compared to the systems operating at carrier frequency under 6 GHz.

- Phase shifters are source of imbalance and the accuracy depends on the components so that the 90 degree phase shift is not really exact. There are different phased-array architectures (i.e., RF phase shifters, LO phase shifters) which differ in functionality of phase shifting and signal combining and have as critical factors power consumption, losses and bandwidth.

### 2.2.2 RX chain

The Rx chain is composed by switch, LNA and phase shifter. This subsection includes the most relevant components in the receiver chain, which should be considered as key building blocks for further study in NR.

Low Noise Amplifier

One of the most important blocks in the receiver is the LNA. It is needed at the input of the receiver to amplify the received signal and suppress the noise contribution of the downconverter. The first stage is the RF block, which begins with the antenna, followed by the switch, the LNA and ends with the phase shifter as explained in [3] the NF will be mainly determined by the LNA. Therefore, the design for the LNA should be optimize for high gain and low NF.

Phase Shifter

In receiver element the phase shifted signal are combined before arriving at the downconverter. The RF phase shifter kind can phase out the interference and then relax power consumption on the blocks down the chain. The advantage of RF phase shifter over the LO phase shifting is that the output signal after RF combiner has a high pattern directivity and can reject an interference before receiving units, maximizing the value of the phased array as a spatial filter. Another advantage is the elimination of LO distribution network resulting in a simple system architecture.

Filter

With the increasing complexity of the transceiver at mmWave, the filter has to satisfy more constraints at the same time, which puts a number of requirements on the filter such as resonance frequency, coupling factor, quality factor, temperature sensitivity, etc. Besides the technical limitation of filter at high frequencies, filters at the RF front end have the disadvantage of including a considerable extra loss to the RF chain. At mmWave is much more difficult than at frequencies below 6 GHz to get a high output power and adding a filter will implies extra loss, which is not easy to compensate. For these reasons FDD will probably remain a main duplex method for lower frequencies and TDD for mmWave range. [4]

Observation 4: The most relevant components in the RF front of end in the receiver chain should be considered as key building blocks for further study in NR

- The NF will be mainly determined by the LNA. Therefore, the design for the LNA should be optimize for high gain and low NF.

- The advantage of RF phase shifter over the LO phase shifting is that the output signal after RF combiner has a high pattern directivity and can reject an interference before receiving units, maximizing the value of the phased array as a spatial filter.

- At mmWave is much more difficult than at frequencies below 6 GHz to get a high output power and adding a filter will implies extra loss, which is not easy to compensate. For these reasons FDD will probably remain a main duplex method for lower frequencies and TDD for mmWave range

### 2.2.3 Beamforming

Due to high path loss encounter at mmWave, beamforming (BF) assumes an important role to stablish and maintain a robust communication link. There are several discussions about digital and analogue BF implementation. Digital beamforming provides a high flexibility in shaping beams at cost of increasing complexity. The high cost, power consumption and complexity of mmWave mixed signal components constrains the use of fully-digital BF architectures at the UE unlike simpler, lower antenna count more conventionally used in sub-6 GHz systems.  On the other hand, analogue BF has fewer RF chain than antenna elements, which is more cost effective with lower power consumption and therefore, more suitable for the UE at mmWave. The disadvantage of this approach is having less flexibility compared to digital BF, since it can only handle one beam at a time.

The hybrid BF architecture is another approach that consists of reducing the hardware complexity with limited number of RF chains but using digital precoding in order to have similar performance to the digital BF, which enables multi-stream communication and maximizes the sum rate with minimum interference.

Hybrid BF enables a viable solution of having a good trade-off between cost and performance but for 16 antenna array or larger.

Observation 5: Analogue BF has fewer RF chain than antenna elements, which is more cost effective with lower power consumption and therefore, more suitable for the UE at mmWave.

# 3	Conclusions

In this document, we have discussed the transceiver architectures including UE reference architecture for NR for mmWave with a more detailed analysis of the key components in the transmitter and receiver chain, as well as a comparison between digital, analogue and hybrid beamforming.

Observation 1: It will be beneficial to define a UE reference architecture to carry out the RF specification discussions, since different architectures determine efficiency, gain and linearity of the transceiver.

Observation 2: The UE reference architecture in Figure 3 with IF conversion is proposed in mmWave, this does not preclude from using direction conversion at a more advance stage in NR.

Observation 3:  The most relevant components in the RF front of end in the transmitter chain should be considered as key building blocks for further study in NR

- At higher frequency the efficiency and output power decreases. This means that for transmitting a certain amount of signal power, more power is required by the PA compared to the systems operating at carrier frequency under 6 GHz.

- Phase shifters are source of imbalance and the accuracy depends on the components so that the 90 degree phase shift is not really exact. There are different phased-array architectures (i.e., RF phase shifters, LO phase shifters) which differ in functionality of phase shifting and signal combining and have as critical factors power consumption, losses and bandwidth.

Observation 4: The most relevant components in the RF front of end in the receiver chain should be considered as key building blocks for further study in NR

- The NF will be mainly determined by the LNA. Therefore, the design for the LNA should be optimize for high gain and low NF.

- The advantage of RF phase shifter over the LO phase shifting is that the output signal after RF combiner has a high pattern directivity and can reject an interference before receiving units, maximizing the value of the phased array as a spatial filter.

- At mmWave is much more difficult than at frequencies below 6 GHz to get a high output power and adding a filter will implies extra loss, which is not easy to compensate. For these reasons FDD will probably remain a main duplex method for lower frequencies and TDD for mmWave range

Observation 5: Analogue BF has fewer RF chain than antenna elements, which is more cost effective with lower power consumption and therefore, more suitable for the UE at mmWave.

# 4	Reference

[1] “Flex5Gware Project, “Deliverable 2.1: Requirements and concepts for the analogue HW in 5G mobile systems”, Dec. 2015.

[2] R4-167263, “PA model using a Memory Polynomial”

[3] R4-1609123, “NF values in ITU-R WP5D related coexistence simulations”

[4] R4-1609122, “NR RF parameters and template for WP5D”