3GPP TSG-RAN WG4 Meeting #82	R4-1700706

# Athens, Greece, 13-17 February 2017

Source:	SoftBank Corp.

Title:	TP for TR36.714-03-01: Support of LTE-A Inter-band Carrier Aggregation (3DL/1UL) of Band 3, Band 11 and Band 28

Agenda item:	7.3.2

Document for:	Approval

1.  Introduction

3DL/1UL of 3A-11A-28A was approved in the basket WID [1]. This paper is to propose all the technical elements to complete this 3DL CA, including TP.

Note that proposals in a companion paper [2] are already captured in this TP so advanced agreement of [2] is a prerequisite for this TP.

2.  Basic Scheme

The bands and BW configurations for the CA are shown in Table 1 and 2.

Table 1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3-11-28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |

Table 2: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_3A-11A-28A | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  | 11 |  |  | Yes | Yes |  |  |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: For the UE that signals support of a 3DL CA bandwidth combination set, the UE shall support all single carrier channel bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 and all the 2DL CA bandwidth combination sets in Table 5.6A.2-1 of TS 36.101 whose channel bandwidths are entirely covered by the signaled 3DL CA bandwidth combination set. |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 [2] when operating in single carrier mode.

3.  Co-existence study

The 3rd order IMD products caused in the BS by transmitting of Band 3, Band 11 and Band 28 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 3 below:

Table 3: Band 3, Band 11 and Band 28 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 1475.9 | 1495.9 | 1805 | 1880 |
| 3rd order IMD products | ¦f1-high + f2-high – f3-low¦ |  |  | ¦f1-low + f2-low – f3-high¦ |  |  |
| IMD frequency limits (MHz) | 353.9 |  |  | 493.9 |  |  |
| 3rd order IMD products | ¦f1-low + f3-low – f2-high¦ |  |  | ¦f1-high + f3-high – f2-low¦ |  |  |
| IMD frequency limits (MHz) | 1067.1 |  |  | 1207.1 |  |  |
| 3rd order IMD products | ¦f2-low + f3-low – f1-high¦ |  |  | ¦f2-high + f3-high – f1-low¦ |  |  |
| IMD frequency limits (MHz) | 2477.9 |  |  | 2617.9 |  |  |
| 3rd order IMD products | ¦f1-low + f2-low + f3-low¦ |  |  | ¦f1-high + f2-high + f3-high¦ |  |  |
| IMD frequency limits (MHz) | 4038.9 |  |  | 4178.9 |  |  |

It can be seen from Table 3 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (3 + 11 + 28) may fall into BS receive band of Bands 7, 31, 38 and 41. However, Band 7, 31 and 38 are not intended for use in the same geographical area as Bands 3,11 and 28. Therefore, the focus here is on the 3rd order IMD products falling into Band 41.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 41 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (3 + 11 + 28) BS transmitter does not share the same antenna with Band 41 BS receiver.

Therefore, it is recommended that Band (3 + 11 + 28) BS transmitter should not share the same antenna with Band 41 BS receiver to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 41 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

Table 4 gives the 2nd and 3rd harmonic for CA with B3+ B11 + B28. It can be seen that distortions caused by 2nd order harmonics of Band 28 located within the pass band of Band 11. It was already identified in previous work and specified in TS 36.101. Except 2nd order harmonics of Band 28, none of the products falls into the B3, B11 or B28 UE receiving bands.

Table 4: UE harmonic table for 3DL/1UL

| UE DL Carriers with B3, B11, B28 | [MHz] |
| --- | --- |
| 2*fx_low to 2*fx_high | 3420 to 3570 |
| 2*fy_low to 2*fy_high | 2855.8 to 2895.8 |
| 2*fz_low to 2*fz_high | 1406 to 1496 |
| 3*fx_low to 3*fx_high | 5130 to 5355 |
| 3*fy_low to 3*fy_high | 4283.7 to 4343.7 |
| 3*fz_low to 3*fz_high | 2109 to 2244 |

4.  ΔTIB and ΔRIB

Based on the constituent 2DL/1UL values in Table 5 and 6, the following values are proposed.

Table 5: ΔTIB of constituent 2DL/1UL

| ΔTIB | 3+11 | 3+28 | 11+28 | Proposed Value |
| --- | --- | --- | --- | --- |
| B3 | 0.8 | 0.3 | - | 0.8 |
| B11 | 0.9 | - | 0.4 | 0.9 |
| B28 | - | 0.3 | 0.6 | 0.6 |

Table 6: ΔRIB of constituent 2DL/1UL

| ΔRIB | 3+11 | 3+28 | 11+28 | Proposed Value |
| --- | --- | --- | --- | --- |
| B3 | 0.3 | 0 | - | 0.3 |
| B11 | 0.5 | - | 0 | 0.5 |
| B28 | - | 0 | 0.2 | 0.2 |

Table 7: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_3A-11A-28A | 3 | 0.8 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |

Table 8: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_3A-11A-28A | 3 | 0.3 |
|  | 11 | 0.5 |
|  | 28 | 0.2 |

5.  REFSENS

As mentioned above, this CA includes a harmonic issue when the uplink is Band 28. MSDs have been obtained through the studies of relevant 2DL CA and the following is the proposal based on the results.

Table 9 (7.3.1A-0a): Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_3A-11A-28A9,10 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 11 |  |  | -75.2 | -75.2 |  |  |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_1A-3A-42A, CA_3A-19A-42A, and CA_1A-3A-19A-42A. FHD = 0MHz for CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 10 (7.3.1A-0b): Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_3A-11A-28A | 28 |  |  | 12 | 25 |  |  | FDD |

6.  Conclusion

This paper is to propose all the necessary items to complete 3A-11A-28A. TP is attached to capture the discussion above.

7.  Reference

[1]     RP-162146	Revision of WI LTE Advanced inter-band CA Rel-14 for 3DL/1UL, Huawei (Rapporteur)

[2]     R4-1700682   Handling of the 2nd harmonic note for CA_11A-28A, SoftBank

Annex: TP for TR36.714-03-01 (v0.6.0)

[Unchanged portions skipped]

6.Y 	CA_3A-11A-28A_BCS0

6.Y.1	Operating bands for CA

Table 6.Y.1-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3-11-28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |

6.Y.2	Channel bandwidths per operating band for CA

Table 6.Y.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_3A-11A-28A | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  | 11 |  |  | Yes | Yes |  |  |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: For the UE that signals support of a 3DL CA bandwidth combination set, the UE shall support all single carrier channel bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 and all the 2DL CA bandwidth combination sets in Table 5.6A.2-1 of TS 36.101 whose channel bandwidths are entirely covered by the signaled 3DL CA bandwidth combination set. |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 [2] when operating in single carrier mode.

6.Y.3 	Harmonics and IMD study

6.Y.3.1	For the BS

The 3rd order IMD products caused in the BS by transmitting of Band 3, Band 11 and Band 28 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.Y.3.1-1 below:

Table 6.Y.3.1-1: Band 3, Band 11 and Band 28 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 1475.9 | 1495.9 | 1805 | 1880 |
| 3rd order IMD products | ¦f1-high + f2-high – f3-low¦ |  |  | ¦f1-low + f2-low – f3-high¦ |  |  |
| IMD frequency limits (MHz) | 353.9 |  |  | 493.9 |  |  |
| 3rd order IMD products | ¦f1-low + f3-low – f2-high¦ |  |  | ¦f1-high + f3-high – f2-low¦ |  |  |
| IMD frequency limits (MHz) | 1067.1 |  |  | 1207.1 |  |  |
| 3rd order IMD products | ¦f2-low + f3-low – f1-high¦ |  |  | ¦f2-high + f3-high – f1-low¦ |  |  |
| IMD frequency limits (MHz) | 2477.9 |  |  | 2617.9 |  |  |
| 3rd order IMD products | ¦f1-low + f2-low + f3-low¦ |  |  | ¦f1-high + f2-high + f3-high¦ |  |  |
| IMD frequency limits (MHz) | 4038.9 |  |  | 4178.9 |  |  |

It can be seen from Table 6.Y.3.1-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (3 + 11 + 28) may fall into BS receive band of Bands 7, 31, 38 and 41. However, Band 7, 31 and 38 are not intended for use in the same geographical area as Bands 3,11 and 28. Therefore, the focus here is on the 3rd order IMD products falling into Band 41.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 41 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (3 + 11 + 28) BS transmitter does not share the same antenna with Band 41 BS receiver.

Therefore, it is recommended that Band (3 + 11 + 28) BS transmitter should not share the same antenna with Band 41 BS receiver to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 41 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

6.Y.3.2	For the UE

Table 6.Y.3.2-1 gives the 2nd and 3rd harmonic for CA with B3+ B11 + B28. It can be seen that distortions caused by 2nd order harmonics of Band 28 located within the pass band of Band 11. It was already identified in previous work and specified in [6]. Except 2nd order harmonics of Band 28, none of the products falls into the B3, B11 or B28 UE receiving bands.

Table 6.Y.3.2-1: UE harmonic table for 3DL/1UL

| UE DL Carriers with B3, B11, B28 | [MHz] |
| --- | --- |
| 2*fx_low to 2*fx_high | 3420 to 3570 |
| 2*fy_low to 2*fy_high | 2855.8 to 2895.8 |
| 2*fz_low to 2*fz_high | 1406 to 1496 |
| 3*fx_low to 3*fx_high | 5130 to 5355 |
| 3*fy_low to 3*fy_high | 4283.7 to 4343.7 |
| 3*fz_low to 3*fz_high | 2109 to 2244 |

6.Y.4	ΔTIB,c and ΔRIB,c values

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c is defined for applicable bands in Table 6.Y.4-1. Note that constituent CA 11A-28A is subject to HTF (Harmonic Trap Filter) in Band 28 and the values below are taken this fact into account.

Table 6.Y.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_3A-11A-28A | 3 | 0.8 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |

For the UE which supports inter-band carrier aggregation configuration in Table 6.Y.4-2 with uplink in one E-UTRA band, the minimum requirement for reference sensitivity in Table 7.3.1-1 (of TS 36.101 [6]) shall be increased by the amount given in ΔRIB,c in Table 6.Y.4-2 for the applicable E-UTRA bands. Note again that constituent CA of 11A-28A is subject to HTF (Harmonic Trap Filter) in Band 28 and the values below are taken this fact into account.

Table 6.Y.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_3A-11A-28A | 3 | 0.3 |
|  | 11 | 0.5 |
|  | 28 | 0.2 |

6.Y.5	REFSENS

This CA includes two harmonic issues when the uplink is Band 28. MSDs have been obtained through the studies of relevant 2DL CAs and the following is the proposal based on the results. Two sets of uplink configurations are required to be defined for Band 28 with new notes (Note 4, 5).

Table 6.Y.5-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_3A-11A-28A9,10 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 11 |  |  | -75.2 | -75.2 |  |  |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_1A-3A-42A, CA_3A-19A-42A, and CA_1A-3A-19A-42A. FHD = 0MHz for CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 6.Y.5-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_3A-11A-28A | 28 |  |  | 12 | 25 |  |  | FDD |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"