3GPP TSG-RAN WG4 Meeting #82	R4-1700756

Athens, Greece, 13 – 17 February, 2017

Source:	Ericsson

Title:	TP to TR 36.714-02-02: CA_2DL_2A-30A_2UL_2A-30A_BCS0

Agenda item:	7.6.2

Document for:	Approval

# 1. Introduction

This is a text proposal for 36.714-02-02 for inclusion of CA_2DL_2A-30A_2UL_2A-30A_BCS0 as defined in RP-162148 [1].

# 2. Text Proposal for TR 36.714-02-02

##### ---Start of changes---

## 6.x	CA_2DL_2A-30A_2UL_2A-30A_BCS0

### 6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: E-UTRA CA configuration and bandwidth combination sets for CA_2A-30A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-30A | CA_2A-30A | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |

### 6.x.2	Co-existence studies

#### 6.x.2.1	UE-UE co-existence

Table 6.x.2-1 lists B2+B30 2UL CA 2nd and 3rd order harmonics. Table 6.x.2-2 lists the IMDs from 2nd order to 5th order for UE-UE co-existence analysis.

Table 6.x.2.1-1: Band 2 and Band 30 UL harmonics

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) |  | 1850 |  | 1910 |  | 2305 |  | 2315 |  |
| 2nd harmonics frequency limits |  | 2*fx_low |  | 2*fx_high |  | 2* fy_low |  | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) |  | 3700 |  | 3820 |  | 4610 |  | 4630 |  |
| 3rd harmonics frequency limits |  | 3*fx_low |  | 3*fx_high |  | 3* fy_low |  | 3* fy_high |  |
| 3rd harmonics frequency limits (MHz) |  | 5550 |  | 5730 |  | 6915 |  | 6945 |  |

Table 6.x.2.1-2: 2UL CA_2A-30A IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 2305 | 2315 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 465 | 395 | 4155 | 4225 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1385 | 1515 | 2700 | 2780 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6005 | 6135 | 6460 | 6540 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 930 | 790 | 8310 | 8450 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3235 | 3425 | 5005 | 5095 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7855 | 8045 | 8765 | 8855 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7410 | 7310 | 5335 | 5085 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11070 | 11170 | 9705 | 9955 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3245 | 3095 | 1120 | 920 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10615 | 10765 | 10160 | 10360 |

Based on table 6.x.2.1-1:

2nd order harmonics may fall into Rx frequencies of band 43.

3rd order harmonics may fall into Rx frequencies of band 46.

Based on table 6.x.2.1-2:

2nd order IMD may also fall into Rx frequencies of band 31.

3rd order IMD may also fall into Rx frequencies of band 11, 21, 32 and 45.

4th order IMD may also fall into Rx frequencies of band 5, 8, 18, 19, 20, 26, 27, 28 and 42.

5th order IMD may also fall into Rx frequencies of band 8 and 46.

#### 6.x.2.2	Co-existence with other systems

When 2ULs inter-band CA UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.x.2.2-1 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.x.2.2-1: 2UL B2 + B30 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | - | - | - |
| Galileo | 1559 | - | 1591 | - | - | - |
| GLONASS | 1591 | - | 1610 | - | - | - |
| GPS | 1563 | - | 1587 | - | - | - |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | - | US/Europe | - |
|  | 2400 | - | 2494 | - | Asia | - |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD5 3rd order harmonics of B2 |
|  | 5150 | - | 5350 | Yes | Europe | IMD5 3rd order harmonics of B2 |
|  | 5470 | - | 5725 | Yes |  | IMD5 3rd order harmonics of B2 |
|  | 5150 | - | 5825 | Yes | Asia | IMD5 3rd order harmonics of B2 |

### 6.x.3 	∆TIB and ∆RIB values

For inter-band CA_2A-30A, same requirements on values for ∆TIB and ∆RIB can apply with 1UL-2DL case. Therefore, for two simultaneous UL, the TIB,c and RIB,c values are shown in table 6.1.3-1 and in table 6.1.3-2.

Table 6.x.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-30A | 2 | 0.5 |
|  | 30 | 0.3 |

Table 6.x.3-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2A-30A | 2 | 0.4 |
|  | 30 | 0.5 |

### 6.x.4 	MSD

As shown in Table 6.x.2.1-2 IMD up to 5th order does not fall into own Rx of band 2 and 30. Therefore, no MSD requirement is needed for this 2DL/2UL CA configuration in TS 36.101.

##### ---End of changes---

# Reference

[1]	 RP-162148, “Revised WID: LTE Advanced inter-band CA Rel-14 for 2DL/2UL”, RAN#74, Huawei, HiSilicon