3GPP TSG-RAN4 Meeting #82	R4-1700840

# Athens, Greece, February 13-17, 2017

Title:	Reply LS to RAN1 on possible RF impacts related to sPUSCH design

Release:	Release 15

Work Item:	LTE_sTTIandPT

Source:	RAN4

To:	RAN1

Contact Person:

#### Name:	Dominique Everaere

E-mail Address:	dominique.everaere@ericsson.com

Attachments: None

- Description:

RAN4 would like to thank RAN1 for the questions in R1-1613334 regarding the possible RF impacts related to sPUSCH design.

After discussing the issue in RAN4, we have the following answers to the questions:

Q1: If there is any RF impacts that would degrade sPUSCH demodulation performance when having shared DMRS symbols in between 2 sTTIs but using different output power levels?

A1:   To guaranty performance, both sTTIs should have same ouput power when sharing DMRS symbols.

Q1: If there is any RF impacts that would degrade sPUSCH demodulation performance when having shared DMRS symbols in between 2 sTTIs but using differentbadnwidth allocations?

A2: To guaranty performance, both sTTIs should have same bandwidth allocation when sharing DMRS symbols.

Q1: If there is any RF impacts that would degrade sPUSCH demodulation performance when allocating DMRS symbol(s) non-contiguously in time?

A3:   This would need further investigations in RAN4. A new UE requirement (phase discontinuity) should be specified.

- Actions:

RAN4 asks RAN1 to consider above answers in their evaluation.

- Date of Next TSG-RAN4 Meetings:

TSG-RAN4 Meeting#82b		April 3-7					Spokane, USA

TSG-RAN4 Meeting#83		May 15-19				Hangzhou, China