3GPP TSG-RAN WG4 Meeting #82	R4-1700938

Athens, Greece, 13 – 17 February, 2017

Source:	KT

Title:	TP for TR36.714-03: the support of CA_1A-3A-3A

Agenda item:	7.3.2

Document for:	Approval

1.  Introduction

This is text proposal for TR 36.714-03 for B1A+B3A+B3A 3DL/1UL CA.

2. Basic Configurations

As written in the WID, the following configurations are assumed for B1+B3+B3.

Table 1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-3-3 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |

Table 2: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-3A | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 | See the CA_3A-3A Bandwidth combination set 0 in the Table below |  |  |  |  |  |  |  |

3.  Co-existence

3.1 For the BS

Table 3 gives the harmonics and IMD products for Band 1 + Band 3+ Band 3 CA with 3DL. It shows that the second harmonics may fall into BS receive band of Band 43, and no third harmonics will fall into any BS receive band specified by 3GPP. In addition, no second order intermodulation products will fall into any BS receive band. The third order intermodulation products may fall into the BS receive band of Band 2, 7, 11, 21, 24, 25, 35, 39, 40 and 41.

Note that Band 2, 25 and 35 are not intended for use in the same geographical area as Band 1 and Band 3. It is suggested BS transmitters supporting CA of Band 1 + Band 3 +Band3 should not share the same antenna with Band 7, 11, 21, 24, 39, 40, 41 and 43 BS receiver, unless the antenna path meets very stringent third order PIM specification so that the PIM will not cause Band 7, 11, 21, 24, 33, 39, 40, 41 and 43 BS receiver desensitization. Specific analysis on the harmonics and IMD products with consideration on the adopted bandwidth and the carrier places are needed if considering the shared antenna.

# Table 3: 3DL B1 + B3 + B3 Harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2110 | 2170 | 1805 | 1880 |
| 2nd order harmonics frequency range (MHz) | 4220 to 4340 |  | 3610 to 3760 |  |
| 3rd order harmonics frequency range (MHz) | 6330 to 6510 |  | 5415 to 5640 |  |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 230 to 365 |  | 3915 to 4050 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high &(2*f1_low + f2_low) | 2*f1_high – f2_low & (2*f1_high + f2_high) | 2*f2_low – f1_high &(2*f2_low + f1_low) | 2* f2_high – f1_low &(2*f2_high + f1_high) |
| IMD frequency range (MHz) | 2340 to 2535 6025 to 6220 |  | 1440 to 1650 5720 to 5930 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 2070 to 2210 |  | 1785 to 1900 |  |

3.2	For the UE

As shown in Table 4, the harmonic frequencies of band 1 and band 3 in UL are away from the receive bands of B1 and B3 in the DL, so that we can conclude that there is no issue on UL harmonic interference.

# Table 4: Impact of UL Harmonic Interference

| Band | 1 |  | 3 |  |
| --- | --- | --- | --- | --- |
| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| DL frequency (MHz) | 2110 | 2170 | 1805 | 1880 |
| 2nd order harmonics frequency range (MHz) | 3840 to 3960 |  | 3420 to 3570 |  |
| 3rd order harmonics frequency range (MHz) | 5760 to 5940 |  | 5130 to 5355 |  |

4.  ΔTIB and ΔRIB

For the UE which supports inter-band carrier aggregation configuration with uplink assigned to one E-UTRA band the ΔTIB,c and RIB,c are defined for applicable bands in Table 5 and Table 6

# Table 5: IB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-3A | 1 | 0.3 |
|  | 3 | 0.3 |

# Table 6: RIB.c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-3A | 1 | 0 |
|  | 3 | 0 |

5.  REFSENS

Considered what has been discussed for B1+B3 CA and B1+B3C, the CA_B1A_B3A_B3A also need to be separated into two cases when the uplink is configured in Band 1. One is without MSD issues when the separation between the upper edge of the aggregated DL continuous carrier in Band 3 and the lower edge of UL carrier in Band 1 is equal or larger than 60MHz. The other one is degradation of Band 3 DL from Band 1 UL due to narrow frequency that the edge-to-edge separation between the Band 1 UL and Band 3 DL is less than 60MHz. The former MSD is specified in Tables 6 and 7, and the latter is specified in Tables 8 and 9. When the uplink is configured in Band3, the TX-RX separation is large enough so that there would be no MSD issues either.

# Table 6: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-3A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
| NOTE: For the UE that signals support of a 3DL CA bandwidth combination set, the UE shall support all single carrier channel bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 [2] and all the 2DL CA bandwidth combination sets in Table 5.6A.2-1 of TS 36.101 [2] whose channel bandwidths are entirely covered by the signaled 3DL CA bandwidth combination set. |  |  |  |  |  |  |  |  |

# Table 7 Uplink configuration

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A1, 2-3A1, 2 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

# Table 8: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-3A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
| NOTE: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1, and the requirements apply to both Band3 downlink carriers. |  |  |  |  |  |  |  |  |

# Table 9 Uplink configuration

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A1, 2-3A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. |  |  |  |  |  |  |  |  |

6.  Summary

This paper investigates all the necessary elements to complete CA_1A-3A-3A. The relevant TP is shown in Annex.

Annex: TP for TR36.714-03

[Unchanged portions skipped]

X.Y 	Band 1A, Band 3A and Band 3A

X.Y.1	Operating bands for CA

Table X.Y.1-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-3-3 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |

X.Y.2	Channel bandwidths per operating band for CA

Table X.Y.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-3A | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 | See the CA_3A-3A Bandwidth combination set 0 in the Table below |  |  |  |  |  |  |  |

X.Y.3 	Harmonics and IMD study

X.Y.3.1	For the BS

Table X.Y.3.1-1 gives the harmonics and IMD products for Band 1 + Band 3+ Band 3 CA with 3DL. It shows that the second harmonics may fall into BS receive band of Band 43, and no third harmonics will fall into any BS receive band specified by 3GPP. In addition, no second order intermodulation products will fall into any BS receive band. The third order intermodulation products may fall into the BS receive band of Band 2, 7, 11, 21, 24, 25, 35, 39, 40 and 41.

Note that Band 2, 25 and 35 are not intended for use in the same geographical area as Band 1 and Band 3. It is suggested BS transmitters supporting CA of Band 1 + Band 3 +Band3 should not share the same antenna with Band 7, 11, 21, 24, 39, 40, 41 and 43 BS receiver, unless the antenna path meets very stringent third order PIM specification so that the PIM will not cause Band 7, 11, 21, 24, 33, 39, 40, 41 and 43 BS receiver desensitization. Specific analysis on the harmonics and IMD products with consideration on the adopted bandwidth and the carrier places are needed if considering the shared antenna.

Table X.Y.3.1-1: Band 1, Band 3 and Band 3 DL 3rd order IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2110 | 2170 | 1805 | 1880 |
| 2nd order harmonics frequency range (MHz) | 4220 to 4340 |  | 3610 to 3760 |  |
| 3rd order harmonics frequency range (MHz) | 6330 to 6510 |  | 5415 to 5640 |  |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 230 to 365 |  | 3915 to 4050 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high &(2*f1_low + f2_low) | 2*f1_high – f2_low & (2*f1_high + f2_high) | 2*f2_low – f1_high &(2*f2_low + f1_low) | 2* f2_high – f1_low &(2*f2_high + f1_high) |
| IMD frequency range (MHz) | 2340 to 2535 6025 to 6220 |  | 1440 to 1650 5720 to 5930 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 2070 to 2210 |  | 1785 to 1900 |  |

X.Y.3.2	For the UE

As shown in Table X.Y.3.2-1, the harmonic frequencies of band 1 and band 3 in UL are away from the receive bands of B1 and B3 in the DL, so that we can conclude that there is no issue on UL harmonic interference.

Table X.Y.3.2-1: UE harmonic table for 3DL/1UL

| Band | 1 |  | 3 |  |
| --- | --- | --- | --- | --- |
| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| DL frequency (MHz) | 2110 | 2170 | 1805 | 1880 |
| 2nd order harmonics frequency range (MHz) | 3840 to 3960 |  | 3420 to 3570 |  |
| 3rd order harmonics frequency range (MHz) | 5760 to 5940 |  | 5130 to 5355 |  |

X.Y.4	ΔTIB,c and ΔRIB,c values

For the UE which supports inter-band carrier aggregation configuration with uplink assigned to one E-UTRA band the ΔTIB,c and RIB,c are defined for applicable bands in Table X.Y.4-1 and Table X.Y.4-2

Table X.Y.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-3A | 1 | 0.3 |
|  | 3 | 0.3 |

Table X.Y.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-3A | 1 | 0 |
|  | 3 | 0 |

X.Y.5	REFSENS requirements

Considered what has been discussed for B1+B3 CA and B1+B3C, the CA_B1A_B3A_B3A also need to be separated into two cases when the uplink is configured in Band 1. One is without MSD issues when the separation between the upper edge of the aggregated DL continuous carrier in Band 3 and the lower edge of UL carrier in Band 1 is equal or larger than 60MHz. The other one is degradation of Band 3 DL from Band 1 UL due to narrow frequency that the edge-to-edge separation between the Band 1 UL and Band 3 DL is less than 60MHz. The former MSD is specified in Tables X.Y.5-1 and X.Y.5-2, and the latter is specified in Tables X.Y.5-3 and X.Y.5-4. When the uplink is configured in Band3, the TX-RX separation is large enough so that there would be no MSD issues either.

# Table X.Y.5-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-3A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
| NOTE: For the UE that signals support of a 3DL CA bandwidth combination set, the UE shall support all single carrier channel bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 [2] and all the 2DL CA bandwidth combination sets in Table 5.6A.2-1 of TS 36.101 [2] whose channel bandwidths are entirely covered by the signaled 3DL CA bandwidth combination set. |  |  |  |  |  |  |  |  |

# Table X.Y.5-2: Uplink configuration

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A1, 2-3A1, 2 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

# Table X.Y.5-3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-3A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
| NOTE: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1, and the requirements apply to both Band3 downlink carriers. |  |  |  |  |  |  |  |  |

# Table X.Y.5-4: Uplink configuration

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A1, 2-3A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. |  |  |  |  |  |  |  |  |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"