3GPP TSG-RAN WG4 Meeting #82	R4-1701331

Athens, Greece, 13 - 17 February, 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 4267 | rev | - | Current version: | 14.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of bands to support 4Rx APs to TS 36.101 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_4Rx_AP_DL_bands-Core |  |  |  |  |  | Date: |  |  | 2017-1-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | This CR is to introduce the following bands to support 4Rx APs. - Band 21, 25, 40. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following requirements are introduced. - Single carrier 4Rx requirements for Band 21, 25, 40. - CA+4Rx requirements for Band 21, 40 4Rx. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 4Rx APs can’t be supported by the above bands. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A, 7.3.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# <Start of changes>

Table 7.3.1-1a: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -102.7 | -99.7 | -97.9 | -96.7 | FDD |
| 2 | -105.4 | -102.4 | -100.7 | -97.7 | -95.9 | -94.7 | FDD |
| 3 | -104.4 | -101.4 | -99.7 | -96.7 | -94.9 | -93.7 |  |
| … |  |  |  |  |  |  |  |
| 7 |  |  | -100.7 | -97.7 | -95.9 | -94.7 | FDD |
| … |  |  |  |  |  |  |  |
| 20 |  |  | -99.7 | -96.7 | -93.9 | -92.7 | FDD |
| 21 |  |  | -102.7 | -99.7 | -97.9 |  | FDD |
| 25 | -103.9 | -100.9 | -99.2 | -96.2 | -94.4 | -93.2 | FDD |
| … |  |  |  |  |  |  |  |
| 39 |  |  | -102.7 | -99.7 | -97.9 | -96.7 | TDD |
| 40 |  |  | -102.7 | -99.7 | -97.9 | -96.7 | TDD |
| … |  |  |  |  |  |  |  |
| 41 |  |  | -100.7 | -97.7 | -95.9 | -94.7 | TDD |
| 42 |  |  | -101.2 | -98.2 | -96.4 | -95.2 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port. |  |  |  |  |  |  |  |

# <Next change>

Table 7.3.1A-0a: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) |  | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-7A-8A4,5,6 | 1 |  |  | -100 | -97 | -95.2 |  | -94 | FDD |
|  |  |  |  | -102.720 | -99.720 | -97.920 |  | -96.720 |  |
|  | 3 |  |  | N/A | N/A | N/A |  | N/A |  |
|  | 7 |  |  | -88 | -87.4 | -87 |  | -86.7 |  |
|  | 8 |  |  | -96.8 | -93.8 |  |  |  |  |
| CA_1A-3A-7A-20A-42A9,10 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 7 |  |  |  | -94.8 | -93 |  | -91.8 |  |
|  |  |  |  |  | -97.520 | -95.720 |  | -94.520 |  |
|  | 20 |  |  | -97 | -94 | -91.2 |  | -90 |  |
|  |  |  |  | -99.720 | -96.720 | -93.920 |  | -92.720 |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_1A-3A-7A-20A-42A11 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 7 |  |  |  | -94.8 | -93 |  | -91.8 |  |
|  |  |  |  |  | -97.520 | -95.720 |  | -94.520 |  |
|  | 20 |  |  | -97 | -94 | -91.2 |  | -90 |  |
|  |  |  |  | -99.720 | -96.720 | -93.920 |  | -92.720 |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_1A-3A-7A-28A5,6 | 1 |  |  | -89.8 | -89.4 | -89 |  | -88.7 | FDD |
|  | 3 |  |  |  | -94 | -92.2 |  | -91 |  |
|  |  |  |  |  | -96.720 | -94.920 |  | -93.720 |  |
|  | 7 |  |  |  | -95 | -93.2 |  | -92 |  |
|  |  |  |  |  | -97.720 | -95.920 |  | -94.720 |  |
|  | 28 |  |  |  | -95.3 | -93.5 |  | -90.8 |  |
| CA_1A-3A-7A-42A9,10 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 7 |  |  |  | -94.8 | -93 |  | -91.8 |  |
|  |  |  |  |  | -97.520 | -95.720 |  | -94.520 |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_1A-3A-7A-42A11 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 7 |  |  |  | -94.8 | -93 |  | -91.8 |  |
|  |  |  |  |  | -97.520 | -95.720 |  | -94.520 |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_1A-3A-8A4 | 1 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 3 |  |  | N/A | N/A | N/A |  | N/A |  |
|  | 8 |  | N/A | N/A | N/A |  |  |  |  |
| CA_1A-3A-8A-11A4 | 1 |  |  | -100 | -97 | -95.2 |  | -94 | FDD |
|  |  |  |  | -102.720 | -99.720 | -97.920 |  | -96.720 |  |
|  | 3 |  |  | N/A | N/A | N/A |  | N/A |  |
|  | 8 |  |  | -97 | -94 |  |  |  |  |
|  | 11 |  |  | -100 | -97 |  |  |  |  |
| CA_1A-3A-8A-40A4 | 1 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 3 |  |  | N/A | N/A | N/A |  | N/A |  |
|  | 8 |  | N/A | N/A | N/A |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 |  | -94 | TDD |
| CA_1A-3A-19A-42A9,10 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_1A-3A-19A-42A11 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_1A-3A-21A-28A4,21 | 1 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 3 |  |  | N/A | N/A | N/A |  | N/A |  |
|  | 21 |  |  | N/A | N/A | N/A |  |  |  |
|  | 28 |  |  | N/A | N/A |  |  |  |  |
| CA_1A-3A-21A-42A22,23 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.7 | -93.7 | -91.9 |  | -90.7 |  |
|  |  |  |  | -99.420 | -96.420 | -94.620 |  | -93.420 |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |  |
|  |  |  |  | -102.220 | -99.220 | -97.420 |  |  |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_1A-3A-21A-42A24 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.7 | -93.7 | -91.9 |  | -90.7 |  |
|  |  |  |  | -99.420 | -96.420 | -94.620 |  | -93.420 |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |  |
|  |  |  |  | -102.220 | -99.220 | -97.420 |  |  |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_1A-3A-28A-42A22,23 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_1A-3A-28A-42A24 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_1A-3A-28A | 1 |  |  | -89.8 | -89.4 | -89 |  | -88.7 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 |  | -91 |  |
|  |  |  |  | -99.720 | -96.720 | -94.920 |  | -93.720 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 |  | -90.8 |  |
| CA_1A-3A-42A9,10 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_1A-3A-42A11 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_1A-7A-8A5,6 | 1 |  |  | -100 | -97 | -95.2 |  | -94 | FDD |
|  |  |  |  | -102.720 | -99.720 | -97.920 |  | -96.720 |  |
|  | 7 |  |  |  | -87.4 | -87 |  | -86.7 |  |
|  | 8 |  |  | -96.8 | -93.8 |  |  |  |  |
| CA_1A-7A-28A5,6 | 1 |  |  | -89.8 | -89.4 | -89 |  | -88.7 | FDD |
|  | 7 |  |  |  | -95 | -93.2 |  | -92 |  |
|  |  |  |  |  | -97.720 | -95.920 |  | -94.720 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 |  | -90.8 |  |
| CA_1A-18A-28A14 | 1 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 18 |  |  | N/A | N/A | N/A |  |  |  |
|  | 28 |  |  | N/A | N/A |  |  |  |  |
| CA_1A-19A-28A14 | 1 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 19 |  |  | N/A | N/A | N/A |  |  |  |
|  | 28 |  |  | N/A | N/A |  |  |  |  |
| CA_1A-21A-28A4,21 | 1 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 21 |  |  | N/A | N/A | N/A |  |  |  |
|  | 28 |  |  | N/A | N/A |  |  |  |  |
| CA_1A-21A-28A-42A4,21 | 1 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 21 |  |  | N/A | N/A | N/A |  |  |  |
|  | 28 |  |  | N/A | N/A |  |  |  |  |
|  | 42 |  |  | N/A | N/A | N/A |  | N/A | TDD |
| CA_1A-28A5,6,14 | 1 |  |  | -89.8 | -89.4 | -89 |  | -88.7 | FDD |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 |  | -90.8 |  |
| CA_1A-28A-42A5,6,17,18 | 1 |  |  | -89.8 | -89.4 | -89 |  | -88.7 | FDD |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |
|  | 42 |  |  | -85.7 | -85.4 | -85.1 |  | -84.9 | TDD |
| CA_2A-2A-12A-30A-66A5,6 | 2 |  |  | -97.6 | -94.6 | -92.8 |  | -91.6 | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |  |
|  | 66 |  |  | -89.5 | -89 | -88.5 |  | -88 |  |
| CA_2A-4A-12A5,6 | 2 |  |  | -97.7 | -94.7 | -92.9 |  | -91.7 | FDD |
|  |  |  |  | -100.420 | -97.420 | -95.620 |  | -94.420 |  |
|  | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_2A-4A-5A-12A5,6 | 2 |  |  | -97.7 | -94.7 | -92.9 |  | -91.7 | FDD |
|  |  |  |  | -100.420 | -97.420 | -95.620 |  | -94.420 |  |
|  | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 |  |
|  | 5 |  |  | -97.5 | -94.5 |  |  |  |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_2A-4A-7A-12A5,6 | 2 |  |  | -97.7 | -94.7 | -92.9 |  | -91.7 | FDD |
|  |  |  |  | -100.420 | -97.420 | -95.620 |  | -94.420 |  |
|  | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 |  |
|  | 7 |  |  | -97.5 | -94.5 | -92.7 |  | -91.5 |  |
|  |  |  |  | -100.220 | -97.220 | -95.420 |  | -94.220 |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_2A-4A-12A-30A5,6 | 2 |  |  | -97.6 | -94.6 | -92.8 |  | -91.6 | FDD |
|  |  |  |  | -100.320 | -97.320 | -95.520 |  | -94.320 |  |
|  | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |  |
| CA_2A-12A-66A5,6 | 2 |  |  | -97.7 | -94.7 | -92.9 |  | -91.7 | FDD |
|  |  |  |  | -100.420 | -97.420 | -95.620 |  | -94.420 |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
|  | 66 |  |  | -89.5 | -89 | -88.5 |  | -88 |  |
| CA_3A-7A-8A4,5,6 | 3 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 7 |  |  | -88 | -87.4 | -87 |  | -86.7 |  |
|  | 8 |  |  | -96.8 | -93.8 |  |  |  |  |
| CA_3A-7A-20A-42A9,10 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | [-99.5]20 | [-96.5]20 | [-94.7]20 |  | [-93.5]20 |  |
|  | 7 |  |  | -97.8 | -94.8 | -93 |  | -91.8 |  |
|  |  |  |  | -100.520 | -97.520 | -95.720 |  | -94.520 |  |
|  | 20 |  |  | -97 | -94 | -91.2 |  | -90 |  |
|  |  |  |  | -99.720 | -96.720 | -93.920 |  | -92.720 |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_3A-7A-20A-42A11 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 7 |  |  | -97.8 | -94.8 | -93 |  | -91.8 |  |
|  | 20 |  |  | -97 | -94 | -91.2 |  | -90 |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_3A-7A-42A9,10 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | [-99.5]20 | [-96.5]20 | [-94.7]20 |  | [-93.5]20 |  |
|  | 7 |  |  | -97.8 | -94.8 | -93 |  | -91.8 |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_3A-7A-42A11 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | [-99.5]20 | [-96.5]20 | [-94.7]20 |  | [-93.5]20 |  |
|  | 7 |  |  | -97.8 | -94.8 | -93 |  | -91.8 |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_3A-8A4 | 3 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 8 |  | N/A | N/A | N/A |  |  |  |  |
| CA_3A-8A-11A4 | 3 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 8 |  |  | N/A | N/A |  |  |  |  |
|  | 11 |  |  | N/A | N/A |  |  |  |  |
| CA_3A-19A-21A-42A25,26 | 3 |  |  | -96.7 | -93.7 | -91.9 |  | -90.7 | FDD |
|  |  |  |  | -99.420 | -96.420 | -94.620 |  | -93.420 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |  |
|  |  |  |  |  |  |  |  | -102.220 |  | -99.220 |  | -97.420 |  |  |  |  |  |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_3A-19A-21A-42A27 | 3 |  |  | -96.7 | -93.7 | -91.9 |  | -90.7 | FDD |
|  |  |  |  | -99.420 | -96.420 | -94.620 |  | -93.420 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |  |
|  |  |  |  |  |  |  |  | -102.220 |  | -99.220 |  | -97.420 |  |  |  |  |  |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_3A-19A-42A9,10 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_3A-19A-42A11 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_3A-20A-42A9,10 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 20 |  |  | -97 | -94 | -91.2 |  | -90 |  |
|  |  |  |  | -99.711 | -96.711 | -93.911 |  | -92.711 |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_3A-20A-42A11 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 20 |  |  | -97 | -94 | -91.2 |  | -90 |  |
|  |  |  |  | -99.720 | -96.720 | -93.920 |  | -92.720 |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_3A-21A-28A4,21 | 3 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 21 |  |  | N/A | N/A | N/A |  |  |  |
|  | 28 |  |  | N/A | N/A |  |  |  |  |
| CA_3A-21A-42A1,2 | 3 |  |  | -96.7 | -93.7 | -91.9 |  | -90.7 | FDD |
|  |  |  |  | -99.44 | -96.44 | -94.64 |  | -93.44 |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |  |
|  |  |  |  |  |  |  |  | -102.220 |  | -99.220 |  | -97.420 |  |  |  |  |  |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_3A-21A-42A3 | 3 |  |  | -96.7 | -93.7 | -91.9 |  | -90.7 | FDD |
|  |  |  |  | -99.44 | -96.44 | -94.64 |  | -93.44 |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |  |
|  |  |  |  |  |  |  |  | -102.220 |  | -99.220 |  | -97.420 |  |  |  |  |  |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_3A-28A-40A15,16 | 3 |  |  | -97 | -94 | -92.2 |  | -91 | FDD |
|  |  |  |  | -99.720 | -96.720 | -94.920 |  | -93.720 |  |
|  | 28 |  |  | -60.7 | -60.7 | -60.7 |  | -60.7 |  |
|  | 40 |  |  | -100 | -97 | -95.2 |  | -94 | TDD |
| CA_3A-28A-42A9,10 | 3 |  |  | -96.8 | -93.8 |  | -92 | -90.8 | FDD |  |
|  |  |  |  | -99.56 | -96.56 |  | -94.76 | -93.56 |  |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |  |
|  | 42 |  |  | -71.7 | -71.7 |  | -71.7 | -71.7 | TDD |  |
| CA_3A-28A-42A11 | 3 |  |  | -96.8 | -93.8 |  | -92 | -90.8 | FDD |  |
|  |  |  |  | -99.56 | -96.56 6 |  | -94.76 | -93.56 |  |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |  |
|  | 42 |  |  | -97.1 | -94.7 |  | -93.2 | -92.5 | TDD |  |
| CA_3A-28A-42A17,18 | 3 |  |  | -96.8 | -93.8 |  | -92 | -90.8 | FDD |  |
|  |  |  |  | -99.56 | -96.56 |  | -94.76 | -93.56 |  |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |  |
|  | 42 |  |  | -85.7 | -85.4 |  | -85.1 | -84.9 | TDD |  |
| CA_3A-31A12,13 | 3 |  |  | -86.9 | -86.4 | -86 |  | -85.6 | FDD |
|  | 31 |  | -95.5 | -93.3 |  |  |  |  |  |
| CA_3A-42A9,10 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_3A-42A11 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_4A-5A-12A5,6 | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 5 |  |  | -97.5 | -94.5 |  |  |  |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_4A-7A-12A5,6 | 4 |  |  | [-90] | [-89.5] | [-89] |  | [-88.5] | FDD |
|  | 7 |  |  | -97.5 | -94.5 |  |  |  |  |
|  |  |  |  | -100.220 | -97.220 |  |  |  |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_4A-12A5,6 | 4 | -89.2 | -89.2 | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 12 |  | -98.2 | -96.5 | -93.5 |  |  |  |  |
| CA_4A-12A-12A5,6 | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_4A-12A-30A5,6 | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |  |
| CA_4A-17A5,6 | 4 |  |  | -90 | -89.5 |  |  |  | FDD |
|  | 17 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_4A-28A5,6 | 4 |  |  | -89.8 | -89.4 | -89 |  | -88.7 | FDD |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 |  | -90.8 |  |
| CA_5A-12A-66A5,6 | 5 |  |  | -97.5 | -94.5 |  |  |  | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
|  | 66 |  |  | -90 | -89.5 | -89 |  | -88.5 |  |
| CA_5A-38A19 | 5 |  |  | N/A | N/A |  |  |  | FDD |
|  | 38 |  |  | N/A | N/A | N/A |  | N/A | TDD |
| CA_7A-8A5,6 | 7 |  |  | -88 | -87.4 | -87 |  | -86.7 | FDD |
|  | 8 |  | -99 | -96.8 | -93.8 |  |  |  |  |
| CA_7A-8A-20A5,6 | 7 |  |  |  | -87.4 | -87 |  | -86.7 | FDD |
|  | 8 |  | -99 | -96.8 | -93.8 |  |  |  |  |
|  | 20 |  |  | [-96.8] | [-93.8] |  |  |  |  |
|  |  |  |  | [-99.5]20 | [-96.5]20 |  |  |  |  |
| CA_7A-20A-38A19 | 7 |  |  |  | N/A | N/A |  | N/A | FDD |
|  | 20 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 38 |  |  | N/A | N/A | N/A |  | N/A | TDD |
| CA_8A-28A-41A28 | 8 |  |  | N/A | N/A |  |  |  | FDD |
|  | 28 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 41 |  |  | N/A | N/A | N/A |  | N/A | TDD |
| CA_8A-41A8 | 8 | N/A | N/A | N/A | N/A |  |  |  | FDD |
|  | 41 |  |  | N/A | N/A | N/A |  | N/A | TDD |
| CA_8A-42A12,13 | 8 | -102 | -99 | -96.8 | -93.8 |  |  |  | FDD |
|  | 42 |  |  | -84.8 | -84.7 | -84.6 |  | -84.5 | TDD |
| CA_11A-28A | 11 |  |  | -75.2 | -75.2 |  |  |  | FDD |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 |  | -90.8 | FDD |
| CA_12A-30A-66A5,6 | 12 |  |  | -96.5 | -93.5 |  |  |  | FDD |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |  |
|  | 66 |  |  | -89.5 | -89 | -88.5 |  | -88 |  |
| CA_12A-66A5,6 | 12 |  | -98.2 | -96.5 | -93.5 |  |  |  | FDD |
|  | 66 | -88.7 | -88.7 | -89.5 | -89 | -88.5 |  | -88 |  |
| CA_12B-66A5,6 | 12 |  |  | -96.5 | -93.5 |  |  |  | FDD |
|  | 66 |  |  | -89.5 | -89 | -88.5 |  | -88 |  |
| CA_20A-38A19 | 20 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 38 |  |  | N/A | N/A | N/A |  | N/A | TDD |
| CA_20A-40A15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | -60.7 | FDD |
|  | 40 |  |  | -100 | -97 | -95.2 |  | -94 | TDD |
| CA_21A-28A4,21 | 21 |  |  | N/A | N/A | N/A |  |  | FDD |
|  | 28 |  |  | N/A | N/A |  |  |  |  |
| CA_21A-28A-42A4,21 | 21 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 28 |  |  | N/A | N/A | N/A |  |  |  |
|  | 42 |  |  | N/A | N/A |  |  |  |  |
| CA_21A-28A-42C4,21 | 21 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 28 |  |  | N/A | N/A | N/A |  |  |  |
|  | 42 |  |  | N/A | N/A |  |  |  |  |
| CA_26A-41A8 | 26 |  |  | N/A | N/A | N/A |  |  | FDD |
|  | 41 |  |  | N/A | N/A | N/A |  | N/A | TDD |
| CA_28A-40A15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 |  | -60.7 | FDD |
|  | 40 |  |  | -100 | -97 | -95.2 |  | -94 | TDD |
| CA_28A-41A-42A17,18 | 28 |  |  | -98.3 | -95.3 |  |  |  | FDD |
|  | 41 |  |  |  | -94.6 | -92.8 |  | -91.6 |  |
|  | 42 |  |  |  | -85.4 | -85.1 |  | -84.9 | TDD |
| CA_28A-42A17,18 | 28 |  |  | -98.3 | -95.3 | -93.5 |  | -92.3 | FDD |
|  | 42 |  |  | -85.7 | -85.4 | -85.1 |  | -84.9 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 7: Void. NOTE 8: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_1A-3A-42A, CA_3A-19A-42A, and CA_1A-3A-19A-42A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 12: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 13: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 14: For the UE that supports CA_1A-18A-28A or CA_1A-19A-28A, no requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 15: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 16: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 17: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 18: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 19: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 20: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 21: No requirements apply when there is at least one individual RE on band 28 uplink outside frequencies 728 – 738 MHz. The reference sensitivity is only verified when all configured RE’s are confined within frequencies 728 – 738 MHz (the requirements specified in clause 7.3.1 of [6] apply). NOTE 22: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_1A-3A-21A-42A. NOTE 23: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 24: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 25: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-19A-21A-42A. NOTE 26: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 27: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 28: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of Band 8 for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of Band 41. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |  |

# <Next change>

Table 7.3.1A-0bC: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for three bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) |  |  | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-5A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 5 |  |  | -98 |  |  | -95 |  |  |  |
| CA_1A-3A-5A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 5 |  |  | -98 |  |  | -95 |  |  |  |
| CA_1A-3A-7A9 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 7 |  |  |  |  |  | -95 | -93.2 | -92 |  |
|  |  |  |  |  |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3A-7A10 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 7 |  |  |  |  |  | -95 | -93.2 | -92 |  |
|  |  |  |  |  |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3A-7A-7A | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  | 39 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 310 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  |  |  | -95 | -93.2 | -92 |  |
|  |  |  |  |  |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3A-7C9 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  |  |  |  | -91.5 | -90 | -89 |  |
|  | 7 |  |  |  |  |  | -95 | -93.2 | -92 |  |
|  |  |  |  |  |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3A-7C10 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  |  |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  |  |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 7 |  |  |  |  |  | -95 | -93.2 | -92 |  |
|  |  |  |  |  |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3C-7C | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 39 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 310 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  |  |  | -95 | -93.2 | -92 |  |
|  |  |  |  |  |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3A-8A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 8 |  | -99.2 | -97 |  |  | -94 |  |  |  |
| CA_1A-3A-8A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 8 |  | -99.2 | -97 |  |  | -94 |  |  |  |
| CA_1A-3C-8A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 8 |  | -99.2 | -97 |  |  | -94 |  |  |  |
| CA_1A-3C-8A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 8 |  | -99.2 | -97 |  |  | -94 |  |  |  |
| CA_1A-3A-11A4 | 1 |  |  |  | -100 | -97 |  | -95.2 | -94 | FDD |
|  |  |  |  |  | -102.711 | -99.711 |  | -97.911 | -96.711 |  |
|  | 3 |  |  |  | -94 | -91.5 |  | -90 | -89 |  |
|  | 11 |  |  |  | -100 | -97 |  |  |  |  |
| CA_1A-3A-11A5 | 1 |  |  |  | -100 | -97 |  | -95.2 | -94 | FDD |
|  |  |  |  |  | -102.711 | -99.711 |  | -97.911 | -96.711 |  |
|  | 3 |  |  |  | -97 | -94 |  | -92.2 | -91 |  |
|  |  |  |  |  | -99.711 | -96.711 |  | -94.911 | -93.711 |  |
|  | 11 |  |  |  | -100 | -97 |  |  |  |  |
| CA_1A-3A-19A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 19 |  |  | -100 |  |  | -97 | -95.2 |  |  |
| CA_1A-3A-19A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 19 |  |  | -100 |  |  | -97 | -95.2 |  |  |
| CA_1A-3A-20A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 20 |  |  | -97 |  |  | -94 | -91.2 | -90 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -93.911 | -92.711 |  |
| CA_1A-3A-20A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 20 |  |  | -97 |  |  | -94 | -91.2 | -90 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -93.911 | -92.711 |  |
| CA_1A-3A-21A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 21 |  |  | -100 |  |  | -97 | -95.2 |  |  |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 |  |  |
| CA_1A-3A-21A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 21 |  |  | -100 |  |  | -97 | -95.2 |  |  |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 |  |  |
| CA_1A-3A-26A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 26 |  |  | -97.57 |  |  | -94.57 |  |  |  |
| CA_1A-3A-26A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 26 |  |  | -97.57 |  |  | -94.57 |  |  |  |
| CA_1A-3A-28A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 28 |  |  | -98.3 |  |  | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-28A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 28 |  |  | -98.3 |  |  | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-42A4 | 1 |  |  | -99.8 |  |  | -96.8 | -95 | -93.8 | FDD |
|  |  |  |  | -102.511 |  |  | -99.511 | -97.711 | -96.511 |  |
|  | 3 |  |  | -93.8 |  |  | -91.3 | -89.8 | -88.8 |  |
|  | 42 |  |  | -98.5 |  |  | -95.5 | -93.7 | -92.5 | TDD |
|  |  |  |  | -100.711 |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3A-42A5 | 1 |  |  | -99.8 |  |  | -96.8 | -95 | -93.8 | FDD |
|  |  |  |  | -102.511 |  |  | -99.511 | -97.711 | -96.511 |  |
|  | 3 |  |  | -96.8 |  |  | -93.8 | -92 | -90.8 |  |
|  |  |  |  | -99.511 |  |  | -96.511 | -94.711 | -93.511 |  |
|  | 42 |  |  | -98.5 |  |  | -95.5 | -93.7 | -92.5 | TDD |
|  |  |  |  | -100.711 |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3A-42C4 | 1 |  |  | -99.8 |  |  | -96.8 | -95 | -93.8 | FDD |
|  |  |  |  | -102.511 |  |  | -99.511 | -97.711 | -96.511 |  |
|  | 3 |  |  | -93.8 |  |  | -91.3 | -89.8 | -88.8 |  |
|  | 42 |  |  | -98.5 |  |  | -95.5 | -93.7 | -92.5 | TDD |
|  |  |  |  | -100.711 |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3A-42C5 | 1 |  |  | -99.8 |  |  | -96.8 | -95 | -93.8 | FDD |
|  |  |  |  | -102.511 |  |  | -99.511 | -97.711 | -96.511 |  |
|  | 3 |  |  | -96.8 |  |  | -93.8 | -92 | -90.8 |  |
|  |  |  |  | -99.511 |  |  | -96.511 | -94.711 | -93.511 |  |
|  | 42 |  |  | -98.5 |  |  | -95.5 | -93.7 | -92.5 | TDD |
|  |  |  |  | -100.711 |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-18A-28A6 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 18 |  |  | -100 |  |  | -97 | -95.2 |  |  |
|  | 28 |  |  | -94 |  |  | -92.5 |  |  |  |
| CA_1A-19A-28A8 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 19 |  |  | -100 |  |  | -97 | -95.2 |  |  |
|  | 28 |  |  | -94 |  |  | -92 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the Bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the Bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: These requirements apply when the uplink is active in Band 18 and the downlink channels in Band 28 are confined within the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 18. NOTE 7: 7 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 8: These requirements apply when the uplink is active in Band 19 and the downlink channels in Band 28 are allocated at the middle of the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 19. NOTE 9: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 7, the requirement applies regardless of channel bandwidth in Band 1. NOTE 10: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 7, the requirement applies regardless of channel bandwidth in Band 1. NOTE 11: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |

# <Next change>

# Table 7.3.1A-0bE: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |  |
| CA_1A-3A-5A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3A-5A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-7A-20A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 7 |  |
|  |  |  |  | -102.511 | -99.511 | -97.711 | -96.511 |  |  |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |  |  |
|  |  |  |  | -99.511 | -96.511 | -94.711 | -93.511 |  |  |  |
|  | 7 |  |  |  | -94.8 | -93 | -91.8 |  |  |  |
|  |  |  |  |  | -97.711 | -95.911 | -94.711 |  |  |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -93.911 | -92.711 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
| CA_1A-3A-7A-20A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 42 |  |
|  |  |  |  | -102.511 | -99.511 | -97.711 | -96.511 |  |  |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |  |  |
|  |  |  |  | -99.511 | -96.511 | -94.711 | -93.511 |  |  |  |
|  | 7 |  |  |  | -93 | -91.3 | -90.1 |  |  |  |
|  |  |  |  | -97 | -94 | -91.2 | -90 |  |  |  |
|  | 20 |  |  | -99.711 | -96.711 | -93.911 | -92.711 |  |  |  |
|  |  |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |  |  |
|  | 42 |  |  | -100.711 | -97.711 | -96.911 | -94.711 | TDD |  |  |
| CA_1A-3A-7A-40A15 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  |  |  |  |  | -97.411 | -95.611 | -94.411 |  |  |  |
|  | 40 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-40A16 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  |  |  |  |  | -97.411 | -95.611 | -94.411 |  |  |  |
|  | 40 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  |  |  |  |  | -97.411 | -95.611 | -94.411 |  |  |  |
|  | 40 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD |  |  |
| CA_1A-3A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 7 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  |  |  |  |  | -97.411 | -95.611 | -94.411 |  |  |  |
|  | 40 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |  |
| CA_1A-3A-7A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 7 |  |
|  |  |  |  | -102.511 | -99.511 | -97.711 | -96.511 |  |  |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |  |  |
|  |  |  |  | -99.511 | -96.511 | -94.711 | -93.511 |  |  |  |
|  | 7 |  |  |  | -94.8 | -93 | -91.8 |  |  |  |
|  |  |  |  |  | -97.511 | -95.711 | -94.511 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 42 |  |
|  |  |  |  | -102.511 | -99.511 | -97.711 | -96.511 |  |  |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |  |  |
|  |  |  |  | -99.511 | -96.511 | -94.711 | -93.511 |  |  |  |
|  | 7 |  |  |  | -93 | -91.3 | -90.1 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |  |
| CA_1A-3A-7A-40A | 1 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 7 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |  |
| CA_1A-3A-8A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3A-8A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-38A12 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 1 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |  |
|  | 3 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |  |
|  | 38 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-38A13 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 1 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |  |
|  | 3 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |  |
|  | 38 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |  |  |
| CA_1A-3A-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3A-40C | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |  |  |
| CA_1A-3C-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3C-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3C-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |  |  |
| CA_1A-3A-41A12,14 | 3 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |  |
|  | 41 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 41 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-41A13,14 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 41 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 41 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3C-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |
|  | 40 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD |  |
| CA_1A-3C-40C | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_1A-3C-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |  |
| CA_1A-5A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -91.9 | -90.4 | -89.4 | TDD |  |  |
| CA_1A-5A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-7A-20A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 7 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |  |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |  |  |
|  | 20 |  |  | -96.8 | -93.8 | -91 | -89.8 |  |  |  |
|  |  |  |  | -99.511 | -96.511 | -93.711 | -92.511 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
| CA_1A-7A-20A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 42 |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 |  |  |  |
|  | 20 |  |  | -96.8 | -93.8 | -91 | -89.8 |  |  |  |
|  |  |  |  | -99.511 | -96.511 | -93.711 | -92.511 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |  |
| CA_1A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 7 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |  |
| CA_1A-7A-40A | 1 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 7 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |  |
| CA_1A-7A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 7 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |  |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 42 |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |  |
| CA_1A-8A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |  |  |
| CA_1A-8A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 40 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |  |  |
| CA_1A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 40 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |  |  |
| CA_1A-40C | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-5A-40A | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  | -101.411 | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 5 |  | -100.2 | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-5A-40A | 3 |  | [-95.3] | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 5 |  | -100.2 | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-7A-38A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 7 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] |  |  |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3A-7A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 7 |  |  | -97.7 | -94.7 | -92.9 | -91.7 |  |  |  |
|  |  |  |  | -100.411 | -97.411 | -95.611 | -94.411 |  |  |  |
|  | 40 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD |  |  |
| CA_3A-7A-40A | 3 |  |  | -100 | -97 | -95.2 | -94 | FDD | 7 |  |
|  | 7 |  |  | -97.7 | -94.7 | -92.9 | -91.7 |  |  |  |
|  |  |  |  | -100.411 | -97.411 | -95.611 | -94.411 |  |  |  |
|  | 40 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |  |
| CA_3A-7A-40A | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 7 |  |  | -96.8 | -94 | -92.4 | -91.2 |  |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |  |
| CA_3A-7A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 7 |  |  | -97.7 | -94.7 | -92.9 | -91.7 |  |  |  |
|  |  |  |  | -100.411 | -97.411 | -95.611 | -94.411 |  |  |  |
|  | 40 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD |  |  |
| CA_3A-7A-40C | 3 |  |  | -100 | -97 | -95.2 | -94 | FDD | 7 |  |
|  | 7 |  |  | -97.7 | -94.7 | -92.9 | -91.7 |  |  |  |
|  |  |  |  | -100.411 | -97.411 | -95.611 | -94.411 |  |  |  |
|  | 40 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |  |
| CA_3A-7A-40C | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 7 |  |  | -96.8 | -94 | -92.4 | -91.2 |  |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |  |
| CA_3A-7A-42A | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD | 7 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |  |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 3 |  |  | [-96.6] | [-93.6] | [-91.8] | [-90.6] | FDD | 42 |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |  |
| CA_3A-8A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-8A-40A | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-20A-32A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -93.911 | -92.711 |  |  |  |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |  |  |
| CA_3A-28A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-28A-40A | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-28A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 28 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |  |
| CA_3A-28A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-28A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 28 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |  |
| CA_3A-28A-40C | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-28A-41A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  |  |  |  | -100.211 | -97.211 | -95.411 | -94.211 |  |  |  |
| CA_3A-40A | 3 | -101.7 | -98.7 | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-40A | 3 | [-97.4] | [-95.3] | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
| CA_3A-40A-40A | 3 |  |  | -97 | -94 |  |  | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 |  |  |  |  |  |
|  | 40 |  |  |  | -92.9 |  | -90.2 | TDD |  |  |
| CA_3A-40A-40A | 3 |  |  | -94.2 | -91.2 |  |  | FDD | 40 |  |
|  | 40 |  |  |  | -97 |  | -94 | TDD |  |  |
|  |  |  |  |  | -99.711 |  | -96.711 |  |  |  |
| CA_3A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-40C | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
| CA_3C-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3C-40A | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
| CA_3C-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3C-40C | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
| CA_3A-41A5 | 3 |  | [-95.3] | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  |  |  |  | -100.211 | -97.211 | -95.411 | -94.211 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  | -98.7 | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3A-41C5 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  |  |  |  | -100.211 | -97.211 | -95.411 | -94.211 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3A-41D5 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3C-41A5 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3C-41C5 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3C-41D5 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
| CA_3A-41A-42A5,6,7,8 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41A-42A5,6,9 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41A-42A5,6,10 | 3 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  |  |  |  | -100.211 | -97.211 | -95.411 | -94.211 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |  |
| CA_3A-41A-42C5,6,7,8 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 41 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  |  | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41A-42C5,6,9 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 41 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  |  | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41A-42C5,6,10 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 42 |  |
|  | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-41A-42C5,6,10 | 3 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
|  | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-41C-42A5,6,7,8 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 41 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  |  | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41C-42A5,6,9 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 41 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  |  | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41C-42A5,6,10 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 42 |  |
|  | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-41C-42A5,6,10 | 3 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
|  | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-41C-42C5,6,7,8 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 | -71.7 |  |  |
| CA_3A-41C-42C5,6,9 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 | -92.5 |  |  |
| CA_3A-41C-42C5,6,10 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 42 |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |
| CA_3A-41C-42C5,6,10 | 3 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |
| CA_7A-20A-42A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |  |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -93.911 | -92.711 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -93.911 | -92.711 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |  |
| CA_7A-40A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |  |  |
|  | 40 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD |  |  |
|  | 7 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
|  | 40 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | TDD |  |  |
| CA_7A-40C | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |  |  |
|  | 40 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD |  |  |
|  | 7 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
|  | 40 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | TDD |  |  |
| CA_7A-42A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |  |
| CA_7A-42A-42A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |  |
| CA_28A-40A | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD | 28 |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |  |
| CA_28A-40A | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_28A-40C | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD | 28 |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |  |
| CA_28A-40C | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_28A-40D | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD | 28 |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 |  |  |  |
| CA_28A-40D | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply regardless of the channel bandwidth and the location of UL band. NOTE 5: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 6: The antenna isolation for MSD calculation is assumed as 10 dB. For conducted mode REFSENS test such antenna isolation is not observed as the antennas are disconnected. Additionally antenna isolation assumption is under discussion depending on the frequency range NOTE 7: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-42C, CA_1A-3A-42A, CA_1A-3A-42C, CA_3A-19A-42A, CA_3A-19A-42C and CA_1A-3A-19A-42A, CA 3A-41A-42A. NOTE 8: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 9: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 11: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 12: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 13: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 14: The B41 requirements also apply to the supported CA_1A-41A. NOTE 15: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1 NOTE 16: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. |  |  |  |  |  |  |  |  |  |  |

# <Next change>

Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-5A-7A-46A CA_1A-5A-7A-46C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.741 | -99.74 | -97.94 | -96.74 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  |  |  |  |  | -97.74 | -95.94 | -94.74 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-5A-46A CA_1A-5A-46C CA_1A-5A-46D | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.741 | -99.74 | -97.94 | -96.74 |  |
|  | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-7A-46A CA_1A-7A-46C CA_1A-7A-46D | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.741 | -99.74 | -97.94 | -96.74 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  |  |  |  |  | -97.74 | -95.94 | -94.74 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-46A CA_1A-46C CA_1A-46D CA_1A-46E | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.74 | -99.74 | -97.94 | -96.74 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_2A-46A CA_2A-46C CA_2A-46A-46A CA_2A-46A-46C CA_2A-46D CA_2A-46A-46D | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  |  |  |  | -100.74 | -97.74 | -95.94 | -94.74 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-46A CA_3A-46C CA_3A-46D CA_3A-46E | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
|  |  |  |  | -99.74 | -96.74 | -94.94 | -93.74 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_4A-46A CA_4A-46C CA_4A-46A-46A CA_4A-46A-46C CA_4A-46D CA_4A-46A-46D | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-7A-46A CA_5A-7A-46C CA_5A-7A-46D | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  |  |  |  |  | -97.74 | -95.94 | -94.74 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-46A CA_5A-46C CA_5A-46D CA_5A-46E | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_7A-46A CA_7A-46C CA_7A-46D CA_7A-46E | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  |  |  |  | -100.74 | -97.74 | -95.94 | -94.74 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_8A-46A CA_8A-46D CA_8A-46E CA_8B-46C CA_8B-46D | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_8A-46C | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_8B-46A | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46A | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46C | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46D | 11 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46E | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_13A-46A CA_13A-46D CA_13A-46E | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_13A-46C | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_19A-46A CA_19A-46C CA_19A-46D CA_19A-46E | 19 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_21A-46A CA_21A-46C CA_21A-46D CA_21A-46E | 21 |  |  | -100 | -97 | -95.2 |  | FDD |
|  |  |  |  | -102.74 | -99.74 | -97.94 |  |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_26A-46A | 26 |  | -99.7 | -97.57 | -94.57 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_28A-46A CA_28A-46C CA_28A-46D CA_28A-46E | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_39A-46A | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39A-46C | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39C-46A | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39A-46E CA_39C-46D | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 |  |
| CA_40A-46A CA_40A-46D CA_40A-46E CA_40C-46C CA_40C-46D CA_40D-46A CA_40D-46C | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_40A-46C | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_40C-46A | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_41A-46A | 41 |  |  | -98 | -95 | -93.2 | -92 | TDD |
|  |  |  |  | -100.74 | -97.74 | -95.94 | -94.74 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_42A-46A | 42 |  |  | -99 | -96 | -94.2 | -93 | TDD |
|  |  |  |  | -100.74 | -97.74 | -95.94 | -94.74 |  |
|  | 46 |  |  |  |  |  | -83 | TDD |
| CA_46A-66A CA_46A-46A-66A CA_46A-46C-66A CA_46A-46D-66A CA_46C-66A CA_46D-66A CA_46E-66A | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The signal power is specified per port. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 5: The requirement for B46 does not apply when there is at least one individual RE within the B46 downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 7.3.1A-0eC. NOTE 6: Void NOTE 7: 7 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 8: When Band 46 have self interference problems by dual uplink CA, then the requirements not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_1A-5A-46A; CA_1A-5A; 2*fc_1A + 2*fc_5A; 2*BW_1A + 2*BW_5A. CA_1A-7A-46A; CA_1A-7A; 3*fc_7A - fc_1A; 3*BW_7A – BW_1A. CA_5A-7A-46A; CA_5A-7A; 2*fc_7A + fc_5A; 2*BW_7A + BW_5A. |

# <End of changes>