3GPP TSG-RAN WG4 #82	R4-1701625

Athens, Greece, 13–17 February 2017

Source: 		Ericsson, Qualcomm, Huawei, Nokia, Alcatel-Lucent Shanghai Bell, Skyworks, AT&T, Verizon

Title:  	Traffic test cases related to multi-node tests for Rel-13 LAA

Agenda Item:	6.1.1

Document for:	Approval

# Introduction

During RAN plenary in June 2016, a new study item has been approved in [1]. The coexistence tests are divided in two parts: LBT functional tests and multi-node tests. LBT functionalities were discussed and agreed in RAN4#79 meeting and further updated later. LBT functional tests are specified in TS 36.141, while the multi-node tests are captured in TR 36.789.

Previously, three way forwards [2],[3],[4] have been agreed on coexistence tests for Rel-13 LAA in RAN4#78bis, RAN4#79 and RAN4#80, respectively. In RAN4#81, a number of contributions related to many aspects of multi-node tests for LAA has been discussed extensively, as listed in [9], [10], [11], [12], [13], [14], [15], [16].

In this contribution, we provide discussions related to different traffic test cases and our proposals related to this for multi-node tests.

# Traffic test cases for multi-node tests

The latest agreements are described in [7]. In the following sections we provide our view on the remaining issues which are still open for discussions

## Considering both best effort and voice traffic separately

According to our understanding, when best effort traffic is tested in aggressor link, then it is important to test both best effort and voice traffic in the victim link.

Scenario 1 and 2 in the table below relates to 802.11 victim devices when LAA devices are aggressors. As agreed in [7], Scenario 3 and 4 are mirror scenarios with respect to scenario 1 and 2, respectively. The purpose of the tests in scenario 3 and 4 when IEEE 802.11 is the aggressor is to help 3GPP to validate LAA performance and enhancement of system performance; the test procedures will be exactly the same as the ones defined for scenario 1 and 2, although there will be no pass/fail criterial for IEEE 802.11. Note that, 802.11 technologies considered here are 802.11n and 802.11ac.

| Scenario | Case | Victim DUT | Companion victim DUT | Aggressor DUT | Companion aggressor DUT | Traffic in victim link | Traffic in aggressor link |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Baseline (1a) | 802.11 AP | 802.11 STA | 802.11 AP | 802.11 STA | Best effort | Best effort |
|  | Test (1a) | 802.11 AP | 802.11 STA | LAA eNB | LAA UE | Best effort | Best effort |
|  | Baseline (1b) | 802.11 AP | 802.11 STA | 802.11 AP | 802.11 STA | Voice | Best effort |
|  | Test (1b) | 802.11 AP | 802.11 STA | LAA eNB | LAA UE | Voice | Best effort |
| 2 | Baseline (2a) | 802.11 AP | 802.11 STA | 802.11 AP | 802.11 STA | Voice | Voice |
|  | Test (2a) | 802.11 AP | 802.11 STA | LAA eNB | LAA UE | Voice | Voice |
|  | Baseline (2b) | 802.11 AP | 802.11 STA | 802.11 AP | 802.11 STA | Best effort | Voice |
|  | Test (2b) | 802.11 AP | 802.11 STA | LAA eNB | LAA UE | Best effort | Voice |
| 3 | Baseline (3a) | LAA eNB | LAA UE | LAA eNB | LAA UE | Best effort | Best effort |
|  | Test (3a) | LAA eNB | LAA UE | 802.11 AP | 802.11 STA | Best effort | Best effort |
|  | Baseline (3b) | LAA eNB | LAA UE | LAA eNB | LAA UE | Voice | Best effort |
|  | Test (3b) | LAA eNB | LAA UE | 802.11 AP | 802.11 STA | Voice | Best effort |
| 4 | Baseline (4a) | LAA eNB | LAA UE | LAA eNB | LAA UE | Voice | Voice |
|  | Test (4a) | LAA eNB | LAA UE | 802.11 AP | 802.11 STA | Voice | Voice |
|  | Baseline (4b) | LAA eNB | LAA UE | LAA eNB | LAA UE | Best effort | Voice |
|  | Test (4b) | LAA eNB | LAA UE | 802.11 AP | 802.11 STA | Best effort | Voice |
| Note: IEEE 802.11 technologies considered here are 802.11n and 802.11ac |  |  |  |  |  |  |  |

Based on the above discussions, we propose the following:

Proposal-1: Agree on traffic cases for multi-node tests as described in Section 2.1

## Regarding time-multiplexed mixed best effort and VOIP traffic at the aggressor link

One other alternative in addition to traffic cases in Section 2.1 could be to consider time-multiplexed mixed best effort and VoIP traffic in the aggressor link, while considering any traffic type in the victim link.

As expressed in [9], this kind of traffic case in the aggressor link may be needed to verify whether “if the aggressor adheres to the regulations that specify the channel access rules for carrying a mix of higher and lower priority traffic. Specifically they are needed to test that the aggressor does not piggyback lower priority data with higher priority data in a COT that is obtained using the access parameters of the higher priority data.” [9].

The above mentioned test is an issue under LBT functionality tests, and this is not related to co-existence tests as far as we have understood. In the LBT functionality test, channel priority tests are performed based on the number of priority classes that the BS declares. So, when channel access priority test is done, then it is ensured that only high priority data is transmitted from LAA BS in a COT when the channel is obtained using access parameters related to higher priority class.

Thus, our understanding is that, if the traffic test cases as listed in in Section 2.1 are tested properly, then all the necessary coexistence performance with respect to traffic types can be investigated. Thus, we observe that:

Observation-1: The mixed traffic type at the aggressor is an LBT functionality issue, which is tested under basic LBT functionality tests

Observation-2: We will not get any additional information compared to test cases in section 2.1 with respect to coexistence behavior

Thus, we propose not to consider mixed traffic cases, where voice and best efforts are multiplexed in time in the aggressor link. This would only complicate the tests and not really add any additional information compared to tests mentioned above.

Proposal-2: Exclude the traffic case when time multiplexed best effort and voice traffic is applied in aggressor link.

# Conclusion

In this paper, we have described our proposals on traffic test cases related to the multi-node throughput tests. The proposals are listed below:

Proposa-1: Agree on traffic cases for multi-node tests as described in Section 2.1

Regarding possibility to use time multiplexed mixed traffic at the aggressor, we observe the following:

Observation-1: The mixed traffic type at the aggressor is an LBT functionality issue, which is tested under basic LBT functionality tests

Observation-2: We will not get any additional information compared to test cases in section 2.1 with respect to coexistence behavior

Based on the above observation, we propose the following:

Proposal-2: Exclude the traffic case when time multiplexed best effort and voice traffic is applied in aggressor link.

Based on these proposals and based on the discussions above, we propose a TP in [6].

# References

RP-161197, New Study Item proposal: Multi-node tests for LAA, Huawei, HiSilicon, Ericsson, Qualcomm, Nokia.

R4-162989, “Way Forward on LAA coexistence tests”, Ericsson, Huawei, Qualcomm, Nokia, RAN4 #78bis.

R4-164971, WF on LAA co-existence testing, Source: Qualcomm, Ericsson, Huawei, Nokia, RAN4#79.

R4-166971, WF on LAA Multi-node test, Huawei, Ericsson, Qualcomm, Nokia.

R4-166542, Details on multi-node tests for Rel-13 LAA, Source: Ericsson.

R4-17xxxxx, TP on traffic test cases for TR36.789: Multi-node tests for LAA.

R4-168893, Way Forward on additional issues related to multi-node tests for LAA, Ericsson, Qualcomm Incorporated, Huawei, Nokia, Verizon.

Presentation in WFA Workshop on Coexistence, “LTE-U Forum Way Forward on WFA Test Plan”, Source companies: Verizon, Ericsson, Samsung, Nokia, LGE, T-Mobile, Qualcomm

R4-1609968, Remaining issues in multi-node tests for LAA Wi-Fi coexistence, Broadcom, CableLabs, Hewlett Packard Enterprise, Marvell, Microsoft, Orange

R4-1609363	Remaining issues on multi-node testing, Huawei

R4-1609039	Implications of Wi-Fi Field Measurements for Multi-Node Testing, Hewlett-Packard Enterprise

R4-1609607	On Multi-node tests pass/fail criteria, Nokia, Alcatel-Lucent Shanghai Bell

R4-1609973	Device selection and configuration for LAA multi-node tests, BROADCOM CORPORATION

R4-1609983	Received SIR in multi-node tests for LAA Wi-Fi coexistence, BROADCOM CORPORATION

R4-1610139	Traffic type in LAA multi-node test, Nokia, Alcatel-Lucent Shanghai Bell

R4-1610140	Simulation results in support of SIR levels for LAA multi-node tests , Nokia, Alcatel-Lucent Shanghai Bell