3GPP TSG-RAN WG4 Meeting RAN4#82	            R4-1702043

Athens, Greece, 13 - 17 February, 2017

Agenda item:	10.4.2.5- Receiver characteristics

Source: 	Intel Corporation

Title: 	Out of band blocking for UE receivers

Document for:	Approval

# Introduction

Out of band blocking is an important RX characteristic for UE to ensure selectivity towards wanted RX signal in the presence of interfering blocker signals. This paper proposes templates for out of band blocking tables similar to [1] for new radio standard [2].

# Discussion on out of band blocking scenarios

Out of band blocker signals could be present from either of the following sources:

- TX emission from nearby UEs within the same cell

- TX emission from adjacent BS

- TX emission from connectivity radios like Wi-Fi

- TX emission from non-mobile phone ecosystem like V2V, V2X, and M2M and so on

In general, these emissions are filtered by RF front end (RFFE) SAW/FBAR/BAW filters. But     in new radio standard, where sub-6 GHz and above 6 GHz frequency bands are discussed, the availability of acoustic filters mentioned above is sceptical/ not-possible. The natural solution is to find alternate methods to achieve RFFE filtering which may not offer sufficient rejection of blocker signal frequencies if blocker signal power defined in [1] are preserved for [2]. The expectation is that enough amount of relaxation from approach in [1] should be carried out while defining the values for [2]. This document proposes template for tables in order to initiate the discussion.

Proposal 1: RAN4 should study the new radio ecosystem aspects before defining blocker signal frequency range and power values for out of band blocking.

Proposal 2: RAN4 should propose different out-of-band blocking requirements for NR bands:

- Sub - 6GHz as seen in table 1 & 2.

- Above - 6 GHz as seen in tables 3 & 4.

| Rx Parameter | Units | Channel bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 20 MHz | 40 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |
|  |  | 9 | 12 | 16 |

Table 1. Out-of-band blocking parameters (sub-6 GHz)

In table 1, Channel bandwidth (CBW) starts from 20 MHz because of re-farming from LTE.

Table 2. Out-of-band blocking (sub-6 GHz)

In table 2, the step size used for sweeping OOB frequencies should be 1 MHz.

| Rx Parameter | Units | Channel bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |
|  |  | 16 | 19 | 22 |

Table 3. Out-of-band blocking parameters (above-6 GHz)

Table 4. Out-of-band blocking (above-6 GHz)

In table 4, the step size used for sweeping OOB frequencies should be:

- 1 MHz for:

Frequency range from 1 MHz to 6000 MHz

H2, H3, f/2, f/3 (harmonics and sub-harmonics of the NR mmW frequency band)

- 10 MHz in rest of the frequency range.

If required, individual exceptions should be considered for values proposed above. The motivation is to initiate the discussion and take the topic further by agreement with other companies.

# Conclusion

Proposal 1: RAN4 should study the new radio ecosystem aspects before defining blocker signal frequency range and power values for out of band blocking.

Proposal 2: RAN4 should propose different out-of-band blocking requirements for NR bands:

- Sub - 6GHz as seen in table 1 & 2.

- Above - 6 GHz as seen in tables 3 & 4.

# References

- 3GPP TS 36.101 V14.2.1 (2017-01)

- 3GPP TR 38.803 V1.1.0 (2017-01)