3GPP TSG-RAN WG4 #82	R4-1702086

Athens, Greece, 13th – 17th February 2017

Source: 	Ericsson

Title:  	TP for NR Rel-14 TR 38.803: Noise figure in mm-wave systems as assumed for ITU-R related coexistence simulations

Agenda Item:	10.7

Document for:	Approval

# Introduction

The draft skeleton of technical report of NR studies on RF and coexistence aspects has been approved in RAN4#79 in [1]. The latest version of the TR 38.803 is approved in December RAN plenary [2].

In this contribution, we propose texts related to noise figure values of mmwave technologies for TR 38.803.

# Text proposal

The following text proposal is related to Section 6.1.

<<<<< START of TEXT PROPSOAL >>>>>

### 5.2.8	Other simulation parameters

| Parameters | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Channel bandwidth | 200MHz | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (DL) | 200MHz | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (UL) | 200MHz | 200MHz | 200MHz |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam | Same as the number of BS beam |
| The number of active UE (UL) | Same as the number of BS beam | Same as the number of BS beam | Same as the number of BS beam |
| Traffic model | Full buffer | Full buffer | Full buffer |
| DL power control | NO | NO | NO |
| UL power control | YES | YES | YES |
| BS max TX power in dBm | 23dBm | 43dBm | 33dBm |
| UE max TX power in dBm | 23dBm | 23dBm | 23dBm |
| UE min TX power in dBm | -40dBm | -40dBm | -40dBm |
| BS Noise figure in dB | note 1 | note 1 | note 1 |
| UE Noise figure in dB | note 1 | note 1 | note 1 |
| Handover margin | 3dB | 3dB | 3dB |
| note 1: For deriving ACIR/ACS values which are included in respond to WP5D, following NF are used in co-existence simulation study for both UE and BS. 30GHz: 9 and 11 dB, 45GHz: 11 and 13 dB, 70GHz: 13 and 15 dB. |  |  |  |

### 5.2.9 Noise figure

It is assumed that the performance differs less between UE and BS for mm-waves on transceiver level compared to lower frequencies below 6 GHz. The estimated NF for both BS and UE are used as same values for the ITU-R related coexistence simulations.

Two sets of noise figure values have been used for the simulations, as shown below in Table 5.2.9-1.

Table 5.2.9-1: Two sets of noise figure values for coexistence simulations

| Frequency | Set 1 |  | Set 2 |  |
| --- | --- | --- | --- | --- |
|  | UE | BS | UE | BS |
| 30GHz | 9dB | 9dB | 11dB | 11dB |
| 45GHz | 11dB | 11dB | 13dB | 13dB |
| 70GHz | 13dB | 13dB | 15dB | 15dB |

The following noise figure values are finally agreed for reporting to ITU WP5D sharing studies, as shown in Table 5.2.9-2.

Table 5.2.9-2: Noise figure values for ITU WP5D response

| Frequency | UE | BS |
| --- | --- | --- |
| 30GHz | 10dB | 10dB |
| 45GHz | 12dB | 12dB |
| 70GHz | 14dB | 14dB |

These NF values in Table 5.2.9-2 shall be used only for WP5D response. Further study on the actual noise figure to be used to define RF requirements for UE and BS shall be performed in the WI phase.

## 5.3	Co-existence simulation methodology

Editor’s note: intended to capture specific simulation methodology. How to handle even wider channel bandwidths for NR than those for LTE-A, different RF parameters such as [TRP or EIRP or others] etc. are captured.

Adopt following simulation steps.

- Aggressor and victim network are generated.

  - UEs are distributed randomly across the network.

- UE associations: UEs are associated to base station based on coupling loss.

  - Associations are made assuming a single element at both UE and BS.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

- Throughput is computed in the victim systems without considering ACI as below:

  - ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

- Throughput is computed considering ACI as below:

  - ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

- RF parameters are determined based on the degradation cause by ACI as below:

  - LossACI=1-ThputACIThputSINGLE.

<<<<< END of TEXT PROPSOAL >>>>>

# Conclusion

We propose to adopt the above mentioned text proposal for TR 38.803.

# References

R4-164535, Draft TR skeleton for New Radio SI (TR 38.803), 38.803 v0.0.1, NTT DOCOMO, INC.

RP-162256, “TR38.803 v1.0.0 on TR for Study on New Radio Access Technology: RF and co-existence aspects”, NTT Docomo Inc.