3GPP TSG-RAN Working Group 4 Meeting #82	R4-1702233

Athens, Greece, 13th Feburary – 17th Feburary 2017

Source:	DISH Network

Title:	TP for TR 36.714-03-01: Operating bands, Channel bandwidths, Co-existence study and Delta Tib/Rib values for CA_46A-66C and CA_46A-66A-66A

Agenda item:	7.3.2

Document for:	Approval

## 1	Introduction

This contribution is a text proposal for CA_46A-66C and CA_46A-66A-66A which were approved in RAN #74 [1].  Co-existence studies for both configurations are also included.

## 2	Text Proposal for TR 36.714-03-01

----- Start of Text proposal -----

## 6.X	CA_46A-66C_BCS0 and CA_46A-66A-66A_BCS0

### 6.X.1	Operating bands for CA

Table 6.X.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_46-66 | 46 | N/A |  |  | 5150 MHz | – | 5925 MHz | TDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
| CA_46-66-66 | 46 | N/A |  |  | 5150 MHz | – | 5925 MHz | TDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: E-UTRA CA configurations and bandwidth combination sets defined for CA_46A-66C and CA_46A-66A-66A

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_46A-66C | 46 |  |  |  |  |  | Yes | 60 | 0 |
|  | 66 | See the CA_66C Bandwidth combination set 0 in the Table 5.6A.1-1 from 36.101 |  |  |  |  |  |  |  |
| CA_46A-66A-66A | 46 |  |  |  |  |  | Yes | 60 | 0 |
|  | 66 | See the CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 from 36.101 |  |  |  |  |  |  |  |

Table 6.X.2-2: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66C | 5 | 20 |  |  | 40 | 0 |
|  | 10 | 15, 20 |  |  |  |  |
|  | 15 | 10, 15, 20 |  |  |  |  |
|  | 20 | 5, 15, 10, 20 |  |  |  |  |

Table 6.X.2-3: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with two sub-blocks)

| E-UTRACA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66A-66A | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 |  |

### 6.X.3	Co-existence studies

Table 6.X.3-1 summarizes frequency ranges where harmonics occur due to Band 66 or Band 46 for UL and DL. Therefore, we can conclude that there is an issue on 3rd harmonic interference from Band 66 UL to Band 46 DL. MSD value without HTF is provided in 6.X.5.

# Table 6.X.3-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 | 4220 | 4400 | 6330 | 6600 |
| 46 | N/A |  | 5150 | 5925 | N/A |  | N/A |  | 10300 | 11850 | 15450 | 17775 |

#### 6.X.3.1	CA_46A-66C

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 66 and Band 46 DL carriers can be calculated as shown in table 6.X.3.1-1 below:

Table 6.X.3.1-1: Band 46 and Band 66 DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2110 | 2200 | 5150 | 5925 |
| 2nd harmonics frequency limits (MHz) | 4220 | 4400 | 10300 | 11850 |
| 3rd harmonics frequency limits (MHz) | 6330 | 6600 | 15450 | 17775 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 2950 | 3815 | 7260 | 8125 |
| 3rd order IMD products | (f2-low – 2*f1-high) | (f2-high – 2*f1-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 750 | 1705 | 8100 | 9740 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 9370 | 10325 | 12410 | 14050 |
| 3rd order IMD products | (f1-low – f2-high + f2-low) | (f1-high + f2-high – f2-low) | (f2-low – f1-high + f1-low) | (f2-high + f1-high – f1-low) |
| IMD frequency limits (MHz) | 1335 | 2975 | 5060 | 6015 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 2090 | 2220 | 5110 | 5965 |

It can be seen from table 6.X.3.1-1 that some 2nd IMD products caused by BS supporting carrier aggregation of Band 66 and Band 46 fall into the BS receive band of Bands 22, 42 43, and 48 while some 3rd IMD products fall into the BS receive band of Bands 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 13, 14, 18, 19, 20, 21, 23, 24, 25, 26, 27, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 44, 45, 65, 66 and 70. Note that the calculation in table 6.X.3.1-1 (except the last row) assumes the BS transmits the whole 90 and 775 MHz DL frequency of Band 66 and Band 46 respectively.

If the BS only transmits up to 40MHz and 20 MHz DL in Band 66 and Band 46 respectively as stated in Table 6.X.2-3, the 3rd IMD products may still fall into the BS receive band of the Bands 5, 6, 8, 11, 13, 14, 18, 19, 20, 21, 24, 26, 27, 44, 45, and 70 as shown in the last row in table 6.X.3.1-1.

The following Bands 6, 8, 11, 18, 19, 20, 21, 44, 45, and 70 are not used in the same region, therefore, it is recommended that Bands 66 and 46 BS transmitters should not share the same antenna, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause the BS receiver desensitisations in Band 5, 13, 14, 24, 26, 27, and 70.

#### 6.X.3.2	CA_46A-66A-66A

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of CA_46A-66A-66A DL carriers can be calculated as shown in Table 6.X.3.2-1 below:

Table 6.X.3.2-1: CA_46A-66A-66A DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 717 | 728 | 2110 | 2200 | 2110 | 2200 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 750 |  |  | 1705 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 5060 |  |  | 6015 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 5060 |  |  | 6015 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 9370 |  |  | 10325 |  |  |

It can be seen from Table 6.X.3.2-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3DL CA_46A-66A-66A may fall into the BS receive band of Bands 5, 6, 8, 11, 12 - 16, 18 – 21, 24, 26, 27, 29, and 32.

Note that Bands 8, 11, and 18 – 21 are not intended for use in the same geographical area as Bands 46 and 66. Therefore, the focus here is on the IMD falling into Bands 5, 6, 12 - 16, 24, 26, 27, 29, and 32.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 5, 6, 12 - 16, 24, 26, 27, 29, or 32 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 46 and 66 BS transmitters do not share the same antenna with Band 5, 6, 12 - 16, 24, 26, 27, 29, or 32 BS receiver..

Therefore, in order to prevent BS receiver desensitization, it is recommended that Bands 46 and 66 BS transmitters should not share the same antenna with Band 5, 6, 12 - 16, 24, 26, 27, 29, or 32 BS receiver, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 5, 6, 12 - 16, 24, 26, 27, 29, or 32 receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

### 6.X.4	∆TIB value

For CA_46A-66C and CA_46A-66A-66A combination, the TIB = 0 dB and RIB = 0 dB assuming separate antenna architecture without HTF.

### 6.X.5	REFSENS requirements

For the REFSENS of 3DL/1UL CA_46A-66A-66A, the requirements are shown in table 6.X.5-1 with the uplink configuration in table 6.X.5-2.

The REFSENS is specified in the following: Table 6.X.5-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_46A-66C | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_46A-66A-66A | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |

Table 6.X.5-2: Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_46A-66C | 66 |  |  | 25 | 50 | 75 | 100 | FDD |
| CA_46A-66A-66A | 66 |  |  | 25 | 50 | 75 | 100 | FDD |

MSD value without the harmonic trap filter is provided by companies in the following:

| Tdoc | Source | MSD [dB] | Number of UL RB in lower band | Note |
| --- | --- | --- | --- | --- |
| R4-164204 | Qualcomm | 22.3 | 100 | Separate antenna w/o HTF |

----- End of Text proposal -----

## References

RP-162146, “Revised WID: LTE Advanced inter-band CA Rel-14 for 3DL/1UL”, Huawei, Hisilicon, 3GPP RAN#74, Vienna, Austria, December, 2016