3GPP TSG-RAN WG4 Meeting #82 	R4-1702241

# Athens, Greece, 13-17 February 2017

Source:	SoftBank Corp.

Title:	TP for TR36.714-05-01: the support of CA_3A-28A-41C-42A BCS0

Agenda item:	7.5.2

Document for:	Approval

1.  Introduction

3A-28A-41C-42A BCS0 was approved in 5DL/1UL basket WID [1]. This paper is to evaluate IM/harmonics and relaxation required, including the relevant TP to complete this combination. This TP needs to agree constituent 4CA of 3A-28A-41A-42A[2] and 3A-28A-41C[3] in advance.

In this revision, (1)4Rx REFSENS is added to B3/B41, (2) REFSENS is adjusted by delta RIB and (3) B41 notes are added for frequency in REFSENS. Note that the modifications do not change the content of TP as TP does not explicitly include REFSENS exception (i.e. saying lower order exception can be reused.)

2. Basic Configurations

As written in the WID, the following configurations are assumed for B3+B28+B41+B42.

Table 1: 5DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3-28-41-42 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz |  |

Table 2: Supported E-UTRA bandwidths per CA configuration for 5DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_3A-28A-41C-42A | 3 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  | 28 |  |  | Yes | Yes |  |  |  |  |
|  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |
|  | 42 |  |  |  | Yes | Yes | Yes |  |  |

Note that B41 and B42 are assumed to be synchronized (i.e. without simultaneous Rx/Tx).

3.  Co-existence

The 3rd order IMD products caused in the BS by transmitting of Band 3, Band 28, Band 41 and Band 42 DL carriers are derived in the constituent 4DL to 2DL CAs, which have been already analyzed.

4.  ΔTIB and ΔRIB

This 5DL CA is simply extend B42 portion from [2] so there is no need to modify RF configuration. Therefore, ΔTIB and ΔRIB derived in 4CA [2] can be reused as:

Table 3: Proposed ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_3A-28A-41C-42A | 3 | 1 |
|  | 28 | 0.5 |
|  | 41 | 0.35/0.86 |
|  | 42 | 0.8 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 4: Proposed ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_3A-28A-41C-42A | 3 | 0.5 |
|  | 28 | 0.2 |
|  | 41 | 0.45/0.56 |
|  | 42 | 0.5 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

5.  REFSENS

Similar to section 4, this 5CA can reuse the same RF as in [2] then the same REFSENS and exceptions can be applied as below. TP mentions that 4DL CA result can be referred.

Table 5 (7.3.1A-0a): Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_3A-28A-41C-42A9,10,29 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
|  |  |  |  | -99.220 | -96.220 | -94.420 | -93.220 |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |
|  | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |
|  |  |  |  |  | -97.220 | -95.420 | -94.220 |  |
|  | 42 |  |  |  | -71.7 | -71.7 | -71.7 |  |
| CA_3A-28A-41C-42A11,29 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
|  |  |  |  | -99.220 | -96.220 | -94.420 | -93.220 |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |
|  | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |
|  |  |  |  |  | -97.220 | -95.420 | -94.220 |  |
|  | 42 |  |  |  | -94.7 | -93.2 | -92.5 |  |
| CA_3A-28A-41C-42A17,18, 29 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
|  |  |  |  | -99.220 | -96.220 | -94.420 | -93.220 |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |
|  | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |
|  |  |  |  |  | -97.220 | -95.420 | -94.220 |  |
|  | 42 |  |  |  | -85.4 | -85.1 | -84.9 |  |
| NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_1A-3A-42A, CA_3A-19A-42A, and CA_1A-3A-19A-42A and CA_3A-28A-41A-42A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 17: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 18: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 20: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 29: The B41 requirements are modified by -0.1dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. |  |  |  |  |  |  |  |  |

Table 6 (7.3.1A-0b): Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_3A-28A-41C-42A | 3 |  |  |  | 25 | 36 | 50 | FDD |
| CA_3A-28A-41C-42A | 28 |  |  |  | 10 | 15 | 20 | FDD |

Table 7(7.3.1A-0bE): Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_3A-28A-41C-42A | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |
|  | 41 |  |  |  | -94.517 | -92.717 | -91.517 | TDD |  |
|  |  |  |  |  | -97.211,17 | -95.411,17 | -94.211,17 |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |
|  | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |
|  |  |  |  | -99.211 | -96.211 | -94.411 | -93.211 |  |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |
|  | 41 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |
| NOTE 11: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 17: The B41 requirements are modified by -0.1dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. |  |  |  |  |  |  |  |  |  |

Table 8(7.3.1A-0bF): Uplink configuration for the uplink band (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_3A-28A-41C-42A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 28 |  |  | 25 | 251 |  |  | FDD |
|  | 41 |  |  |  | 50 | 75 | 100 | TDD |
|  | 42 |  |  |  | 50 | 75 | 100 | TDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |  |  |  |

6.  Summary

This paper investigates all the necessary elements to complete CA_3A-28A-41C-42A. The relevant TP is shown in Annex.

7.  Reference

[1]      RP-162005    Revised WID: LTE Advanced inter-band CA Rel-14 for 5DL/1UL, Ericsson (Rapporteur).

[2]      R4-1700710  TP for TR36.714-04-01: the support of CA_3A-28A-41A-42A BCS0, SoftBank

[3]      R4-1700709  TP for TR36.714-04-01: the support of CA_3A-28A-41C BCS0, SoftBank

Annex: TP for TR36.714-05-01 (v0.5.0)

[Unchanged portions skipped]

6.Y	CA_3A-28A-41C-42A_BCS0

6.Y.1	Channel bandwidths per operating band for CA

Table 6.Y.1-1: 5DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3-28-41-42 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

Table 6.Y.1-2: Supported E-UTRA bandwidths per CA configuration for 5DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_3A-28A-41C-42A | 3 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  | 28 |  |  | Yes | Yes |  |  |  |  |
|  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |
|  | 42 |  |  |  | Yes | Yes | Yes |  |  |

Note that B41 and B42 are assumed to be synchronized (i.e. without simultaneous Rx/Tx).

6.Y.2	Co-existence studies

The 2nd / 3rd harmonics and IMD products caused in the BS by transmitting CA_3A-28A-41C-42A follow those derived for the constituent 2DL, 3DL and 4DL CA fall-back modes.

6.Y.3	∆TIB and ∆RIB values

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c and ΔRIB,c is defined for applicable bands in Table 6.Y.3-1 and 6.Y.3-2 respectively.

Table 6.Y.3-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3A-28A-41C-42A | 3 | 1 |
|  | 28 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | 42 | 0.8 |
| Note1: The requirement is specified for the frequency range of 2545-2690MHz. Note2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

Table 6.Y.3-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3A-28A-41C-42A | 3 | 0.5 |
|  | 28 | 0.2 |
|  | 41 | 0.41/0.52 |
|  | 42 | 0.5 |
| Note1: The requirement is specified for the frequency range of 2545-2690MHz. Note2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

6.Y.4	REFSENS requirements

For the REFSENS of CA_3A-28A-41C-42A the same REFSENS requirements for the constituent 4DL CA fall-back modes of 3A-28A-41A-42A can be re-used.

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"