3GPP TSG-RAN WG4 Meeting #82	R4-1702301

Athens, Greece, 13 - 17 February, 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 4267 | rev | 1 | Current version: | 14.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of bands to support 4Rx APs to TS 36.101 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_4Rx_AP_DL_bands-Core |  |  |  |  |  | Date: |  |  | 2017-1-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | This CR is to introduce the following bands to support 4Rx APs. - Band 21, 25, 40. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following requirements are introduced. - Single carrier 4Rx requirements for Band 21, 25, 40. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 4Rx APs can’t be supported by the above bands. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# <Start of changes>

Table 7.3.1-1a: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -102.7 | -99.7 | -97.9 | -96.7 | FDD |
| 2 | -105.4 | -102.4 | -100.7 | -97.7 | -95.9 | -94.7 | FDD |
| 3 | -104.4 | -101.4 | -99.7 | -96.7 | -94.9 | -93.7 |  |
| … |  |  |  |  |  |  |  |
| 7 |  |  | -100.7 | -97.7 | -95.9 | -94.7 | FDD |
| … |  |  |  |  |  |  |  |
| 20 |  |  | -99.7 | -96.7 | -93.9 | -92.7 | FDD |
| 21 |  |  | -102.7 | -99.7 | -97.9 |  | FDD |
| 25 | -103.9 | -100.9 | -99.2 | -96.2 | -94.4 | -93.2 | FDD |
| … |  |  |  |  |  |  |  |
| 39 |  |  | -102.7 | -99.7 | -97.9 | -96.7 | TDD |
| 40 |  |  | -102.7 | -99.7 | -97.9 | -96.7 | TDD |
| … |  |  |  |  |  |  |  |
| 41 |  |  | -100.7 | -97.7 | -95.9 | -94.7 | TDD |
| 42 |  |  | -101.2 | -98.2 | -96.4 | -95.2 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port. |  |  |  |  |  |  |  |

# <End of changes>