3GPP TSG RAN WG4 Meeting #82	R4-1702420

Athens, Greece, 13 - 17 February 2017

3GPP TSG-RAN WG1 Meeting #88	R1-1703836

Athens, Greece, 13th – 17th February 2017

Title:	LS on NR-SS periodicity

Response to:

Release:	Rel-14

Work Item:	FS_NR_newRAT

Source:	RAN1

To:	RAN2, RAN4

Copy:

Contact Person:

Name:	Hiroki Harada

- E-mail Address:	hiroki.harada.sv@nttdocomo.com

Attachments:	None

# Overall description

RAN1 has been discussing on NR-SS periodicity, and RAN1 made following agreements regarding configurability of NR-SS periodicity for non-standalone NR cell detection.

Agreements: - For detecting non-standalone NR cell, NR should support adaptation and network indication of SS burst set periodicity and information to derive measurement timing/duration (e.g., time window for NR-SS detection) - For detecting non-standalone NR cell, network provides one SS burst set periodicity information per frequency carrier to UE and information to derive measurement timing/duration if possible - In case that one SS burst set periodicity and one information regarding timing/duration are indicated, UE assumes the periodicity and timing/duration for all cells on the same carrier - RAN1 recommends short measurement duration than configured periodicity e.g., 1, 5 or 10 ms - Note that L1/L3 filtering across multiple periods is still allowed - FFS more than one periodicity/timing/duration indication - NR should support set of SS burst set periodicity values for adaptation and network indication - Candidate periodicity values to be evaluated are [20, 40, 80 and 160 ms] - FFS other values with consideration for functionalities provided by NR-SS in connected mode - FFS whether to support NR-PBCH in non-standalone NR cell

RAN1 will continue discussion on candidate value(s) to be configured as SS burst set periodicity and measurement duration. RAN1 will also discuss on default value of NR-SS periodicity for standalone NR cell.

Actions

RAN1 respectfully asks RAN2 and RAN4 to take into account above information in their future work.

Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #88-bis	Apr 3-7, 2017			Spokane, USA

TSG RAN WG1 Meeting #89	May 15-19, 2017		     	Hangzhou, China