3GPP TSG-RAN WG4 Meeting #82b	R4- 1702851

Spokane, US, 3 - 7 April, 2017

Source:	China Telecom, Samsung

Title:	TP for TR36.714-03-01: the support of CA_1A-5A-41A_BCS0

Agenda item:	7.3.2

Document for:	Approval

1.  Introduction

B1+B5+B41 was approved in 3DL/1UL basket WID in the last RAN-P[1]. This paper is to evaluate IM/harmonics and relaxation required. This includes the relevant TP to complete this combination.

2. Basic Configurations

As written in the WID, the following configurations are assumed for B1+B5+B41.

Table 1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-5-41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 2: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-5A-41A | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 41 |  |  |  |  |  | Yes |  |  |

3.  Co-existence

The 3rd order IMD products caused in the BS by transmitting of Band 1, Band 5 and Band 41 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 3 below:

Table 3:Band 1, Band 5 and Band 41 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 869 | 894 | 2110 | 2170 | 2496 | 2690 |
| 3rd order IMD products | ¦f1-low + f2-low – f3-high¦ |  |  | ¦f1-high + f2-high – f3-low¦ |  |  |
| IMD frequency limits (MHz) | 289 |  |  | 568 |  |  |
| 3rd order IMD products | ¦f1-low + f3-low – f2-high¦ |  |  | ¦f1-high + f3-high – f2-low¦ |  |  |
| IMD frequency limits (MHz) | 1195 |  |  | 1474 |  |  |
| 3rd order IMD products | ¦f2-low + f3-low – f1-high¦ |  |  | ¦f2-high + f3-high – f1-low¦ |  |  |
| IMD frequency limits (MHz) | 3712 |  |  | 3991 |  |  |
| 3rd order IMD products | ¦f1-low + f2-low + f3-low¦ |  |  | ¦f1-high + f2-high + f3-high¦ |  |  |
| IMD frequency limits (MHz) | 5475 |  |  | 5754 |  |  |

It can be seen from Table 3 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (1 + 5 + 41) may fall into BS receive band of Bands 31, 11, 45, 21, 43 and 46. However, Bands 31, 11, 45, 21, 43 and 46 are not intended for use in the same geographical area as Bands 1, 5 and 41. Therefore, except the frequency bands impacted by harmonics and IMD products from the constituent 2 DL CA configurations, the 3rd order IMD products caused by BS supporting 3 DL CA_1A-5A-41A will not fall into the BS receive band of any other frequency band operating in the same geographical area.

For UE, Table 4 gives the 2nd and 3rd harmonic for CA with B1+ B5 + B41. The 3rd order harmonic products of Band 5 UL may fall into the passband of B41 DL for UE. But it was concluded in 2DL of 5A-41A BCS0 that whether the harmonic interference issue exist or not still depends on the actual spectrum allocation. In this 3CA, the same can be applied. Other than that, it is found that none of the products falls into the B1, B5 or B41 UE receiving bands.

Table 4: UE harmonic table for 3DL/1UL

| UE DL Carriers with B1, B5, B41 | [MHz] |
| --- | --- |
| 2*fx_low to 2*fx_high | 1648 to 1698 |
| 2*fy_low to 2*fy_high | 3840 to 3960 |
| 2*fz_low to 2*fz_high | 4992 to 5380 |
| 3*fx_low to 3*fx_high | 2472 to 2547 |
| 3*fy_low to 3*fy_high | 5760 to 5940 |
| 3*fz_low to 3*fz_high | 7488 to 8070 |

4.  ΔTIB and ΔRIB

The following ΔTIB and ΔRIB are proposed based on its constituent 2DL/1ULs:

Table 5: ΔTIBof constituent 2DL/1UL

| ΔTIB | 1+5 | 1+41 | 5+41 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.5 | - | 0.5 |
| B5 | 0.3 | - | 0.3 | 0.3 |
| B41 | - | 0.5 | 0.3 | 0.5 |

Table 6: ΔRIBof constituent 2DL/1UL

| ΔRIB | 1+5 | 1+41 | 5+41 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0 | 0 | - | 0 |
| B5 | 0 | - | 0 | 0 |
| B41 | - | 0 | 0 | 0 |

Table 7: Proposed ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-5A-41A | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 41 | 0.5 |

Table 8: Proposed ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-5A-41A | 1 | 0 |
|  | 5 | 0 |
|  | 41 | 0 |

5.  REFSENS

Based on the co-existence study in section 3, there is no harmonic issue in this combination taking actual spectrum allocation into account. And following CA_26A-41A agreement, network should avoid simultaneous Tx/Rx operation for CA_5A-41A when the Band 41 DL and 3rd harmonic of Band 5 UL overlaps each other.

For CA_1A-5A-41A, the reference sensitivity requirement of Band1, Band5 and Band41 are given in Table 9, exceptions are allowed when the uplink is active within a specified frequency range as noted in Table 9. For these exceptions, the UE shall meet the requirements specified in Table 9.

Table 9: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_1A-5A-41A | 5 |  |  | -98 | -95 |  |  | FDD | 1 |
|  | 41 |  |  |  |  |  | -92 | TDD |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 41 |
|  | 5 |  |  | -98 | -95 |  |  | FDD |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 5 |
|  | 411 |  |  |  |  |  | N/A | TDD |  |
| NOTE 1: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of B5 for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of B41. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |  |

6.  Summary

This paper investigates all the necessary elements to complete CA_1A-5A-41A. The relevant TP is shown in Annex.

7.  Reference

[1]      RP-170309 Revised WID: LTE Advanced inter-band CA Rel-14 for 3DL/1UL, Huawei, HiSilicon.

Annex: TP for TR36.714-03-01 (v0.7.0)

[Unchanged portions skipped]

6.Y 	CA_1A-5A-41A_BCS0

6.Y.1	Operating bands for CA

Table 6.Y.1-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-5-41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.Y.2	Channel bandwidths per operating band for CA

Table 6.Y.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-5A-41A | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: For the UE that signals support of a 3DL CA bandwidth combination set, the UE shall support all single carrier channel bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 and all the 2DL CA bandwidth combination sets in Table 5.6A.2-1 of TS 36.101 whose channel bandwidths are entirely covered by the signaled 3DL CA bandwidth combination set. |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 [2] when operating in single carrier mode.

6.Y.3 	Harmonics and IMD study

6.Y.3.1	For the BS

The 3rd order IMD products caused in the BS by transmitting of Band 1, Band 5 and Band 41 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.Y.3.1-1 below:

Table 6.Y.3.1-1:Band 1, Band 5 and Band 41 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 869 | 894 | 2110 | 2170 | 2496 | 2690 |
| 3rd order IMD products | ¦f1-low + f2-low – f3-high¦ |  |  | ¦f1-high + f2-high – f3-low¦ |  |  |
| IMD frequency limits (MHz) | 289 |  |  | 568 |  |  |
| 3rd order IMD products | ¦f1-low + f3-low – f2-high¦ |  |  | ¦f1-high + f3-high – f2-low¦ |  |  |
| IMD frequency limits (MHz) | 1195 |  |  | 1474 |  |  |
| 3rd order IMD products | ¦f2-low + f3-low – f1-high¦ |  |  | ¦f2-high + f3-high – f1-low¦ |  |  |
| IMD frequency limits (MHz) | 3712 |  |  | 3991 |  |  |
| 3rd order IMD products | ¦f1-low + f2-low + f3-low¦ |  |  | ¦f1-high + f2-high + f3-high¦ |  |  |
| IMD frequency limits (MHz) | 5475 |  |  | 5754 |  |  |

It can be seen from Table 3 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (1 + 5 + 41) may fall into BS receive band of Bands 31, 11, 45, 21, 43 and 46. However, Bands 31, 11, 45, 21, 43 and 46 are not intended for use in the same geographical area as Bands 1, 5 and 41. Therefore, except the frequency bands impacted by harmonics and IMD products from the constituent 2 DL CA configurations, the 3rd order IMD products caused by BS supporting 3 DL CA_1A-5A-41A will not fall into the BS receive band of any other frequency band operating in the same geographical area.

6.Y.3.2	For the UE

Table 6.Y.3.2-1 gives the 2nd and 3rd harmonic for CA with B1+ B5 + B41. The 3rd order harmonic products of Band 5 UL may fall into the passband of B41 DL for UE. But it was concluded in 2DL of 5A-41A BCS0 that whether the harmonic interference issue exist or not still depends on the actual spectrum allocation. In this 3CA, the same can be applied. Other than that, it is found that none of the products falls into the B1, B5 or B41 UE receiving bands.

Table 6.Y.3.2-1: UE harmonic table for 3DL/1UL

| UE DL Carriers with B1, B5, B41 | [MHz] |
| --- | --- |
| 2*fx_low to 2*fx_high | 1648 to 1698 |
| 2*fy_low to 2*fy_high | 3840 to 3960 |
| 2*fz_low to 2*fz_high | 4992 to 5380 |
| 3*fx_low to 3*fx_high | 2472 to 2547 |
| 3*fy_low to 3*fy_high | 5760 to 5940 |
| 3*fz_low to 3*fz_high | 7488 to 8070 |

6.Y.4	ΔTIB,c and ΔRIB,c values

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c is defined for applicable bands in Table 6.Y.4-1.

Table 6.Y.4-1: ΔTIB,cfor 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-5A-41A | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 41 | 0.5 |

For the UE which supports inter-band carrier aggregation configuration in Table 6.Y.4-2 with uplink in one E-UTRA band, the minimum requirement for reference sensitivity in Table 7.3.1-1 (of TS 36.101 [2]) shall be increased by the amount given in ΔRIB,c in Table 6.Y.4-2 for the applicable E-UTRA bands.

Table 6.Y.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-5A-41A | 1 | 0 |
|  | 5 | 0 |
|  | 41 | 0 |

6.Y.5	REFSENS requirements

Based on the co-existence study in 6.Y.3, there is no harmonic issue in this combination taking actual spectrum allocation into account. And following CA_26A-41A agreement, network should avoid simultaneous Tx/Rx operation for CA_5A-41A when the Band 41 DL and 3rd harmonic of Band 5 UL overlaps each other.

For CA_1A-5A-41A, the reference sensitivity requirement of Band1, Band5 and Band41 are given in Table 6.Y.5-1, exceptions are allowed when the uplink is active within a specified frequency range as noted in Table 6.Y.5-1. For these exceptions, the UE shall meet the requirements specified in Table 6.Y.5-1.

Table 6.Y.5-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_1A-5A-41A | 5 |  |  | -98 | -95 |  |  | FDD | 1 |
|  | 41 |  |  |  |  |  | -92 | TDD |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 41 |
|  | 5 |  |  | -98 | -95 |  |  | FDD |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 5 |
|  | 411 |  |  |  |  |  | N/A | TDD |  |
| NOTE 1: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of B5 for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of B41. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |  |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read:<FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"