3GPP TSG-RAN WG4 Meeting #82BIS 	R4-1702956

Spokane, U.S.A., 3 – 7 April, 2017

Agenda Item:	7.17.6

Source: 	Intel Corporation

Title: 	Simulation assumptions on SI acquisition enhancement for eNB-IoT UE

Document for:	Discussion

- Introduction

In RAN4#80bis and RAN4#81 Meetings, the problem of large SI acquisition delay, including MIB-NB, SIB1-NB and SIB2-NB, has been extensively discussed and consensus has been captured in the WFs [1], [2], [3], which include:

- RAN4 is to specify the parameters TSI-NB1-NC and TSI-NB1-EC to represent SI acquisition delay for normal coverage and extended coverage, respectively, in paging interruption and RRC re-establishment core requirements.

For example, for extended coverage: TSI-NB-EC = TMIB-NB-EC + TSIB1-NB-EC + TSIB2-NB-EC, where TMIB-NB-EC is the time duration for MIB-NB acquisition, TSIB1-NB-EC and TSIB2-NB-EC are the time duration for SIB1-NB and SIB2-NB acquisition, respectively. Moreover, different types of TTIs have been provided in [3] for simulations and are given below:

- MIB-NB TTI = 640ms

- SIB1-NB TTI = 2560ms

- SIB2-NB TTI = 160ms for normal, and 960ms for extended coverage.

It is also agreed that, as a low complexity “baseline” algorithm, a “keep trying” decoder is assumed. For example, the decoder simply “keeps trying” to decode the transmitted SIB1-NB subframes within single SIB1-NB TTI time period (i.e. 2560ms) until the decoder eventually gets lucky and decodes it correctly.

Previously, RAN4 has discussed the acquisition delays associated with the reception of system information for Rel-13 Category NB1 UEs.  Table 1 below provides a summary [5].

| Parameter | Cat 0 | Cat NB1 NC | Cat NB1 EC |
| --- | --- | --- | --- |
| T_MIB-NB | 50 | 640 | 2560 |
| T_SIB1-NB |  | 5120 | 29440 |
| T_SIB2-NB |  | 2560 | 9560 |
| T_SI for cell re-selection | 1280 | 8320 | 41560 |
| T_SI for RRC re-establishment | 1280 | 8320 | 41560 |
| NOTE 1: The parameters T_MIB-NB and T_SI are defined in TS 36.133 NOTE 2: The terms NC and EC are abbreviations for normal coverage and enhanced coverage, respectively NOTE 3: The values for SI acquisition delays for Category NB1 UEs have been derived using baseband only simulations and do not include RF impairment margin NOTE 4: The SIB2-NB acquisition delay depends on network configuration |  |  |  |

Table 1: Summary of acquisition delays of system information

Based on simulation results provided by different companies, it showed that the acquisition delay for the baseline receiver to successfully acquire both MIB-NB and SIB1-NB may be very large and even greater than or equal to the SIB1-NB 40.96s modification boundary.

Therefore, in RAN4#82 Meeting, RAN1 sent RAN4 an LS [4] regarding possible enchantments that can be used to improve SI acquisition performance. Possible enhancements include:

- Cross-subframe channel estimation

  - The coverage for MIB-NB and SIB1-NB may be improved (and acquisition latency reduced) by considering cross-subframe channel estimation. However, for MIB-NB and SIB1-NB acquisition, only subframes #0, #4, and #9 not containing NSSS for in-band mode, and only subframes #0, #1, #3, #4, and #9 not containing NSSS for guard-band and stand-alone modes can be assumed to have NRS presence.

RAN1 understands this is different to the current RAN4 receiver assumptions.

- Enhanced SIB1-NB accumulations

  - Enhanced SIB1-NB accumulations across multiple SIB1-NB transmission periods of 2560ms may be feasible without UE having to reacquire MIB-NB if the SIB1-NB scheduling information in the MIB can be assumed to be fairly static.

    - This aspect is expected to be confirmed by RAN2 WG.

- Additional NPBCH repetitions and advanced MIB-NB decoding techniques

  - One option to improve the coverage is via transmission of additional repetitions of NPBCH beyond those specified in Rel-13. Advanced MIB-NB decoding techniques may be able to combine across multiple 640ms windows.

Such enhancements could be considered for future releases.

In this contribution, we propose the simulation assumptions to evaluate the SI acquisition delay performance based on SI acquisition enhancement options provided by RAN1 LS [4].

- Simulation Assumptions for MIB-NB and SIB1-NB

In this section, we provide link level simulation assumptions for MIB-NB and SIB1-NB. The details of “cross-subframe channel estimation” and “enhanced SIB1-NB accumulations” are described in [6], [7], [8]. The “baseline” algorithm of “keep trying” can be used for comparison. And the evaluation target metric can be 1% BLER.

|  | Case 1 Normal coverage | Case 2 Enhanced Coverage |
| --- | --- | --- |
| Number of NRS ports {1,2} | 2 | 2 |
| Propagation channel | EPA5 | EPA5 |
| Modulation | QPSK | QPSK |
| Coding rate | 50/1600 | 50/1600 |
| Payload (without CRC) | 34 bits | 34 bits |
| Target SNRs | -6dB | -12dB |
| Cross-subframe CE window length | 8 or 16 ms | 8, 16 or 24 ms |

Table 1 Simulation assumption for MIB-NB acquisition

Table 1 lists the simulation assumptions for MIB-NB acquisition. The purpose is to investigate if cross-subframe CE is a valid enhancement option for MIB-NB acquisition, and how many MIB-NB TTIs are needed to achieve 1% BLER at target SNRs (impairment margin should be considered).

|  | Case 1 Normal coverage | Case 2 Enhanced Coverage |
| --- | --- | --- |
| Deployment mode | In-band | In-band |
| Number of NRS ports {1, 2} | 2 | 2 |
| Propagation channel | EPA5 | EPA5 |
| I_TBS {0/3/6/9} = {208/328/440/680bits} | 208bits | 208bits |
| Repetition number {4 ,8, 16} | 16 | 16 |
| Target SNRs | -6dB | -12dB |
| Cross-subframe CE window length | 8 or 16 ms | 8, 16 or 24 ms |
| SIB1-NB accumulation (1×SIB1-NB TTI = 2560ms) | 2×SIB1-NB TTIs | 2× or 4×SIB1-NB TTIs |

Table 2 Simulation assumption for SIB1-NB acquisition

Table 2 lists the simulation assumptions for SIB1-NB acquisition. The purpose is to investigate if “cross-subframe CE” and “enhanced SIB1-NB accumulation” are valid enhancement options for SIB1-NB acquisition, and how many SIB1-NB TTIs are needed to achieve 1% BLER at target SNRs (impairment margin should be considered).

Proposal: We propose simulation assumptions in Table 1 and Table 2 to evaluate the SI (incl. MIB-NB & SIB1-NB) acquisition delay performance based on SI acquisition enhancement options provided by RAN1 LS [4].

- Conclusion

Proposal: We propose simulation assumptions in Table 1 and Table 2 to evaluate the SI (incl. MIB-NB & SIB1-NB) acquisition delay performance based on SI acquisition enhancement options provided by RAN1 LS [4].

- References

[1] R4-168805, “WF on simulation assumptions for SI acquisition for NB-IoT”, Ericsson, Oct. 2016.

[2] R4-168806, “WF on SI acquisition delay for NB-IoT”, Huawei, HiSilicon, Intel, October 2016.

[3] R4-1610599, “Updated NB-IoT sim assumption for MIB and SIB acquisition”, Ericsson, Intel, November 2016.

[4] R1-1704068, “LS reply on NB-IoT SI acquisition delay”, RAN1, February 2017.

[5] R4-1700580, “On SI acquisition enhancements”, Intel, February 2017.

[6] R4-1702954, “SIB1-NB simulation results for eNB-IoT UE”, Intel, April 2017.

[7] R4-1702953, “MIB-NB simulation results for eNB-IoT UE”, Intel, April 2017.

[8] R4-1702955, “Discussion on SI acquisition enhancement for eNB-IoT UE”, Intel, April 2017.