3GPP TSG-RAN WG4 #82bis	R4-1703219

Spokane, Washington, 3 – 7 April, 2017

# Source:	Intel Corporation

# Title:	Potential coexistence consideration on LTE-NR band combination

# Agenda Item:	10.2.2

# Document for:	Proposal

# 1	Introduction

In RAN4#82 meeting in February, a way forward [1] has been agreed in which guidelines for general aspect, NR frequency ranges, as well as LTE-NR band combination. In this contribution, we identify potential coexistence band combinations based on the way forward.

# 2	Discussion

# 2.1 Proposed LTE-NR band combination

There was an extensive discussion on NR spectrum as well as LTE-NR band combination during RAN4#84 meeting and Table 1 is excerpted from the way forward [1].

Table 1. Proposed LTE-NR band combination

In this contribution, we studied on potential coexistence issues for each of LTE-NR band combinations.

# 2.2 LTE-NR (Sub-6GHz)

2.2.1 The followings are falling into the NR 3.3 GHz – 4.2 GHz range

- 2nd harmonic of B1 Tx

- 2nd harmonic of B3 Tx

- 2nd harmonic of B3 Rx

- 4th and 5th harmonics of B5 Tx

- 4th harmonic of B5 Rx

- 4th harmonic of B8 Tx

- 4th harmonic of B8 Rx

- 4th and 5th harmonics of B19 Tx

- 4th harmonic of B19 Rx

- 4th and 5th harmonics of B20 Tx

- 5th harmonic of B20 Rx

- 4th and 5th harmonics of B26 Tx

- 4th harmonic of B26 Rx

- 5th harmonic of B28 Tx

- 5th harmonic of B28 Rx

- 2nd harmonic of B39 Tx

- B7, B21, and B41 do not have any coexistence up to 10th harmonic.

2.2.2 The followings are falling into the NR 3.4 GHz – 4.99 GHz range

- 6th harmonic of B5 Tx

- 5th harmonic of B5 Rx

- 5th harmonic of B8 Tx

- 5th harmonic of B8 Rx

- 6th harmonic of B19 Tx

- 5th harmonic of B19 Rx

- 3rd harmonic of B21 Rx

- 6th harmonic of B26 Tx

- 5th harmonic of B26 Rx

- 6th and 7th harmonics of B28 Tx

- 6th harmonic of B28 Rx

- B1, B3, B39, and B41 do not have any coexistence up to 10th harmonic.

# 2.3 LTE-NR (mmWave)

For NR frequency 24.25 GHz – 29.5 GHz, 31.8 GHz – 33.4 GHz, and 37 – 40 GHz frequency ranges, any LTE bands listed in Table 1 does not have coexistence up to 10th harmonic

# 2.4 LTE-NR (re-farming LTE bands for NR)

2.4.1 The followings are falling into the NR Band 7

- 3rd harmonic of B8 Tx falls into B7 Rx

- Intermodulation of the 3rd harmonic of B3 Tx with B7 Tx falls into B7 Rx

- B20 (LTE) – B7 (NR) does not have any coexistence up to 10th harmonic

2.4.2 The followings are falling into the NR Band 28

- B20 overlaps with B28

- Intermodulation of the 5th harmonic of B28 Tx with B3 Tx falls into B3 Rx

- B7 (LTE) – B28 (NR) does not have any coexistence up to 10th harmonic

2.4.3 The followings are falling into the NR Band 41

- 3rd harmonic of B5 Tx falls into B41 Rx

- 3rd harmonic of B26 Tx falls into B41 Rx

- Intermodulation between 2nd harmonic of B26 Tx and B41 Tx falls into B26 Rx

- B41 (LTE) – B41 (NR) share the exactly same band at the same time

# 2.5 Summary

Table 2 summarize based on the results in the previous section.

For issue expected LTE-NR band combinations, they are marked as red color. To address the issues, it would need strong filtering or other special technique. For those in which higher harmonics (> 3rd) related, the combinations are marked as yellow color. Green color indicates no issue expected.

Table 2. Summary table of potential coexistence issues for the proposed LTE-NR band combinations

Based on the basic analysis, some combinations seems to be very challenging due to the 2nd harmonics or intermodulation distortions in Rx bands, i.e, B20 (LTE) – B28 (NR), B1 (LTE) – NR (3.3 GHz – 4.2GHz), B3 (LTE) – NR (3.3 GHz – 4.2 GHz), B39 (LTE) – NR (3.3 GHz – 4.2 GHz), and B41 (LTE) – B41 (NR).

Observation1: For the following LTE-NR combinations, 2nd harmonics or intermodulation would fall in Rx bands of interests and would need a strong filtering or special technique:

- B1 (LTE) – NR (3.3 GHz – 4.2GHz)

- B3 (LTE) – NR (3.3 GHz – 4.2 GHz)

- B39 (LTE) – NR (3.3 GHz – 4.2 GHz)

- B20 (LTE) – B28 (NR)

- B41 (LTE) – B41 (NR)

Proposal1: For those issue expected band combinations, we deprioritize them in the initial WI phase.

# 3	Conclusions

Observation1: For the following LTE-NR combinations, 2nd harmonics or intermodulation would fall in Rx bands of interests and would be strong filtering or special technique:

- B1 (LTE) – NR (3.3 GHz – 4.2GHz)

- B3 (LTE) – NR (3.3 GHz – 4.2 GHz)

- B39 (LTE) – NR (3.3 GHz – 4.2 GHz)

- B20 (LTE) – B28 (NR)

- B41 (LTE) – B41 (NR)

Proposal1: For those issue expected band combinations, we deprioritize them in the initial WI phase.

# 4	Reference

- R4-1702504, “WF on NR spectrum”, NTT Docomo, RAN4#82