3GPP TSG RAN WG4 Meeting #82bis	R4-1703402

Spokane, WA, US, April 3 – 7, 2017

Agenda Item:	7.18.3

Source: 	Ericsson

Title:	Improved CE level selection for RACH in feMTC

Document for:	Discussion

# Introduction

Selection of PRACH repetition level under CE has been identified as one area where improvements under the feMTC WI are desirable.

In this contribution we are investigating the diversity gains that come from carrying out RSRP measurements over 2 TX ports, when available.

Based on the results we are proposing that for PRACH repetition level selection, if the cell is supporting CE mode B, the UE shall base the PRACH repetition level selection on measurements on 2 TX ports.

# Background

## Diversity in Propagation paths

In legacy the UE is mandated only to carry out measurements on TX port 0, with measurements on TX port 1, when present, being optional. One justification is that the legacy UE is using 2 RX branches, and hence get a diversity gain as it sees two different propagation channels from TX port 0 to RX branches 0 and 1.

With the introduction of device categories with only 1 RX branch, the diversity gain is lost when conducting measurements only on TX port 0, i.e., the UE only sees one propagation path, and becomes sensitive to fading. One obvious improvement would be to regain the diversity by having the UE conducting measurements on both TX ports, when available.

## RSRP and RSRQ Measurement approach

The legacy requirements on measurement accuracy in static (AWGN) conditions can be met by coherently averaging 8 CRSs, calculate the power, and then average such power estimates non-coherently over 2x5 subframes. The coherent averaging reduces the bias and the non-coherent averaging the variance of the RSRP estimate. The increase in SNR achieved by the coherent averaging is 3dB for every doubling of the number of coherently averaged CRSs. Hence when deriving the legacy requirements an SNR increase of 9dB was achieved, allowing measurement accuracy requirements to be fulfilled at least down to Ês/Iot -6dB.

To be able to accurately support enhanced coverage down to Ês/Iot -6dB, the coherent averaging needs to include more samples to provide sufficient number of REs containing CRS compared to that of an individual CRS. Using all CRSs in two adjacent subframes provides 96 CRSs. Assuming that CRSs in more than two adjacent subframes can be used for coherent averaging is not possible in the general case, firstly due to the frame structures (see above) and secondly due to the potentially larger frequency offset compared to legacy caused by impaired AFC operation at very low SINR. The two sizes of coherent averages are shown in Figure 1.

Figure 1: Illustration of (left) coherent averaging based on 8 CRSs and (right) on 96 CRSs. The former can increase the SINR by 9dB and the latter by 19.8 dB.

Simulations have been carried out using coherent averaging over 96 CRSs (averaging over 2 subframes in time and 6 RBs in frequency). These are then non-coherently averaged over 10 coherent averages. With a 40ms measurement cycle and two subframes captured each measurement occasion, the measurement period is 400ms. In order to investigate different cases of stationary conditions, simulations were carried out for AWGN, ETU 1Hz, EPA 1Hz, ETU 30Hz and EPA 5Hz. Existing requirements are however only applicable for AWGN.  The simulation parameters are shown in Table .

Table 1: Simulation parameters for FeMTC RSRP and RSRQ measurement studies

| Parameters | Value | Comments |
| --- | --- | --- |
| Measurement bandwidth | 6 RBs |  |
| L1 measurement period | 800 ms | Focusing on lower SINR range mainly |
| Measurement sampling rate | 40ms |  |
| Consecutive subframes used | 2 |  |
| L3 filtering | Disabled |  |
| Transmit antennas | 2 | Two TX ports |
| Receive antennas | 1 | Single Rx branches |
| Mobility | Stationary UEs, mobile UEs |  |
| Propagation conditions | AWGN, ETU 1Hz, EPA 1Hz, ETU 30Hz, EPA 5Hz |  |
| Channel estimation techniques | Current method (Rel-8) for RS averaging |  |
| Measured TX ports | 1, 2 | Analysis of diversity gain |
| CP length | Normal |  |
| Carrier frequency | 2 GHz |  |
| Ec/Iot | -15 dB, -12 dB, -6 dB | For CEModeA and CEModeB |
| Residual CFO | 50Hz |  |

# Discussions

## Simulation results, Diversity gain

The simulation results for the propagation scenarios laid out above are shown below in Figure 2 through Figure 6.

Figure 2: AWGN propagation conditions, 1 TX (left) and 2 TX (right)

Figure 3: ETU 1Hz propagation condition, 1 TX (left) and 2 TX (right)

Figure 4: EPA 1Hz propagation condition, 1 TX (left) and 2 TX (right)

Figure 5: ETU 30Hz propagation condition, 1 TX (left) and 2 TX (right)

Figure 6: EPA 5Hz propagation condition, 1 TX (left) and 2 TX (right)

Observations:

- Particularly for slowly fading channels there is a significant reduction (1-2dB) each of the 5th and 95th percentiles around the median when 2 TX ports are used instead of a single one, i.e., the spread is reduced.

- The bias remains essentially unaffected, which can be expected since coherent averaging over the TX branches has been carried out for the 2 TX scenario.

Based on the observations it is proposed that for PRACH repetition level selection, UEs of categories with single RX branch shall be mandated to conduct measurements on two TX ports when present, as this is a simple way to reinstate the similar kind of diversity as experienced by legacy UEs. The most suitable combination scheme for measurements from 2 TX ports is FFS.

The UE shall not detect the presence of 2 TX ports, but shall assume that two ports are available when the cell to which random access is to be conducted supports CE mode B.

Proposal:

- For the purpose of PRACH repetition level selection, UE categories with only one RX branch shall assume the existence and conduct measurements on two TX ports when the cell to which a random access is to be carried out is supporting CE mode B. Combination scheme for measurements on two TX ports is FFS.

Note:

It shall be noted that carrying out parallel measurements assuming the two cases:

- Only TX port 1 available

- Both TX ports 1 and 2 available

is adding a negligible increase in complexity compared to assuming only one TX port being present. The reason is that reference signals from both TX ports are available in the same OFDM symbols. Hence there is no need to carry out additional FFTs compared to measurements on single TX port. Hence it is only de-rotation of QPSK reference signals and one additional filtering operation that is needed in order to support both options in parallel. Once the UE gets to know from system information whether the target cell is supporting CE mode B or not, it can decide whether to base the PRACH repetition level selections on RSRP measurements on a single or on two TX ports.

# Summary and Conclusions

In this contribution we have shown that there are diversity gains from conducting measurements over 2 TX ports, when available. These diversity gains may be used for improving the PRACH repetition level selection by reducing the spread of the RSRP estimates.

We made the following observations and have the following proposal.

Observations:

- Particularly for slowly fading channels there is a significant reduction (1-2dB) each of the 5th and 95th percentiles around the median when 2 TX ports are used instead of a single one, i.e., the spread is reduced.

- The bias remains essentially unaffected, which can be expected since coherent averaging over the TX branches has been carried out for the 2 TX scenario.

Proposal:

- For the purpose of PRACH repetition level selection, UE categories with only one RX branch shall assume the existence and conduct measurements on two TX ports when the cell to which a random access is to be carried out is supporting CE mode B. Combination scheme for measurements on two TX ports is FFS.

As pointed out, at a negligible complexity increase the UE can postpone the decision on whether to base the PRACH repetition level selection on one or two TX ports until it has received system information. Hence, no blind detection of whether one or two ports are present shall be conducted.

Interested companies are invited to provide comments as well as to investigate the best way of combining information from two TX ports. The assumption in the present simulations was that one simply average the RSRP measured over the two ports.

# References

- 36.133