3GPP TSG-RAN WG4 Meeting #83	R4-1704510

Hangzhou, China, 15 - 19May, 2017

Source:	China Telecom, Samsung

Title:	TP for TR36.714-04-01: the support of CA_4DL_1A-3A-5A-41A_1UL_BCS0

Agenda item:	7.4.2

Document for:	Approval

1.  Introduction

CA_4DL_1A-3A-5A-41A_1UL_BCS0 was approved in4DL/1UL basket WID in the last RAN-P[1]. This paper is to evaluate IM/harmonics and relaxation required. This includes the relevant TP to complete this combination.

2. Basic Configurations

As written in the WID, the following configurations are assumed for B1+3+B5+B41.

Table 1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-3-5-41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 2: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-3A-5A-41A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 41 |  |  |  |  |  | Yes |  |  |
| Note 1: UL carrier is only supported on Band 1, Band 3 or Band 5 not Band 41 because the fall back mode 2DL/1UL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. |  |  |  |  |  |  |  |  |  |

3.  Co-existence

The 2ndand 3rd order harmonics and IM products generated in the BS by transmitting of Band 1, Band 3, Band 5 and Band 41 DL carriers (i.e. 4DL CA with one component carrier in each band) follow from those derived for the constituent 2DL and 3DL CA fall-back modes.

Note that there is not any additional 2nd and 3rd order harmonics and IM products generatedin a 4DL CA configuration when compared to the constituent 2DL and 3DL CA configurations, provided that the 4DL CA configuration does not use a larger carrier channel bandwidth or aggregated channel bandwidth in any of the CA bands when compared to the constituent 2DL and 3DL CA configurations.

For UE, Table 3 gives the 2nd and 3rd harmonic for CA with B1+3+ B5 + B41. The 3rd order harmonic products of Band 5 UL may fall into the passband of B41 DL for UE.But it was concluded in 2DL of 5A-41A BCS0 that whether the harmonic interference issue exist or not still depends on the actual spectrum allocation. In this 4CA, the same can be applied. Other than that, it is found that none of the products falls into the B1, B3, B5 or B41 UE receiving bands.

Table 3: UE harmonic table for 4DL/1UL

| UE ULCarriers with B1, B3, B5 | [MHz] |
| --- | --- |
| 2*fx_low to 2*fx_high | 1648 to 1698 |
| 2*fy_low to 2*fy_high | 3420 to 3570 |
| 2*fz_low to 2*fz_high | 3840 to 3960 |
| 3*fx_low to 3*fx_high | 2472 to 2547 |
| 3*fy_low to 3*fy_high | 5130 to 5355 |
| 3*fz_low to 3*fz_high | 5760 to 5940 |

4.  ΔTIB and ΔRIB

The followingΔTIB and ΔRIBare proposed based on its constituent 3DL/1ULs:

Table 4: ΔTIBof constituent 3DL/1UL

| ΔTIB | 1+3+5 | 1+3+41 | 1+5+41 | 3+5+41 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | [0.3] | 0.5 | 0.5 | - | 0.5 |
| B3 | 0.3 | 0.5 | - | 0.5 | 0.5 |
| B5 | 0.3 | - | 0.3 | 0.31 | 0.31 |
| B41 | - | 0.32 | 0.5 | 0.32 | 0.5 |
|  |  | 0.83 |  | 0.83 | 0.83 |
| NOTE 1: The values in the table reflect what can be achieved with the present state of the art technology. They shall be reconsidered when the state of the art technology progresses. NOTE 2: The requirement is applied for UE on the frequency range of 2545-2690MHz. NOTE 3: The requirement is applied for UE on the frequency range of 2496-2545MHz. |  |  |  |  |  |

Table 5: ΔRIBof constituent 3DL/1UL

| ΔRIB | 1+3+5 | 1+3+41 | 1+5+41 | 3+5+41 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | 0 | 0 | 0 | - | 0 |
| B3 | 0 | 0 | - | 0 | 0 |
| B5 | 0 | - | 0 | 0 | 0 |
| B41 | - | 01 | 0 | 01 | 01 |
|  |  | 0.52 |  | 0.52 | 0.52 |
| NOTE 1: The requirement is applied for UE on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE on the frequency range of 2496-2545MHz. |  |  |  |  |  |

Table 6: Proposed ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-3A-5A-41A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 5 | 0.31 |
|  | 41 | 0.52 |
|  |  | 0.83 |
| NOTE 1: The values in the table reflect what can be achieved with the present state of the art technology. They shall be reconsidered when the state of the art technology progresses. NOTE 2: The requirement is applied for UE on the frequency range of 2545-2690MHz. NOTE 3: The requirement is applied for UE on the frequency range of 2496-2545MHz. |  |  |

Table 7: Proposed ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-3A-5A-41A | 1 | 0 |
|  | 3 | 0 |
|  | 5 | 0 |
|  | 41 | 01 |
|  |  | 0.52 |
| NOTE 1: The requirement is applied for UE on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE on the frequency range of 2496-2545MHz. |  |  |

5.  REFSENS

Based on the co-existence study in section 3, there is no harmonic issue in this combination taking actual spectrum allocation into account. And following CA_26A-41A agreement, network should avoid simultaneous Tx/Rx operation for CA_5A-41A when the Band 41 DL and 3rd harmonic of Band 5 UL overlaps each other.

For CA_1A-3A-5A-41A, the reference sensitivity requirement of Band1, Band3, Band5 and Band41are given in Table8, Table 9 and Table 11, exceptions are allowed when the uplink is active within a specified frequency range as noted in Table 8, Table 9and Table 11. And the uplink configuration for reference sensitivityofBand1, Band3, Band5 and Band41are given in Table10 and Table12.For these exceptions, the UE shall meet the requirements specified in Table 8,Table 9and Table 11.

Table 8: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA(exceptionsdue to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-5A-41A | 1 |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 5 |  |  | N/A | N/A |  |  | FDD |
|  | 41 |  |  |  |  |  | N/A | TDD |
| Note: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of band5 for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of band41. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |

Table 9: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_1A-3A-5A-41A1,4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |
|  | 32 |  |  | -94 | -91.5 | -90 | -89 |  |  |
|  | 33 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |
|  | 41 |  |  |  |  |  | [88.1] | TDD |  |
| NOTE 1: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690MHz. NOTE 2: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 3: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 4: The B41 requirements also apply to the supported CA_1A-41A |  |  |  |  |  |  |  |  |  |

Table 10:Uplink configuration for reference sensitivity(exceptions for four bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-3A-5A-41A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60MHz. |  |  |  |  |  |  |  |  |

Table 11: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_1A-3A-5A-41A1 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |
|  | 41 |  |  |  |  |  | [-88.1] | TDD |  |
| NOTE 1: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690MHz. |  |  |  |  |  |  |  |  |  |

Table 12:Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-3A-5A-41A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. |  |  |  |  |  |  |  |  |

6.  Summary

This paper investigates all the necessary elements to complete CA_1A-3A-5A-41A. The relevant TP is shown in Annex.

7.  Reference

[1]      RP-170189 Revised WID: LTE Advanced inter-band CA Rel-14 for 4DL/1UL, Ericsson.

Annex: TP for TR36.714-04-01 (v0.7.0)

[Unchanged portions skipped]

6.Y	CA_1A-3A-5A-41A_BCS0

6.Y.1	Operating bands for CA

Table 6.Y.1-1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1_3-5-41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.Y.2	Channel bandwidths per operating band for CA

Table 6.Y.2-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-3A-5A-41A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 41 |  |  |  |  |  | Yes |  |  |
| Note 1: UL carrier is only supported on Band 1 or Band 5 not Band 41 because the fall back mode 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. |  |  |  |  |  |  |  |  |  |

NOTE: For the UE that signals support of a 4DL CA bandwidth combination set, the UE shall support all single carrier channel bandwidths for the constituent bands as defined in table 5.6A.1-1 of TS 36.101 and all the 3DL CA bandwidth combination sets in Table 5.6A.1-2a of TS 36.101 whose channel bandwidths are entirely covered by the signaled 4DL CA bandwidth combination set.

6.Y.3	Harmonics and IMD study

6.Y.3.1	For the BS

The 2ndand 3rd order harmonics and IM products generated in the BS by transmitting of Band 1, Band 3, Band 5 and Band 41 DL carriers (i.e. 4DL CA with one component carrier in each band) follow from those derived for the constituent 2DL and 3DL CA fall-back modes.

6.Y.3.2	For the UE

Table 6.Y.3.2-1 gives the 2nd and 3rd harmonic for CA with B1+B3+ B5 + B41. The 3rd order harmonic products of Band 5 UL may fall into the passband of B41 DL for UE.But it was concluded in 2DL of 5A-41A BCS0 that whether the harmonic interference issue exist or not still depends on the actual spectrum allocation. In this 4CA, the same can be applied. Other than that, it is found that none of the products falls into the B1, B3, B5 or B41 UE receiving bands.

Table 6.Y.3.2-1: UE harmonic table for 4DL/1UL

| UE ULCarriers with B1, B3, B5 | [MHz] |
| --- | --- |
| 2*fx_low to 2*fx_high | 1648 to 1698 |
| 2*fy_low to 2*fy_high | 3420 to 3570 |
| 2*fz_low to 2*fz_high | 3840 to 3960 |
| 3*fx_low to 3*fx_high | 2472 to 2547 |
| 3*fy_low to 3*fy_high | 5130 to 5355 |
| 3*fz_low to 3*fz_high | 5760 to 5940 |

6.Y.4	ΔTIB,c and ΔRIB,c values

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band, theΔTIB,c is defined for applicable bands in Table 6.Y.4-1.

Table 6.Y.4-1: ΔTIB,cfor4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-3A-5A-41A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 5 | 0.31 |
|  | 41 | 0.52 |
|  |  | 0.83 |
| NOTE 1: The values in the table reflect what can be achieved with the present state of the art technology. They shall be reconsidered when the state of the art technology progresses. NOTE 2: The requirement is applied for UE on the frequency range of 2545-2690MHz. NOTE 3: The requirement is applied for UE on the frequency range of 2496-2545MHz. |  |  |

For the UE which supports inter-band carrier aggregation configuration in Table 6.Y.4-2 with uplink in one E-UTRA band, the minimum requirement for reference sensitivity in Table 7.3.1-1 (of TS 36.101 [2]) shall be increased by the amount given in ΔRIB,c in Table 6.Y.4-2 for the applicable E-UTRA bands.

Table 6.Y.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-3A-5A-41A | 1 | 0 |
|  | 3 | 0 |
|  | 5 | 0 |
|  | 41 | 01 |
|  |  | 0.52 |
| NOTE 1: The requirement is applied for UE on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE on the frequency range of 2496-2545MHz. |  |  |

6.Y.5	REFSENS requirements

Based on the co-existence study in6.Y.3, there is no harmonic issue in this combination taking actual spectrum allocation into account. And following CA_26A-41A agreement, network should avoid simultaneous Tx/Rx operation for CA_5A-41A when the Band 41 DL and 3rd harmonic of Band 5 UL overlaps each other.

For CA_1A-3A-5A-41A, the reference sensitivity requirement of Band1, Band3, Band5 and Band41are given in Table6.Y.5-1,Table6.Y.5-2 and Table 6.Y.5-4, exceptions are allowed when the uplink is active within a specified frequency range as noted in Table6.Y.5-1,Table6.Y.5-2and Table 6.Y.5-4. The relateduplink configuration for reference sensitivityof Band1, Band3, Band5 and Band41are given in Table6.Y.5-3 and Table6.Y.5-5. For these exceptions, the UE shall meet the requirements specified in Table 6.Y.5-1, 6.Y.5-2and Table 6.Y.5-4.

Table 6.Y.5-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptionsdue to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-5A-41A1 | 1 |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 5 |  |  | N/A | N/A |  |  | FDD |
|  | 41 |  |  |  |  |  | N/A | TDD |
| Note1: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of band5 for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of band41. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |

Table 6.Y.5-2: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_1A-3A-5A-41A1,4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |
|  | 32 |  |  | -94 | -91.5 | -90 | -89 |  |  |
|  | 33 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |
|  | 41 |  |  |  |  |  | [88.1] | TDD |  |
| NOTE 1: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690MHz. NOTE 2: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 3: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 4: The B41 requirements also apply to the supported CA_1A-41A |  |  |  |  |  |  |  |  |  |

Table 6.Y.5-3:Uplink configuration for reference sensitivity(exceptions for four bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-3A-5A-41A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60MHz. |  |  |  |  |  |  |  |  |

Table 6.Y.5-4: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_1A-3A-5A-41A1 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |
|  | 41 |  |  |  |  |  | [-88.1] | TDD |  |
| NOTE 1: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690MHz. |  |  |  |  |  |  |  |  |  |

Table 6.Y.5-5:Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-3A-5A-41A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. |  |  |  |  |  |  |  |  |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read:<FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"