3GPP TSG-RAN4 Meeting #83 	R4-1704624

# Hangzhou, China, 15 – 19 May, 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 4388 | rev | - | Current version: | 13.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for demodulation of NB-IoT correction (Rel.13) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  |  | Date: |  | 2017-5-1 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1: Typo still exists; 2: Term insistence still exists. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1: Typo is corrected; 2: Updated the guard-band mode term in table titles of Table 8.12.1.1.2-1 and Table 8.12.1.1.2-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error will still exist in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 35.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

## 8.12	Demodulation of Narrowband IoT

### 8.12.1	NPDSCH

#### 8.12.1.1	Half-duplex FDD

Table 8.12.1.1-1: Common Test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes per component carrier | Processes | 1 |
| Maximum number of HARQ transmission |  | 4 |
| Cyclic Prefix |  | Normal |
| eutraControlRegionSize |  | 3 for In-band and N/A for Standalone/Guard-band unless otherwise stated |
| downlinkBitmap and dl-Gap |  | Not configured |
| dl-GapNonAnchor and downlinkBitmapNonAnchor |  | Not configured |
| Unused REs or RB (Note 1) |  | OCNG |
| OCNG pattern |  | NB.OP.1 |
| Note 1: For in-band mode, the REs for transmission of LTE signals including PDCCH, CRS should be filled by OCNG. |  |  |

Table 8.12.1.1-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N1 |
| scheduling delay field () |  | 1 |
| (ack-NACK-NumRepetitions) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDCCH |  | R.NB.3 for one NRS antenna port; R.NB.4 for two NRS antenna ports |
| nPDCCH-startSF-USS |  | 1.5 |
| npdcch-Offset-USS-r13 |  | 0 |

##### 8.12.1.1.1	Minimum Requirements for In-band

The requirements are specified in Table 8.12.1.1.1-2, with the addition of the parameters in Table 8.12.1.1.1 -1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the performance.

Table 8.12.1.1.1-1: Test Parameters for NPDSCH under In-band

| Parameter |  | Unit |  | Test 1, 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation of LTE signal |  |  | dB | -3 |
|  |  |  | dB | -3(Note 1) |
|  |  |  | dB | 0 |
| at antenna port |  |  | dBm/15kHz | -93 (Note 2) |
|  |  |  | dBm/15kHz | -99 (Note 3) |
| LTE CRS port number (eutra-NumCRS-Ports) |  |  |  | 4 |
| NPDCCH repetition number |  |  | subframe | 8 for Test 1; 16 for Test 2; 128 for Test 3. |
| (npdcch-NumRepetitions-r13) |  |  | subframe | 8 for Test 1;16 for Test 2;128 for Test 3. |
| Note 1: . Note 2: This noise is applied in NPDSCH subframes; Note 3: This noise is applied in NPDCCH subframes. |  |  |  |  |

Table 8.12.1.1.1-2: Minimum performance under In-band with 2 NRS ports

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 200kHz | Anchor | R.NB.5 FDD | 1 | EPA5 | 2x1 Low | 70% | 6.9 |
| 2 | 200kHz | Anchor | R.NB.5 FDD | 32 | EPA5 | 2x1 Low | 70% | -4.8 |
| 3 | 200kHz | Non-anchor | R.NB.5-1 FDD | 256 | ETU1 | 2x1 Low | 70% | -9.8 |

##### 8.12.1.1.2	Minimum Requirements for Standalone/Guard-band

The requirements are specified in Table 8.12.1.1.2-2, with the addition of the parameters in Table 8.12.1.1.2 -1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the performance.

Table 8.12.1.1.2-1: Test Parameters for NPDSCH under Standalone/Guard-band

| Parameter |  | Unit | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1; 256 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 64 for Test 1; 512 for Test 2. |
| Note 1: This noise is applied in NPDSCH subframes;; Note 2: This noise is applied in NPDCCH subframes. |  |  |  |

Table 8.12.1.1.2-2: Minimum performance for NPDSCH under Standalone/Guard-band with 1 NRS port

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 200kHz | Anchor | R.NB.6 FDD | 32 | EPA5 | 1 | 70% | -3.4 |
| 2 | 200kHz | Non-anchor | R.NB.6-1 FDD | 256 | ETU1 | 1 | 70% | -10.2 |

End of Change 1