3GPP TSG-RAN WG4 Meeting #83	R4-1704847

# Hangzhou, China, 15th-19th May, 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | 0778 | rev | - | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add NB-IoT support to Band 21 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT_R14_bands |  |  |  |  |  |  | Date: |  | 2016-05-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR was missing to cover adding NB-IoT support to Band 21 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove “Note 3” for Band 21 in table 4.4-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT support won’t be correctly introduced to Band 21. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR… CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

## 4.4	Operating bands and band categories

MSR requirements are applicable for band definitions and band numbering as defined in the specifications TS 45.005 [6], TS25.104 [3], TS 25.105 [4] and TS 36.104 [5]. For the purpose of defining the BS requirements, the operating bands are divided into three band categories as follows:

-	Band Category 1 (BC1): Bands for E-UTRA FDD and UTRA FDD operation. Bands in this category are also used for NB-IoT operation (all modes).

-	Band Category 2 (BC2): Bands for E-UTRA FDD, UTRA FDD and GSM/EDGE operation. Bands in this category are also used for NB-IoT operation (all modes).

-	Band Category 3 (BC3): Bands for E-UTRA TDD and UTRA TDD operation

NOTE:	For UTRA TDD, requirements in the present document cover the 1.28 Mcps UTRA TDD option.

The paired and unpaired bands for the three Band Categories are shown in Table 4.4-1 and 4.4-2, together with the corresponding E-UTRA, UTRA and GSM/EDGE band designations. In the present specification, the operating band of an MSR Base Stations is designated using the E-UTRA band number according to the tables.

# Table 4.4-1: Paired bands in E-UTRA, UTRA and GSM/EDGE

| MSR and E-UTRA Band number | UTRA Band number | GSM/EDGE Band designation | Uplink (UL) BS receive UE transmit |  |  |  |  | Downlink (DL) BS transmit UE receive |  |  |  |  |  | Band category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | I | - | 1920 MHz | – |  | 1980 MHz |  | 2110 MHz |  | – |  | 2170 MHz |  | 1 |
| 2 | II | PCS 1900 | 1850 MHz | – |  | 1910 MHz |  | 1930 MHz |  | – |  | 1990 MHz |  | 2 |
| 3 | III | DCS 1800 | 1710 MHz | – |  | 1785 MHz |  | 1805 MHz |  | – |  | 1880 MHz |  | 2 |
| 4 | IV | - | 1710 MHz | – |  | 1755 MHz |  | 2110 MHz |  | – |  | 2155 MHz |  | 1 (NOTE 3) |
| 5 | V | GSM 850 | 824 MHz | – |  | 849 MHz |  | 869 MHz |  | – |  | 894 MHz |  | 2 |
| 6 (NOTE 1) | VI | - | 830 MHz | – |  | 840 MHz |  | 875 MHz |  | – |  | 885 MHz |  | 1 (NOTE 1) |
| 7 | VII | - | 2500 MHz | – |  | 2570 MHz |  | 2620 MHz |  | – |  | 2690 MHz |  | 1 (NOTE 3) |
| 8 | VIII | E-GSM | 880 MHz | – |  | 915 MHz |  | 925 MHz |  | – |  | 960 MHz |  | 2 |
| 9 | IX | - | 1749.9 MHz | – |  | 1784.9 MHz |  | 1844.9 MHz |  | – |  | 1879.9 MHz |  | 1 (NOTE 3) |
| 10 | X | - | 1710 MHz | – |  | 1770 MHz |  | 2110 MHz |  | – |  | 2170 MHz |  | 1 (NOTE 3) |
| 11 | XI | - | 1427.9 MHz | – |  | 1447.9 MHz |  | 1475.9 MHz |  | – |  | 1495.9 MHz |  | 1 (NOTE 3) |
| 12 | XII | - | 699 MHz | – |  | 716 MHz |  | 729 MHz |  | – |  | 746 MHz |  | 1 |
| 13 | XIII | - | 777 MHz | – |  | 787 MHz |  | 746 MHz |  | – |  | 756 MHz |  | 1 |
| 14 | XIV | - | 788 MHz | – |  | 798 MHz |  | 758 MHz |  | – |  | 768 MHz |  | 1 (NOTE 3) |
| 15 | XV | - | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  |
| 16 | XVI | - | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  |
| 17 | - | - | 704 MHz | – |  | 716 MHz |  | 734 MHz |  | – |  | 746 MHz |  | 1 (NOTE 4) |
| 18 | - | - | 815 MHz | – |  | 830 MHz |  | 860 MHz |  | – |  | 875 MHz |  | 1 (NOTE 4) |
| 19 | XIX | - | 830 MHz | – |  | 845 MHz |  | 875 MHz |  | – |  | 890 MHz |  | 1 |
| 20 | XX | - | 832 MHz | – |  | 862 MHz |  | 791 MHz |  | – |  | 821 MHz |  | 1 |
| 21 | XXI | - | 1447.9 MHz | – |  | 1462.9 MHz |  | 1495.9 MHz |  | – |  | 1510.9 MHz |  | 1 |
| 22 | XXII | - | 3410 MHz | – |  | 3490 MHz |  | 3510 MHz |  | – |  | 3590 MHz |  | 1 (NOTE 3) |
| 238 | - | - | 2000 MHz | – |  | 2020 MHz |  | 2180 MHz |  | – |  | 2200 MHz |  | 1 (NOTE 2) |
| 24 | - | - | 1626.5 MHz | – |  | 1660.5 MHz |  | 1525 MHz |  | – |  | 1559 MHz |  | 1 (NOTE 2) |
| 25 | XXV | - | 1850 MHz | – |  | 1915 MHz |  | 1930 MHz |  | – |  | 1995 MHz |  | 1 (NOTE 3) |
| 26 | XXVI | - | 814 MHz | – |  | 849 MHz |  | 859 MHz |  | – |  | 894 MHz |  | 1 |
| 27 | - | - | 807 MHz | – |  | 824 MHz |  | 852 MHz |  | – |  | 869 MHz |  | 1 (NOTE 2) |
| 28 | - | - | 703 MHz | – |  | 748 MHz |  | 758 MHz |  | – |  | 803 MHz |  | 1 (NOTE 4) |
| 29 | - | - | N/A |  |  |  |  | 717 MHz |  | – |  | 728 MHz |  | 1 (NOTE 2, NOTE 5) |
| 30 | - | - | 2305 MHz | – |  | 2315 MHz |  | 2350 MHz |  | – |  | 2360 MHz |  | 1 (NOTE 2) |
| 31 | - | - | 452.5 MHz | – |  | 457.5 MHz |  | 462.5 MHz |  | – |  | 467.5 MHz |  | 1 (NOTE 2) |
| 32 (NOTE 5) | XXXII (NOTE 6) | - |  |  | N/A |  |  |  | 1452 MHz |  | – |  | 1496 MHz | 1 (NOTE 3) |
| 64 | - | - |  |  |  |  |  |  | Reserved |  |  |  |  |  |
| 65 | - | - | 1920 MHz |  | – |  | 2010 MHz |  | 2110 MHz |  | – |  | 2200 MHz | 1 (NOTE 2) |
| 66 (NOTE 7) | - | - | 1710 MHz |  | - |  | 1780 MHz |  | 2110 MHz |  | - |  | 2200 MHz | 1 (NOTE 4) |
| 67 (NOTE 5) | - | - | N/A |  |  |  |  |  | 738 MHz |  | – |  | 758 MHz | 1 (NOTE 2) |
| 68 | - | - | 698 MHz |  | - |  | 728 MHz |  | 753 MHz |  | - |  | 783 MHz | 1 (NOTE 2) |
| 69 | - | - | N/A |  |  |  |  |  | 2570 MHz |  | – |  | 2620 MHz | 1 (NOTE 2, NOTE 5) |
| 70 (NOTE 9) | - | - | 1695 MHz |  | – |  | 1710 MHz |  | 1995 MHz |  | – |  | 2020 MHz | 1 (NOTE 2) |
| NOTE 1: The band is for UTRA only. NOTE 2: The band is for E-UTRA only. NOTE 3: The band is for E-UTRA and UTRA only. NOTE 4: The band is for E-UTRA and NB-IoT only. NOTE 5: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 6: Restricted to UTRA operation when dual band is configured (e.g., DB-DC-HSDPA or dual band 4C-HSDPA). The down link frequency(ies) of this band are paired with the uplink frequenc(ies) of the other FDD band (external) of the dual band configuration. NOTE 7: The range 2180-2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. NOTE 8: Band 23 is not applicable. NOTE 9: The range 2010-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz. The range 2005-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE:	For BS capable of multi-band operation, the supported operating bands may belong to different Band Categories.

# Table 4.4-2: Unpaired bands in E-UTRA and UTRA

| MSR and E-UTRA Band number | UTRA Band number | Uplink (UL) BS receive UE transmit |  |  | Downlink (DL) BS transmit UE receive |  |  | Band category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33 | a) | 1900 MHz | – | 1920 MHz | 1900 MHz | – | 1920 MHz | 3 |
| 34 | a) | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | 3 |
| 35 | b) | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | 3 |
| 36 | b) | 1930 MHz | – | 1990 MHz | 1930 MHz | – | 1990 MHz | 3 |
| 37 | c) | 1910 MHz | – | 1930 MHz | 1910 MHz | – | 1930 MHz | 3 |
| 38 | d) | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | 3 |
| 39 | f) | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | 3 |
| 40 | e) | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | 3 |
| 41 | - | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | 3 |
| 42 | - | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | 3 |
| 43 | - | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | 3 |
| 44 | - | 703 MHz | – | 803 MHz | 703 MHz | – | 803 MHz | 3 |
| 45 | - | 1447 MHz | – | 1467 MHz | 1447 MHz | – | 1467 MHz | 3 |
| 48 | - | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | 3 |

# Table 4.4-3. Void

Table 4.4-4. Void

E-UTRA is designed to operate for the carrier aggregation bands defined in Tables 5.5-2, 5.5-3 and 5.5-4 of TS 36.104 [5]. The E-UTRA channel bandwidth BWChannel for a single carrier and the Aggregated Channel Bandwidth BWChannel_CA for E-UTRA carrier aggregation are specified in Section 5.6 of TS 36.104 [5].

<End of change>