3GPP TSG-RAN WG4 Meeting #83 	R4-1704906

# Hangzhou, China, 15 - 19 May 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 4871 | rev | - | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for V2X initiation/cease of SLSS Transmission when GNSS as synchronization source |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_V2X-Pref |  |  |  |  |  |  | Date: |  | 2017-5-3 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The RRM Core requirements has been approved in RAN4#82 meeting. The test cases to verify RRM requirements for V2X sidelink communication should be introduced in specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR defines test case for V2X initiation/cease of SLSS Transmission when GNSS as synchronization source. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM tests for V2X initiation/cease of SLSS Transmission when GNSS as synchronization source will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section A.12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

A.12.3	V2X initiation/Cease of SLSS Transmission when GNSS as synchronization source

A.12.3.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to the maximum evaluation time allowed to initiate and cease SLSS transmissions defined in clause 13.3.1.2. The UE under test is configured for V2X sidelink operation on Band 47 and for WAN operation when GNSS as synchronization source. This test is applicable for a UE capable of V2X sidelink communication.

For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2V sidelink communication.

The test parameters are given in Table A.12.3.1-1.  Table A.12.3.1-2 and Table A.12.3.1-3 below. The UE under test is synchronized to GNSS and evaluates the RSPR of active cell (PCell) to initiate and cease SLSS transmissions in this test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. During T1, the RSRP of the PCell is above syncTxThreshIC and the UE is not expected to be transmitting SLSS. During T2, the RSRP of the PCell is lowered below syncTxThreshIC and the UE is expected to initiate SLSS transmissions. During T3, the RSRP of the PCell is increased back to be above syncTxThreshIC and the UE is expected to cease SLSS transmissions.

Table A.12.3.1-1: Test parameters for initiation/cease of SLSS transmissions test with GNSS as synchronization source

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 2 | One E-UTRA FDD RF channel and one V2X sidelink RF channel |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | E-UTRA FDD Cell1 on RF channel number 1 |
| CP length of Cell 1 |  | Normal |  |
| Layer 3 filtering |  | Disabled | L3 filtering is not used |
| drx-Configuration |  | DRX-V1 | As specified in Table A.3.22.2-1 |
| offsetDFN-r14 |  | 0 | Defined in 36.331: the timing offset for the UE to determine DFN timing. |
| T1 | s | 3 |  |
| T2 | s | 5.24 |  |
| T3 | s | 5.24 |  |

Table A.12.3.1-2: V2X sidelink communication configuration for initiation/cease of SLSS transmissions test with GNSS as synchronization source

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| V2X sidelink RF Channel Number |  | 1 | Band 47 |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| V2X resource pool configuration |  | As specified in Table A.3.22.3-1 (Configuration #2) | IE values unless specified otherwise in this test. |
| PSCCH reference measurement channel |  | CC.1 TDD | As specified in Table A.3.22.4.1-1 |
| PSSCH reference measurement channel |  | CD.1 TDD | As specified in Table A.3.22.4.1-2 |
| networkControlledSyncTx |  | Not configured |  |
| syncTxThreshIC | dBm/15 kHz | -95 | In SIB21 |

Table A.12.3.1-3: Cell specific test parameters for initiation/cease of SLSS transmissions test for E-UTRAN FDD

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Pattern (defined in clause A.3.2) |  | OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note2 | dBm/15 kHz | -95 |  |  |
|  | dB | 4.5 | -4.5 | 4.5 |
| RSRP Note3 | dBm/15 kHz | -90.5 | -99.5 | -90.5 |
| SCH_RP Note 3 | dBm/15 kHz | -90.5 | -99.5 | -90.5 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

A.12.3.2	Test Requirements

The SLSS transmission initiation delay is defined as the time from the beginning of time period T2 up to the moment when the UE initiates the SLSS transmission.

The SLSS transmission initiation delay shall be less than 2.08 s.

The SLSS transmission cease delay is defined as the time from the beginning of time period T3 up to the moment when the UE ceases the SLSS transmission.

The SLSS transmission cease delay shall be less than 2.08 s.

The rate of correct initiation/cease delay of SLSS transmissions observed during repeated tests shall be at least 90%.

NOTE:	The initiation/cease delay of SLSS transmissions can be expressed as: Tevaluate,SLSS + SLSS period,

Where:

Tevaluate,SLSS 	is the evaluation time for initiate/cease of SLSS, and is 1.92 sec (defined in clause 13.3.1.1) for the parameters in this test;

SLSS period	set as 320ms in this test.