3GPP TSG-RAN WG4 Meeting #83 	R4-1704907

# Hangzhou, China, 15 - 19 May 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 4872 | rev | - | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for V2X initiation/cease of SLSS Transmission when GNSS as synchronization source |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_V2X-Pref |  |  |  |  |  |  | Date: |  | 2017-5-3 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The RRM Core requirements has been approved in RAN4#82 meeting. The test cases to verify RRM requirements for V2X sidelink communication should be introduced in specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR defines test case for V2X UE transmission timing accutracy when GNSS as synchronization source. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM tests for V2X UE transmission timing accutracy when GNSS as synchronization source will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section A.12.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

A.12.1	V2X UE Transmission Timing Accuracy Test

A.12.1.1	Test Purpose and Environment

The purpose of this test is to verify the timing requirements for V2X sidelink transmissions specified in clause 13.2. The UE under test is configured for V2X sidelink operation on Band 47 and for WAN operation when GNSS as synchronization source. This test is applicable for a UE capable of V2X sidelink communication.

For this test, the UE is triggered by the test loop function to transmit for V2X sidelink Communication.

Table A.12.1.1-1, Table A.12.1.1-2 and Table A.12.1.1-3 define test parameters for UE transmit timing accuracy tests for V2X. GNSS is used as synchronization source during the test. The test system can emulate and send the GNSS signal to the test UE and the transmit timing accuracy is verified by measuring the reception of the PSSCH sent by the UE.

Table A.12.1.1-1: Test Parameters for V2X UE Transmit Timing Accuracy Test

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 2 | One E-UTRA FDD RF channel and one V2X sidelink RF channel |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | E-UTRA FDD Cell1 on RF channel number 1 |
| CP length of Cell 1 |  | Normal |  |
| drx-Configuration |  | DRX_V1 | As specified in Table A.3.22.2-1 |
| offsetDFN-r14 |  | 0 | Defined in 36.331: the timing offset for the UE to determine DFN timing. |

Table A.12.1.1-2: V2X sidelink communication configuration for V2X UE Transmit Timing Accuracy Test

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| V2X sidelink RF Channel Number |  | 1 | Band 47 |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| V2X resource pool configuration |  | As specified in Table A.3.22.3-1 (Configuration #2) |  |
| PSCCH reference measurement channel |  | CC.1 TDD | As specified in Table A.3.22.4.1-1 |
| PSSCH reference measurement channel |  | CD.1 TDD | As specified in Table A.3.22.4.1-2 |
| Propagation condition |  | AWGN |  |

Table A.13.x.1-3: Cell specific test parameters for V2X UE Transmit Timing Accuracy Test

| Parameter | Unit | Value |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| BWchannel | MHz | 10 |
| OCNG Pattern (defined in clause A.3.2) |  | OP.2 FDD |
| PBCH_RA | dB | 0 |
| PBCH_RB |  |  |
| PSS_RA |  |  |
| SSS_RA |  |  |
| PCFICH_RB |  |  |
| PHICH_RA |  |  |
| PHICH_RB |  |  |
| PDCCH_RA |  |  |
| PDCCH_RB |  |  |
| PDSCH_RA |  |  |
| PDSCH_RB |  |  |
| OCNG_RANote 1 |  |  |
| OCNG_RBNote 1 |  |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -4.5 |
| RSRP Note3 | dBm/15 kHz | -95 |
| SCH_RP Note 3 | dBm/15 kHz | -95 |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

A.12.1.2	Test requirements

For parameters specified in Tables A.11.1.1-1, Tables A.11.1.1-2 and Tables A.11.1.1-3, the timing accuracy for V2V sidelink transmission shall be within the limits defined in clause 13.2.2. The timing accuracy is verified by using PSSCH transmissions.