3GPP TSG-RAN WG4 #83	R4- 1705479

May 15th – 19th, 2017

Hangzhou, China

Agenda item:	5.3.1

Source: 	Qualcomm Incorporated

Title: 	LAA REFSENS exclusion range in case of two contiguous UL licensed carriers

Document for:	Approval

# Introduction

In this contribution, we discuss the need to update/redefine the LAA REFSENS exclusion region in case of B46 combinations with two intra-band contiguous UL CA in licensed bands.

# Discussion

Band 46 MSD due to harmonic coming from license carriers is specified in TS 36.101 by defining a REFSENS exclusion region, i.e. a frequency range in which B46 REFSENS requirement does not apply and it is not tested [1][2]. In RAN4 #81, the exclusion region was also updated to account for the higher desense which could affect LAA 10MHz channels [3][4]. In this contribution, we discuss the need to update/redefine the exclusion region in case of B46 combinations with two intra-band contiguous UL CA in licensed bands.

Table 1. Band 46 Reference sensitivity measurement exclusion region in MHz.

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Channel BW in UL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Licensed Component Carriers | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz |
| 1 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 2 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 3 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 4 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 5 | 7 | +/- 10 | +/- 20 |  |  |
| 71 | 2 | +/- 15 | +/- 25 | +/- 38 | +/- 50 |
| 8 | 6 | +/- 10 | +/- 20 |  |  |
| 11 | 4 | +/- 15 | +/- 25 |  |  |
| 13 | 7 | +/- 10 | +/- 20 |  |  |
| 19 | 7 | +/- 10 | +/- 20 | +/- 30 |  |
| 21 | 4 | +/- 15 | +/- 25 | +/- 38 |  |
| 26 | 7 | +/- 10 | +/- 20 | +/- 30 |  |
| 28 | 7 | +/- 10 | +/- 20 | +/- 30 | +/- 40 |
| 28 | 8 | +/- 10 | +/- 20 | +/- 30 | +/- 40 |
| 39 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 41 | 2 | +/- 15 | +/- 25 | +/- 38 | +/- 50 |
| 66 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| NOTE 1: Even though UL harmonic does not fall directly into Band 46 the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the harmonic order. |  |  |  |  |  |

Table 1 summarized the B46 Reference sensitivity measurement exclusion region in MHz as agreed in [4]. As it can be observed the width of the exclusion region depends on the UL carrier frequency and channel bandwidth. The table was defined by keeping the same exclusion region for different combinations belonging to the same harmonic order. The exclusion region width does not scale linearly with the UL channel BW. This is due to the fact the smaller channel BW allocation would require proportionally larger exclusion region due to the higher PSD of the harmonic interference. This is shown in Figure 1, where a pictorial representation of the agreed LAA REFSENS exclusion region is shown for the 3rd harmonic case. The dark blue region represents the “just miss” region, which for the 5MHz UL configuration is 3 (harmonic order) x 5MHz (channel BW) = 15MHz. The light blue region represents the extra exclusion region compared to the just miss condition. For the 5MHz UL case, the ratio between additional offset and the just miss region is 100% (15MHz/15MHz), while for the case of 20MHz is 50% (30MHz/60MHz). In other words, the 5MHz requires proportionally larger extra offset.

Figure 1. Example of LAA exclusion region for 3rd harmonic case.

A summary of the ratio between additional offset and the "just miss" region is shown in Table 2. The negative values for some harmonic cases implies that the exclusion region is confined inside the just miss region.

The open question we would like to discuss in this paper is how to extend the exclusion region when 2 contiguous carriers are used for UL transmission in licensed bands. First of all two observations can be made:

- In case of 2UL intra-band contiguous CA for the licensed component carriers, the region in which the harmonic interference will spread will be larger compared to the single carrier case.

- In case of 2UL intra-band contiguous CA for the licensed component carriers, the direct hit MSD can be lower compared to the single case (due to the lower PSD of the harmonic interference).

Based on the observations above a rigorous analysis would require to recalculate direct hit MSD and the amount of harmonic side lobe rejection needed for each 2UL case. Another approach would be to extend the existing exclusion region table with an heuristic method. One possibility would be for instance to decide on X% ratio between additional offset and the "just miss" region based on the trend observed in Table 2. If for instance we assume the case of 20+20 CA case and X=50% for the third harmonic (same as the 20MHz single carrier case), we would get a total exclusion region of 180MHz = +/-90MHz. This can be obtained with the following calculation: 120MHz (just miss area=40x3) + 60MHz (50% of 120MHz). This would be exactly doubled compared to the region applied to the single 20MHz case. If instead X=40% is chosen, the total exclusion region would be 168MHz = +/-84MHz.

Table 2. Ratio between additional offset and the "just miss" region.

| Ratio between additional offset and "just miss" region (in %) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Licensed Component Carriers | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz |
| 1 | 3 | 100% | 53% | 56% | 50% |
| 2 | 3 | 100% | 53% | 56% | 50% |
| 3 | 3 | 100% | 53% | 56% | 50% |
| 4 | 3 | 100% | 53% | 56% | 50% |
| 5 | 7 | -43% | -43% |  |  |
| 71 | 2 | 200% | 150% | 153% | 150% |
| 8 | 6 | -33% | -33% |  |  |
| 11 | 4 | 50% | 25% |  |  |
| 13 | 7 | -43% | -43% |  |  |
| 19 | 7 | -43% | -43% | -43% |  |
| 21 | 4 | 50% | 25% | 27% |  |
| 26 | 7 | -43% | -43% | -43% |  |
| 28 | 7 | -43% | -43% | -43% | -43% |
| 28 | 8 | -50% | -50% | -50% | -50% |
| 39 | 3 | 100% | 53% | 56% | 50% |
| 41 | 2 | 200% | 150% | 153% | 150% |
| 66 | 3 | 100% | 53% | 56% | 50% |

Our proposal is to use X as the one supported by the maximum channel BW defined in Table 2. For instance for the case of Band 1, since 20MHz is supported X=50%, while for the case of Band 5 X=-43% is chosen corresponding to the 10MHz channel BW.

The resulting exclusion region for the case of 2UL intra-band contiguous CA in licensed carriers is shown in Table 3. The table shows the exclusion region as a function of the aggregated UL bandwidth. For instance, 30MHz can corresponds to 15MHz + 15MHz, 20MHz + 10MHz or 10MHz + 20MHz.

Proposal 1: in case of B46 aggregated with 2UL intra-band contiguous CA licensed carriers, B46 reference sensitivity measurement exclusion region should be specified as in Table 3.

Table 3. Band 46 Reference sensitivity measurement exclusion region in MHz for the case of intra-band 2UL contiguous CA in licensed carriers.

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Aggregated Channel BW in UL |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Licensed Component Carriers | Harmonic order | 20MHz | 25MHz | 30MHz | 35MHz | 40MHz |
| 1 | 3 | +/- 45 | +/- 56 | +/- 68 | +/- 79 | +/- 90 |
| 2 | 3 | +/- 45 | +/- 56 | +/- 68 | +/- 79 | +/- 90 |
| 3 | 3 | +/- 45 | +/- 56 | +/- 68 | +/- 79 | +/- 90 |
| 4 | 3 | +/- 45 | +/- 56 | +/- 68 | +/- 79 | +/- 90 |
| 5 | 7 | +/- 40 |  |  |  |  |
| 71 | 2 | +/- 50 | +/- 63 | +/- 75 | +/- 88 | +/- 100 |
| 8 | 6 | +/- 40 |  |  |  |  |
| 11 | 4 | +/- 50 |  |  |  |  |
| 13 | 7 | +/- 40 |  |  |  |  |
| 19 | 7 | +/- 40 | +/- 50 | +/- 60 |  |  |
| 21 | 4 | +/- 51 | +/- 63 | +/- 76 |  |  |
| 26 | 7 | +/- 40 | +/- 50 | +/- 60 |  |  |
| 28 | 7 | +/- 40 | +/- 50 | +/- 60 | +/- 70 | +/- 80 |
| 28 | 8 | +/- 40 | +/- 50 | +/- 60 | +/- 70 | +/- 80 |
| 39 | 3 | +/- 45 | +/- 56 | +/- 68 | +/- 79 | +/- 90 |
| 41 | 2 | +/- 50 | +/- 63 | +/- 75 | +/- 88 | +/- 100 |
| 66 | 3 | +/- 45 | +/- 56 | +/- 68 | +/- 79 | +/- 90 |

# Conclusions

In this contribution, we discussed possible approaches to update/extend the LAA REFSENS exclusion region in case of B46 combinations with two contiguous intra-band UL CA in licensed bands. We showed some heuristic approach which could avoid running specific analysis for each combination and we made the following proposal:

Proposal 1: in case of B46 aggregated with 2UL intra-band contiguous CA licensed carriers, B46 reference sensitivity measurement exclusion region should be specified as in Table 3.

# Reference

- R4-168824, “Improvement of REFSENS requirement specification for band 46 CA combos”, Qualcomm Incorporated, Rel 13 CR.

- R4-168825, “Improvement of REFSENS requirement specification for band 46 CA combos”, Qualcomm Incorporated, Rel 14 CR.

- R4-1610451, “MSD and exclusion region specification for 10MHz LAA channels”, Qualcomm Incorporated.

- R4-1610452, “MSD and exclusion region specification for 10MHz LAA channels”, Qualcomm Incorporated.