3GPP TSG-RAN WG4 Meeting #83 	R4-1705628

# Hangzhou, China, 15th – 19th May 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.141 | CR | 1052 | rev | - | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

4

| Title: | Introduce conformance test for eLAA PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eLAA-Perf |  |  |  |  |  |  | Date: |  | 2017-05-03 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no conformance test for eLAA PUSCH |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add conformance test for eLAA PUSCH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no conformance test for eLAA PUSCH |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------------Beginning of Change ----------------------------------------------------------

### 8.2.8	Performance requirements of PUSCH with Frame structure type 3

#### 8.2.8.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A17. The performance requirements assume HARQ re-transmissions.

A test for a specific number of receive antenna is only applicable if the BS supports it.

#### 8.2.8.2	Minimum Requirement

The minimum requirement is in TS 36.104 [2] subclause 8.2.8.

#### 8.2.8.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR when the frame structure type 3 is used for PUSCH transmission.

#### 8.2.8.4	Method of test

##### 8.2.8.4.1	Initial Conditions

Test environment:	Normal, see subclause D.2.

RF channels to be tested:	M; see subclause 4.7.

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex I.3.2.

##### 8.2.8.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth, defined in Table 8.2.8.4.2-1.

Table 8.2.8.4.2-1: AWGN power level at the BS input

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 20 | -80.4dBm / 18MHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A17 and the test parameters in Table 8.2.8.4.2-2.

Table 8.2.8.4.2-2 Test parameters for testing PUSCH with frame structure type 3

| Parameter | Unit | Mode A |
| --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 |
| RV sequences |  | 0, 2, 3, 1, 0, 2, 3, 1 |
| PUSCH starting position |  | ‘01’ |
| PUSCH ending symbol |  | ‘0’ |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex B.

4)	Adjust the equipment so that required SNR specified in Table 8.2.8.5-1 is achieved at the BS input.

5)	For each of the reference channels in Table 8.2.8.5-1 applicable for the base station, measure the throughput, according to annex E.

#### 8.2.8.5	Test Requirement

The throughput measured according to subclause 8.2.8.4.2 shall not be below the limits for the SNR levels specified in Table 8.2.8.5-1.

# Table 8.2.8.5-1 Minimum requirements for PUSCH, 20 MHz Channel Bandwidth, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | EPA 5Hz Low | A17-1 | 70% | TBD |
|  |  |  |  | A17-2 | 70% | TBD |
|  | 4 | Normal | EPA 5Hz Low | A17-1 | 70% | TBD |
|  |  |  |  | A17-2 | 70% | TBD |

-----------------------------------------------------------unchanged section is skiped----------------------------------------------------

# A17 Fixed Reference Channels for PUSCH of Frame structure type 3

# Table A.17-1 FRC parameters for performance requirements (QPSK 1/3)

| Reference channel | A17-1 |
| --- | --- |
| Uplink resource allocation type | 3 |
| Allocated resource blocks | 10 |
| DFT-OFDM Symbols per subframe | 11 |
| Modulation | QPSK |
| Code rate | 1/3 |
| Payload size (bits) | 872 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Coded block size including 12bits trellis termination (bits) | 2700 |
| Total number of bits per sub-frame | 2640 |
| Total symbols per sub-frame | 1320 |

# Table A.17-2 FRC parameters for performance requirements (16QAM 3/4)

| Reference channel | A17-2 |
| --- | --- |
| Uplink resource allocation type | 3 |
| Allocated resource blocks | 10 |
| DFT-OFDM Symbols per subframe | 11 |
| Modulation | 16QAM |
| Code rate | 3/4 |
| Payload size (bits) | 4008 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Coded block size including 12bits trellis termination (bits) | 12108 |
| Total number of bits per sub-frame | 5280 |
| Total symbols per sub-frame | 1320 |

-----------------------------------------------------------End of Change ----------------------------------------------------------