3GPP TSG-RAN WG4 Meeting #84	R4-1707194

# Berlin, Germany, 21 – 25 August, 2017

Source:	SoftBank Corp.

Title:	TP for TR36.715-04-01: the support of CA_1A-3A-11A-28A BCS0

Agenda item:	8.4.2

Document for:	Approval

1.  Introduction

1A-3A-11A-28A BCS0 was approved in 4DL/1UL basket WID [1]. This paper is to evaluate IM/harmonics and relaxation required, including the relevant TP to complete this combination.

[Note to the rapporteur: To create a big CR, table 7-10 of the main text can be used to implement REFSENS exception tables: relevant table no of 36.101 is indicated in the brackets. For table 7, watch out for red letters in Note 9. For table 10, only the red letters should be implemented.]

2. Basic Configurations

As written in the WID, the following configurations are assumed for B1+B3+B11+B28.

Table 1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-3-11-28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |

Table 2: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-3A-11A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 11 |  |  | Yes | Yes |  |  |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

3.  Co-existence

On BS side, there is no need to analyse harmonic/IMD in REL-15. On UE side, there are two harmonic issues within this 4DL CA, B28 Tx to B11 Rx and toward B1 Rx (HTF assumed for both in constituent low-order CAs). The UE issues are discussed in the following sections.

4.  ΔTIB and ΔRIB

In terms of ΔTIB and ΔRIB, we can reuse the results of constituent 3DL CAs.

Table 3: ΔTIB of constituent 3DL/1UL

| ΔTIB | 1+3+11 | 1+3+28 | 1+11+28 | 3+11+28 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.3 | 0.3 |  | 0.3 |
| B3 | 0.8 | 0.3 |  | 0.8 | 0.8 |
| B11 | 0.9 |  | 0.4 | 0.9 | 0.9 |
| B28 |  | 0.6 | 0.6 | 0.6 | 0.6 |
| NOTE 11: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 8 NOTE 12: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 3 or Band 8. |  |  |  |  |  |

Table 4: ΔRIB of constituent 3DL/1UL

| ΔRIB | 1+3+11 | 1+3+28 | 1+11+28 | 3+11+28 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | 0 | 0 | 0 |  | 0 |
| B3 | 0.3 | 0 |  | 0.3 | 0.3 |
| B11 | 0.5 |  | 0 | 0.5 | 0.5 |
| B28 |  | 0.2 | 0.2 | 0.2 | 0.2 |
| NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 8 NOTE 11: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 3 or Band 8. |  |  |  |  |  |

Then the following values are proposed.

Table 5: Proposed ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-3A-11A-28A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |

Table 6: Proposed ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-3A-11A-28A | 1 | 0 |
|  | 3 | 0.3 |
|  | 11 | 0.5 |
|  | 28 | 0.2 |

5.  REFSENS

Unfortunately, there are a couple of REFSENS exceptions to be addressed in this 4DL.

As noted above, two harmonic relations are included in the 4DL CA. B28 Tx would generate the 2nd harmonic to B11 Rx and the 3rd to B1 Rx. This issue has already been discussed in 3DL of B1+B11+B28 [2] and the same result can be used for REFSENS exception of this 4DL CA as below.

(Note that in the 3DL, notes on uplink configuration are missed and to be corrected in a companion CR [3]. The following TP assumes the CR is agreed.)

Table 7 (7.3.1A-0a): Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-11A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 3 |  |  | -96.7 | -93.7 | -91.9 | -90.7 |  |
|  | 11 |  |  | -99.5 | -96.5 |  |  |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-11A-28A9,10 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -96.7 | -93.7 | -91.9 | -90.7 |  |
|  | 11 |  |  | -75.2 | -75.2 |  |  |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_1A-3A-42A, CA_2A-48A, CA_2A-48C, CA_3A-19A-42A, CA_1A-3A-19A-42A and CA_3A-28A-41A-42A. FHD = 0MHz for CA_11A-28A and CA_1A-3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 8 (7.3.1A-0b): Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-11A-28A4 | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-11A-28A5 | 28 |  |  | 12 | 25 |  |  | FDD |
| NOTE 4: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a high band. |  |  |  |  |  |  |  |  |

In B1+B3, relaxation due to proximity of B1 Tx/B3 Rx should also be included. The relevant requirements are shown in Table 9 and 10.

Table 9 (7.3.1A-0bD1): Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-11A-28A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34,9 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 11 |  |  | -99.5 | -96.5 |  |  |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 10 (7.3.1A-0bD2): Uplink configuration for the low band (exceptions for four bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-5A-7A CA_1A-3A-5A-7A-7A CA_1A-3A-5A-40A CA_1A-3A-5A-41A CA_1A-3A-7A-7A-26A CA_1A-3A-7A-8A CA_1A-3A-7A-20A CA_1A-3A-7A-26A CA_1A-3A-7A-28A CA_1A-3A-7C-28A CA_1A-3A-7A-40C CA_1A-3A-7A-42A CA_1A-3A-8A-11A CA_1A-3A-8A-40A CA_1A-3A-11A-28A CA_1A-3A-19A-21A CA_1A-3A-19A-42A CA_1A-3A-19A-42C CA_1A-3A-20A-28A CA_1A-3A-20A-42A CA_1A-3A-21A-28A CA_1A-3A-21A-42A CA_1A-3A-21A-42C CA_1A-3A-28A-42A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

6.  Summary

This paper investigates all the necessary elements to complete CA_1A-3A-11A-28A. The relevant TP is shown in Annex.

7.  Reference

[1]      RP-170963    New WID: LTE Advanced inter-band CA Rel-15 for 4DL/1UL, Ericsson (Rapporteur).

[3]      R4-1702225   TP for TR36.714-03-01: Support of LTE-A Inter-band Carrier Aggregation (3DL/1UL) of Band 1, Band 11 and Band 28, SoftBank

[4]      R4-1707191  Corrections of Notes on REFSENS exception in 3DL/1UL of 1A-11A-28A and 3A-11A-28A (Rel-14 CR), SoftBank

Annex: TP for TR36.715-04-01 (v0.0.1)

[Unchanged portions skipped]

5.X	CA_1A-3A-11A-28A_BCS0

5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-3-11-28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |

Table 5.X.1-2: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-3A-11A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 11 |  |  | Yes | Yes |  |  |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

5.X.2	UE co-existence studies

On UE side, there are two harmonic issues within this 4DL CA, B28 Tx to B11 Rx and toward B1 Rx (HTF assumed for both in constituent low-order CAs). The UE issues are discussed in the following sections.

5.X.3	∆TIB and ∆RIB values

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c and ΔRIB,c is defined for applicable bands in Table 5.X.3-1 and 5.X.3-2 respectively.

Table 5.X.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-11A-28A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |

Table 5.X.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-11A-28A | 1 | 0 |
|  | 3 | 0.3 |
|  | 11 | 0.5 |
|  | 28 | 0.2 |

5.X.4	REFSENS requirements

For the REFSENS of CA_1A-3A-11A-28A the same REFSENS requirements for the constituent 2DL and 3DL CA fall-back modes can be reused. As noted above, two harmonic relations are included in the 4DL CA. B28 Tx would generate the 2nd harmonic to B11 Rx and the 3rd to B1 Rx. Exceptions are as below.

Table 5.X.4-1 : Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-11A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 3 |  |  | -96.7 | -93.7 | -91.9 | -90.7 |  |
|  | 11 |  |  | -99.5 | -96.5 |  |  |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-11A-28A9,10 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -96.7 | -93.7 | -91.9 | -90.7 |  |
|  | 11 |  |  | -75.2 | -75.2 |  |  |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_1A-3A-42A, CA_2A-48A, CA_2A-48C, CA_3A-19A-42A, CA_1A-3A-19A-42A and CA_3A-28A-41A-42A. FHD = 0MHz for CA_11A-28A and CA_1A-3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.X.4-2 : Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-11A-28A4 | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-11A-28A5 | 28 |  |  | 12 | 25 |  |  | FDD |
| NOTE 4: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a high band. |  |  |  |  |  |  |  |  |

In B1+B3, relaxation due to proximity of B1 Tx/B3 Rx should also be included. The relevant REFSENS relaxations are as below.

Table 5.X.4-3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-11A-28A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 34,9 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 11 |  |  | -99.5 | -96.5 |  |  |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.X.4-4: Uplink configuration for the low band (exceptions for four bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-11A-28A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"