3GPP TSG-RAN WG4 Meeting #84	R4-1707242

Berlin, Germany, 21 – 25 Aug, 2017

Source:	Mediatek Inc

Title:	TP for TR36.715-04-01: the support of CA_1A-1A-3A-7A_1UL_BCS0

Agenda item:	8.4.2

Document for:	Approval

1.  Introduction

CA_1A-1A-3A-7A_1UL_BCS0 was approved in 4DL/1UL basket WID in the RAN-P [1]. This paper is to provide TP for the completion of this combination.

2.  Reference

[1]      RP-171093 LTE Advanced inter-band CA Rel-15 for 4DL/1UL, RAN #76.

Annex: TP for TR36. 715-04-01 (va.b.c)

[Unchanged portions skipped]

CA_1A-1A-3A-7A_1UL_BCS0

5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-1A-3A-7A | 1 | See the CA_1A-1A Bandwidth combination set 0 in the Table below |  |  |  |  |  | 80 | 0 |
|  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |

# Table 5.x.1-2: Supported E-UTRA bandwidths of CA_1A-1A configuration

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_1A-1A | 5, 10, 15, 20 | 5, 10, 15, 20 | 40 | 0 |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

5.x.2	Co-existence studies

The 2nd and 3rd harmonic and IMD products caused in the BS by transmitting CA_1A-1A-3A-7A follow from

those derived for the constituent 2DL and 3DL CA fall-back modes , which have been already analyzed.

5.x.3	ΔTIB,c and ΔRIB,c values

The ΔTIB,c and ΔRIB,c is defined in the same way as CA_1A-3A-7A. Same values of TIB,c and ΔRIB,c can be re-used.

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-1A-3A-7A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |

Table 5.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-1A-3A-7A | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |

5.x.4	REFSENS requirements

Additions in 36.101 Table 7.3.1A-0bC for this combination is specified in Table 5.x.4-1 and 5.x.4-2.

Table 5.x.4-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for three bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-1A-3A-7A9 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |
| CA_1A-1A-3A-7A10 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |
| NOTE 9: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 7, the requirement applies regardless of channel bandwidth in Band 1. NOTE 10: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 7, the requirement applies regardless of channel bandwidth in Band 1. NOTE 11: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.x.4-2: Uplink configuration for the uplink band (exceptions for three bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-1A-3A-7A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"