3GPP TSG-RAN WG4 Meeting #84 	R4-1708314

# Berlin, Germany, August 21 – 25, 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 5127 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 36.133 Intra and inter-frequency RSRP and RSRQ measurement accuracies for UE cat M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMTC-Core |  |  |  |  |  |  | Date: |  | 2017-08-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Inter-frequency RSRP and intra- and inter-frequency RSRQ measurement accuracies required for Rel-14 category M1 are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added requirements within brackets to 9.1.21.6-9.1.21.16 for normal conditions and, where applicable, low Io range. Corrected the heading of 9.1.21.15. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurement accuracy requirements for Rel-14 cat M1 will not be clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.21, 9.1.21.1 – 9.1.21.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

### 9.1.21	Measurement accuracy for UE category M1

#### 9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A

The requirements for absolute accuracy of RSRP in this clause apply to a cell on the same frequency as that of the serving cell for UE category M1.

The accuracy requirements in Table 9.1.21.1-1 and Table 9.1.21.1-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.1 for a corresponding Band

At least 1 DL subframe per radio frame of measured cell is available at the UE for RSRP measurement assuming measured cell is identified cell.

Table 9.1.21.1-1: RSRP Intra frequency absolute accuracy for UE category M1 with CE mode A for FDD and TDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 7 | 10 | -6 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| 9 | 12 | -6 dB | FDD-M1_A, TDD-M1_A, FDD-M1_D, FDD-M1_E, TDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.21.1-2: RSRP Intra frequency absolute accuracy for UE category M1 with CE mode A for HD-FDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 7 | 10 | -6 dB | FDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| 9 | 12 | -6 dB | FDD-M1_A, FDD-M1_D, FDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### 9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A

The relative accuracy of RSRP is defined as the RSRP measured from one cell compared to the RSRP measured from another cell on the same frequency for category M1 UE.

The accuracy requirements in Table 9.1. 21.2-1 and Table 9.1. 21.2-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP1,2|dBm according to Annex B.3.8 for a corresponding Band.

At least 1 DL subframe per radio frame of measured cell is available at the UE for RSRP measurement assuming measured cell is identified cell.

Table 9.1.21.2-1: RSRP Intra frequency relative accuracy for UE category M1 with CE mode A for FDD and TDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 5 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 4 | dBm/BWChannel |
| 3 | 4 | -3 dB | FDD-M1_A, TDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| 4 | 4 | -6 dB | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.21.2-2: RSRP Intra frequency relative accuracy for UE category M1 with CE mode A for HD-FDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 5 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 4 | dBm/BWChannel |
| 3 | 4 | -3 dB | FDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| 4 | 4 | -6 dB | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### 9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B

The requirements for absolute accuracy of RSRP in this clause apply to a cell on the same frequency as that of the serving cell for UE category M1.

The accuracy requirements in Table 9.1.21.3-1 and Table 9.1.21.3-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.1 for a corresponding Band

At least 1 DL subframe per radio frame of measured cell is available at the UE for RSRP measurement assuming measured cell is identified cell.

Table 9.1.21.3-1: RSRP Intra frequency absolute accuracy for UE category M1 with CE mode B for FDD and TDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 8 | 11 | -15≤Ês/Iot≤-12 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
| 7 | 10 | -12 dB | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| 10 | 13 | -15≤Ês/Iot≤-12 dB | FDD-M1_A, TDD-M1_A, FDD-M1_D, FDD-M1_E, TDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| 9 | 12 | -12 dB |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.21.3-2: RSRP Intra frequency absolute accuracy for UE category M1 with CE mode B for HD-FDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 8 | 11 | -15≤Ês/Iot≤-12 dB | FDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E | -119 | N/A | -70 |
| 7 | 10 | -12 dB | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| 10 | 13 | -15≤Ês/Iot≤-12 dB | FDD-M1_A, FDD-M1_D, FDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| 9 | 12 | -12 dB |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### 9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B

The relative accuracy of RSRP is defined as the RSRP measured from one cell compared to the RSRP measured from another cell on the same frequency for category M1 UE.

The accuracy requirements in Table 9.1.21.4-1 and Table 9.1.21.4-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP1,2|dBm according to Annex B.3.8 for a corresponding Band.

At least 1 DL subframe per radio frame of measured cell is available at the UE for RSRP measurement assuming measured cell is identified cell.

Table 9.1.21.4-1: RSRP Intra frequency relative accuracy for UE category M1 with CE mode B for FDD and TDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 5 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 4 | dBm/BWChannel |
| 4 | 4 | -12 dB | FDD-M1_A, TDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| 5 | 5 | -15≤Ês/Iot≤-12 dB | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.21.4-2: RSRP Intra frequency relative accuracy for UE category M1 with CE mode B for HD-FDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 5 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 4 | dBm/BWChannel |
| 4 | 4 | -12 dB | FDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| 5 | 5 | -15≤Ês/Iot≤-12 dB | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### 9.1.21.5	RSRP Measurement Report Mapping

The reporting range of RSRP is the same as defined in section 9.1.4.

#### 9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A

The requirements for absolute accuracy of RSRQ in this clause apply to a cell on the same frequency as that of the serving cell.

The accuracy requirements in Tables 9.1.21.6-1 and 9.1.21.6-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.1 for a corresponding Band

Table 9.1.21.6-1: RSRQ Intra frequency absolute accuracy UE category M1 with CE mode A for FDD and TDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| [±4] | [TBD] | -3 dB | FDD-M1_A, TDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [4] | [TBD] | -6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.21.6-2: RSRQ Intra frequency absolute accuracy UE category M1 with CE mode A for HD-FDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| [4] | [TBD] | -3 dB | FDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [4] | [TBD] | -6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### 9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B

The requirements for absolute accuracy of RSRQ in this clause apply to a cell on the same frequency as that of the serving cell.

The accuracy requirements in Tables 9.1.21.7-1 and 9.1.21.7-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.1 for a corresponding Band

Table 9.1.21.7-1: RSRQ Intra frequency absolute accuracy UE category M1 with CE mode B for FDD and TDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| [5] | [TBD] | -12 dB | FDD-M1_A, TDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [5] | [TBD] | -15≤Ês/Iot≤-12 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.21.7-2: RSRQ Intra frequency absolute accuracy UE category M1 with CE mode B for HD-FDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| [5] | [TBD] | -12 dB | FDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [5] | [TBD] | -15≤Ês/Iot≤-12 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### 9.1.21.8	RSRQ Measurement Report Mapping

The reporting range of RSRQ is the same as defined in section 9.1.7.

#### 9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A

The requirements for absolute accuracy of RSRP in this clause apply to a cell on another frequency than that of the serving cell for UE category M1.

The accuracy requirements in Table 9.1.21.9-1 and Table 9.1.21.9-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.1 for a corresponding Band

At least 1 DL subframe per radio frame of measured cell is available at the UE for RSRP measurement assuming measured cell is identified cell.

Table 9.1.21.9-1: RSRP Inter frequency absolute accuracy for UE category M1 with CE mode A for FDD and TDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| [±7] | [TBD] | -6 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| [TBD] | [TBD] | -6 dB | FDD-M1_A, TDD-M1_A, FDD-M1_D, FDD-M1_E, TDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.21.9-2: RSRP Inter frequency absolute accuracy for UE category M1 with CE mode A for HD-FDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| [±7] | [TBD] | -6 dB | FDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| [TBD] | [TBD] | -6 dB | FDD-M1_A, FDD-M1_D, FDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### 9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A

The relative accuracy of RSRP is defined as the RSRP measured from one cell compared to the RSRP measured from another cell on another frequency for category M1 UE.

The accuracy requirements in Table 9.1.21.10-1 and Table 9.1.21.10-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP1,2|dBm according to Annex B.3.8 for a corresponding Band.

At least 1 DL subframe per radio frame of measured cell is available at the UE for RSRP measurement assuming measured cell is identified cell.

Table 9.1.21.10-1: RSRP Inter frequency relative accuracy for UE category M1 with CE mode A for FDD and TDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 5 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 4 | dBm/BWChannel |
| [7] | [TBD] | -3 dB | FDD-M1_A, TDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [7] | [TBD] | -6 dB | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.21.10-2: RSRP Inter frequency relative accuracy for UE category M1 with CE mode A for HD-FDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 5 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 4 | dBm/BWChannel |
| [7] | [TBD] | -3 dB | FDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [7] | [TBD] | -6 dB | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### 9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B

The requirements for absolute accuracy of RSRP in this clause apply to a cell on another frequency than that of the serving cell for UE category M1.

The accuracy requirements in Table 9.1.21.11-1 and Table 9.1.21.11-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.1 for a corresponding Band

At least 1 DL subframe per radio frame of measured cell is available at the UE for RSRP measurement assuming measured cell is identified cell.

Table 9.1.21.11-1: RSRP Inter frequency absolute accuracy for UE category M1 with CE mode B for FDD and TDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| [8] | [TBD] | -15≤Ês/Iot≤-12 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
| [7] | [TBD] | -12 dB | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| [TBD] | [TBD] | -15≤Ês/Iot≤-12 dB | FDD-M1_A, TDD-M1_A, FDD-M1_D, FDD-M1_E, TDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| [TBD] | [TBD] | -12 dB |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.21.11-2: RSRP Inter frequency absolute accuracy for UE category M1 with CE mode B for HD-FDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| [8] | [TBD] | -15≤Ês/Iot≤-12 dB | FDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E | -119 | N/A | -70 |
| [7] | [TBD] | -12 dB | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| [TBD] | [TBD] | -15≤Ês/Iot≤-12 dB | FDD-M1_A, FDD-M1_D, FDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| [TBD] | [TBD] | -12 dB |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### 9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B

The relative accuracy of RSRP is defined as the RSRP measured from one cell compared to the RSRP measured from another cell on another frequency for category M1 UE.

The accuracy requirements in Table 9.1.21.12-1 and Table 9.1.21.12-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP1,2|dBm according to Annex B.3.8 for a corresponding Band.

At least 1 DL subframe per radio frame of measured cell is available at the UE for RSRP measurement assuming measured cell is identified cell.

Table 9.1.21.12-1: RSRP Inter frequency relative accuracy for UE category M1 with CE mode B for FDD and TDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 5 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 4 | dBm/BWChannel |
| [8] | [TBD] | -12 dB | FDD-M1_A, TDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [7] | [TBD] | -15≤Ês/Iot≤-12 dB | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.21.12-2: RSRP Inter frequency relative accuracy for UE category M1 with CE mode B for HD-FDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 5 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 4 | dBm/BWChannel |
| [8] | [TBD] | -12 dB | FDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [7] | [TBD] | -15≤Ês/Iot≤-12 dB | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### 9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A

The requirements for absolute accuracy of RSRQ in this clause apply to a cell that has different carrier frequency from the serving cell.

The accuracy requirements in Tables 9.1.21.13-1 and 9.1.21.13-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.3 for a corresponding Band

Table 9.1.21.13-1: RSRQ Inter frequency absolute accuracy UE category M1 with CE mode A for FDD and TDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| [4] | [TBD] | -3 dB | FDD-M1_A, TDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [4] | [TBD] | -6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.21.13-2: RSRQ Inter frequency absolute accuracy UE category M1 with CE mode A for HD-FDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| [4] | [TBD] | -3 dB | FDD-M1_A, TDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [4] | [TBD] | -6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### 9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A

The relative accuracy of RSRQ in inter frequency case is defined as the RSRQ measured from one cell compared to the RSRQ measured from another cell on a different frequency.

The accuracy requirements in Tables 9.1.21.14-1 and 9.1.21.14-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP1,2|dBm according to Annex B.3.4 for a corresponding Band

| Channel 1_Io -Channel 2_Io |  20 dB

Table 9.1.21.14-1: RSRQ Inter frequency relative accuracy UE category M1 with CE mode A for FDD and TDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 5 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 4 | dBm/BWChannel |
| [4.5] | [TBD] | -3 dB | FDD-M1_A, TDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [4.5] | [TBD] | -6 dB | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.21.14-2: RSRQ Inter frequency relative accuracy UE category M1 with CE mode A for HD-FDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 5 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 4 | dBm/BWChannel |
| [4.5] | [TBD] | -3 dB | FDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [4.5] | [TBD] | -6 dB | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### 9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B

The requirements for absolute accuracy of RSRQ in this clause apply to a cell that has different carrier frequency from the serving cell.

The accuracy requirements in Tables 9.1.21.15-1 and 9.1.21.15-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.3 for a corresponding Band

Table 9.1.21.15-1: RSRQ Inter frequency absolute accuracy UE category M1 with CE mode B for FDD and TDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| [5] | [TBD] | -12 dB | FDD-M1_A, TDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [5] | [TBD] | -15≤Ês/Iot≤-12 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.21.15-2: RSRQ Inter frequency absolute accuracy UE category M1 with CE mode B for HD-FDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| [5] | [TBD] | -12 dB | FDD-M1_A, TDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [5] | [TBD] | -15≤Ês/Iot≤-12 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### 9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B

The relative accuracy of RSRQ in inter frequency case is defined as the RSRQ measured from one cell compared to the RSRQ measured from another cell on a different frequency.

The accuracy requirements in Tables 9.1.21.16-1 and 9.1.21.16-2 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 Clause 7.3 for reference sensitivity are fulfilled.

RSRP1,2|dBm according to Annex B.3.4 for a corresponding Band

| Channel 1_Io -Channel 2_Io |  20 dB

Table 9.1.21.16-1: RSRQ Inter frequency relative accuracy UE category M1 with CE mode B for FDD and TDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 5 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 4 | dBm/BWChannel |
| [5.5] | [TBD] | -12 dB | FDD-M1_A, TDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [5.5] | [TBD] | -15≤Ês/Iot≤-12 dB | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.21.16-2: RSRQ Inter frequency relative accuracy UE category M1 with CE mode B for HD-FDD

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2 | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 5 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 4 | dBm/BWChannel |
| [5.5] | [TBD] | -12 dB | FDD-M1_A | -121 | -50 |
|  |  |  | FDD-M1_D | -119.5 | -50 |
|  |  |  | FDD-M1_E | -119 | -50 |
|  |  |  | FDD-M1_F | -118.5 | -50 |
|  |  |  | FDD-M1_G | -118 | -50 |
|  |  |  | FDD-M1_N | -114.5 | -50 |
| [5.5] | [TBD] | -15≤Ês/Iot≤-12 dB | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

<END OF CHANGED SECTIONS>