3GPP TSG-RAN WG4 Meeting #83 					R4-1708410

Berlin, Germany, 21 - 25 August, 2017

Source:	Airbus DS SLC

Title:	A-MPR Simulation results for Band 68 – LTE 10MHz and 15MHz

Agenda item:	8.11.2

Document for:	Approval

# Introduction

A draft CR [1] was technically endorsed in RAN4#83. However, there are “TBD” values for some configurations with LTE signals of 10 and 15MHz.

In this contribution, simulations results for the missing configuration are provided.

# Simulation results

## LTE 10 and 15 MHz confined in the lower part of Band 68

In this section, the following cases are studied:

- LTE 10MHz, Fc = 703MHz

- LTE 15MHz, Fc = 705.5MHz

### Filter characteristics

The table below is copied from [2]. Simulation results in the next section are provided for the filter corresponding to Option B/Extreme environmental conditions.

|  | TR Band 68 |  | Option A |  |  |  | Option B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TC SAW [2] 30MHz passband |  | Vendor A TCSAW 30MHz passband |  | Vendor B SAW 30MHz passband |  | Vendor B SAW 15MHz passband |  | Vendor B TCSAW 15MHz passband |  |
| MHz | Extreme | Normal | Extreme | Normal | Extreme | Normal | Extreme | Normal | Extreme | Normal |
| 693.5 | 5.7 | 10.9 | 1.2 | 5.1 | 0.0 | 2.0 | 0.0 | 10.0 | 18.0 | 20.0 |
| 692.5 | 10.9 | 18.4 | 2.4 | 12.9 | 2.0 | 8.0 | 4.0 | 16.0 | 18.0 | 20.0 |
| 691.5 | 18.4 | 26.2 | 3.3 | 27.9 | 5.0 | 15.0 | 10.0 | 20.0 | 22.0 | 24.0 |
| 690.5 | 26.2 | 30.6 | 10.0 | 33.9 | 12.0 | 21.0 | 16.0 | 26.0 | 24.0 | 24.0 |
| 689.5 | 30.6 | 31.7 | 26.0 | 33.9 | 20.0 | 28.0 | 20.0 | 30.0 | 24.0 | 24.0 |
| 688.5 | 31.7 | 32.2 | 31.0 | 37.9 | 26.0 | 28.0 | 26.0 | 30.0 | 24.0 | 24.0 |
| 687.5 | 32.2 | 32.2 | 32.0 | 47.9 | 28.0 | 28.0 | 30.0 | 30.0 | 24.0 | 24.0 |
| 686.5 | 32.2 | 32.2 | 36.0 | 48.9 | 28.0 | 28.0 | 30.0 | 30.0 | 24.0 | 24.0 |

### Simulation results

A-MPR requirements are simulated in order to meet -42dBm/8MHz requirement below 694MHz.

MPR + A-MPR for LTE 10MHz, Fc = 703MHz

MPR + A-MPR for LTE 15MHz, Fc = 705.5MHz

## LTE 15 MHz with Fc ≥ 710.5MHz

### Filter Characteristics

A Band 28A filter is assumed. From a vendor datasheet, it is proposed to use a guaranteed value of 27dB attenuation for the frequencies below 694MHz.

### Simulation results

With such filter, it was found that no A-MPR was necessary to meet -42dBm/8MHz requirement below 694MHz for a 15MHz LTE signal within frequency range [703-718MHz].

# Conclusion

Simulation results for the transmitter configurations that are missing from the draft CR [1] are presented in this document.

It is proposed to take these results into account when deriving the A-MPR requirements to be included in the CR on TS36.101 resulting of the Work Item LTE700_Europe_PPDR.

# References

[1]	R4-1705442, new draft CR Band 68 modification to enable operation in Europe REL-14, Nokia, RAN4#83

[2]	R4-1701021 , Band 68 A-MPR simulation results, Nokia