3GPP TSG-RAN WG4 Meeting #84bis 	R4-1710184

# Dubrovnik, Croatia, 9th – 13th Oct 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 4718 | rev | - | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce burst model for eLAA PUSCH demodulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eLAA-Perf |  |  |  |  |  |  | Date: |  | 2017-09-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 group agreed to have burst model for eLAA PUSCH demodulation, this model is not captured in current specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the burst model for eLAA PUSCH demodulation and make some clarifcation on the terminology |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no burst model for eLAA PUSCH demodulation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.8 A.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------------Beginning of Change ----------------------------------------------------------

### 8.2.8	Requirements for PUSCH of Frame structure type 3

For the parameters specified in Table 8.2.8-1 the throughput shall be equal to or larger than the fraction of maximum throughput stated in the tables 8.2.8-2 at the given SNR when the PUSCH is transmitted with Frame structure type 3.

Table 8.2.8-1: Test Parameters for PUSCH with frame structure type 3

| Parameter | Unit | Value (NOTE 1) |
| --- | --- | --- |
| Maximum number of valid HARQ transmissions (NOTE 2) |  | 4 |
| RV sequences |  | [0 ,2, 0, 2] |
| PUSCH starting position (NOTE 3) |  | ‘01’ |
| PUSCH ending symbol |  | ‘0’ |
| NOTE 1: PUSCH scheduling pattern is changed as per the following bitmap that repeats every 20ms, PUSCH scheduling subframe bitmap = {0000 0a11 1100 000b cccc}. In the subframe indicated by “a” or “1”, PUSCH is scheduled and transmitted, in the subframe indicated by “b” or “c”, PUSCH is scheduled but not transmitted, and the subframe indicated by “0” is reserved for downlink transmission. NOTE 2: The PUSCH which is scheduled and transmitted is counted as valid HARQ transmission and the PUSCH which is scheduled and not transmitted is not counted as valid HARQ transmission NOTE 3: In the subframe indicated by “a” or “b”, PUSCH is scheduled and the associated DCI indicates “PUSCH starting position” as ‘01’, and in the subframe indicated by “1” or “c”, PUSCH is scheduled and the assocated DCI indicates “PUSCH starting positon” as “00”. |  |  |

Table 8.2.8-2: Minimum requirements for PUSCH, 20 MHz Channel Bandwidth, 1Tx

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of norminal maximum throughput (NOTE 1) | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | EPA 5Hz Low | A20-1 | 70% | [-0.2] |
|  |  |  |  | A20-2 | 70% | [12.1] |
|  | 4 | Normal | EPA 5Hz Low | A20-1 | 70% | [-3.2] |
|  |  |  |  | A20-2 | 70% | [8.3] |
| NOTE1: Fraction of nominal maximum throughput is calculated based on the actual transmitted PUSCH norminal |  |  |  |  |  |  |

----------------------------------------------------------unchanged section is skiped----------------------------------------------------

# A.20 Fixed Reference Channels for PUSCH of Frame structure type 3

Table A.20-1: FRC parameters for performance requirements (QPSK 1/3)

| Reference channel | A20-1 |
| --- | --- |
| Uplink resource allocation type | 3 |
| Allocated resource blocks | 50 |
| DFT-OFDM Symbols per subframe | 11 |
| Modulation | QPSK |
| Code rate | 1/3 |
| Payload size (bits) | 4392 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Coded block size including 12bits trellis termination (bits) | 13260 |
| Total number of bits per the sub-frame with the PUSCH starting postions indicated as “01” in the associated DCI | 13200 |
| Total number of bits per the sub-frame with the PUSCH starting postions indicated as “00” in the associated DCI | 14400 |
| Total symbols per the sub-frame with the PUSCH starting positions indicated as “01” in the associated DCI | 6600 |
| Total symbols per the sub-frame with the PUSCH starting positions indicated as “00” in the associated DCI | 7200 |

Table A.20-2 FRC parameters for performance requirements (16QAM 3/4)

| Reference channel | A20-2 |
| --- | --- |
| Uplink resource allocation type | 3 |
| Allocated resource blocks | 50 |
| DFT-OFDM Symbols per subframe | 11 |
| Modulation | 16QAM |
| Code rate | ¾ |
| Payload size (bits) | 19848 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 24 |
| Number of code blocks - C | 4 |
| Coded block size including 12bits trellis termination (bits) | 14988 |
| Total number of bits per the sub-frame with the PUSCH starting positions indicated as “01” in the associated DCI | 26400 |
| Total number of bits per the sub-frame with the PUSCH starting positions indicated as “00” in the associated DCI | 28800 |
| Total symbols per the sub-frame with the PUSCH starting positons indicated as “01” in the associated DCI | 6600 |
| Total symbols per the sub-frame with the PUSCH starting positons indicated as “00” in the associated DCI | 7200 |

--------------------------------------------------------------End of Change ---------------------------------------------------------------