3GPP TSG-RAN WG4 Meeting #84bis 	R4-1710190

# Dubrovnik, Croatia, 9th – 13th Oct 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 4657 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the reference channel for the LAA CSI test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Perf |  |  |  |  |  |  | Date: |  | 2017-09-29 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current reference channel in CSI test is for 10MHz. In LAA, the bandwidth is 20MHz The current reference channel in CSI test is for 1-port CRS, but the test is accurately designed for 2-port CRS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the reference channel bandwidth from 10MHz to 20MHz and refer to 2-port CRS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test can not be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4, 9.2.6, 9.2.6, 9.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------------Beginning of Change ----------------------------------------------------------

### 9.2.6	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)

#### 9.2.6.1	Frame structure type 3 with FDD Pcell

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.6.1-1, Table 9.2.6.1-2, and using the downlink physical channels specified in tables C.3.6-1 and C.3.6-2, two sets of CQI reports are obtained for LAA Scell. The first one is obtained by reports whose reference resource is in the downlink subframes with 6 dB transmission power boost, i.e., high power subframes. The second one is obtained by reports whose reference resource is in the downlink subframe with 0 dB transmission power boost, i.e., low power subframe. In the test, PDSCH transport format in high power subframe is determined by first set of CQI reports and PDSCH transport format in low power subframe is determined by second set of CQI reports.

The reported CQI value in the first set of reports shall be in the range of ±1 of the reported median more than 90% of the first set of reports. The reported CQI value in the second set of reports shall be in the range of ±1 of the reported median more than 90% of the second set of reports.

If the PDSCH BLER in the high power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in high power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the high power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in high power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

If the PDSCH BLER in the low power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in low power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the low power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in low power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

The value of the wideband CQI median for first set of CQI reports minus the wideband CQI median for second set of CQI reports shall be larger than or equal to 2 in Test 1 and Test 2.

Table 9.2.6.1-1: Parmeters for PUSCH 3-0 static test on FDD Pcell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| PDSCH transmission mode |  |  | 3 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |
| SNR |  | dB | 20 |
|  |  | dB[mW/15kHz] | -78 |
|  |  | dB[mW/15kHz] | -98 |
| Max number of HARQ transmissions |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-0 |
| CSI request field |  |  | ‘10’ |
| trigger1 (Note 2) |  |  | 01000000 |
| trigger2 (Note 2) |  |  | 00000000 |
| Note 1: PCell is used for HARQ ACK/NACK feedback and aperiodic CSI triggering/reporting. PDSCH is not transmitted on PCell. Note 2: trigger1 and trigger2 are defined as TS 36.331 for aperiodicCSI-Trigger. They Indicate for which serving cell(s) the aperiodic CSI report is triggered when one or more SCells are configured. PDCCH DCI format 0 with a trigger for aperiodic CQI is transmitted periodically in subframe 1 and subframe 6 with 5ms periodicity. |  |  |  |

Table 9.2.6.1-2: PUSCH 3-0 static test on LAA Scell

| Parameter |  |  | Unit | Test 1 | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 |  |  |
| PDSCH transmission mode |  |  |  | 3 |  |  |
| Downlink power allocation |  |  | dB | -3 |  |  |
|  |  |  | dB | -3 |  |  |
|  |  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (2x2) |  |  |
| SNR in subframes with 6 dB power boost (Note 2) |  |  | dB | 9 |  | 10 |
| SNR in subframes with 0 dB power boost (Note 2) |  |  | dB | 3 |  | 4 |
| in subframes with 6 dB power |  |  | dB[mW/15kHz] | -89 |  | -88 |
| in subframes with 0 dB power |  |  | dB[mW/15kHz] | -95 |  | -94 |
|  |  |  | dB[mW/15kHz] | -98 |  |  |
| MBSFN subframe Configuration |  |  |  | Non-MBSFN |  |  |
| Cell Id |  |  |  | 0 |  |  |
| dmtc-Periodicity |  |  | ms | 80 |  |  |
| dmtc-Offset |  |  |  | 0 |  |  |
| Number of control OFDM symbols |  |  |  | 3 |  |  |
| Max number of HARQ transmissions |  |  |  | 1 |  |  |
| Reporting mode |  |  |  | PUSCH 3-0 |  |  |
| PDSCH transmission model | Basic model |  |  | As specified in Section B.7 |  |  |
|  | subframeStartPosition |  |  | s0 |  |  |
|  | Number of occupied symbols per subframe |  |  | 14 |  |  |
|  | The number of subframes set (S1) per burst |  |  | {3,8} |  |  |
|  | Power configuration for each burst |  |  | is randomly selected from 6 dB power boosting or 0 dB power boosting with equal probability |  |  |
| Note 1: Reference measurement channel RC.2A FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1, except for category 1 UE use RC.4A FDD with two sided dynamic OCNG Pattern OP.2 FDD as described in Annex A.5.1.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

#### 9.2.6.2	Frame structure type 3 with TDD Pcell

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.6.2-1, Table 9.2.6.2-2, and using the downlink physical channels specified in tables C.3.6-1 and C.3.6-2, two sets of CQI reports are obtained for LAA Scell. The first one is obtained by reports whose reference resource is in the downlink subframes with 6 dB transmission power boost, i.e., high power subframes. The second one is obtained by reports whose reference resource is in the downlink subframe with 0 dB transmission power boost, i.e., low power subframe. In the test, PDSCH transport format in high power subframe is determined by first set of CQI reports and PDSCH transport format in low power subframe is determined by second set of CQI reports.

The reported CQI value in the first set of reports shall be in the range of ±1 of the reported median more than 90% of the first set of reports. The reported CQI value in the second set of reports shall be in the range of ±1 of the reported median more than 90% of the second set of reports.

If the PDSCH BLER in the high power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in high power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the high power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in high power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

If the PDSCH BLER in the low power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in low power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the low power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in low power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

The value of the wideband CQI median for first set of CQI reports minus the wideband CQI median for second set of CQI reports shall be larger than or equal to 2 in Test 1 and Test 2.

Table 9.2.6.2-1: Parmeters for PUSCH 3-0 static test on TDD Pcell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| Uplink downlink configuration |  |  | 2 |
| Special subframe configuration |  |  | 4 |
| PDSCH transmission mode |  |  | 3 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |
| SNR |  | dB | 20 |
|  |  | dB[mW/15kHz] | -78 |
|  |  | dB[mW/15kHz] | -98 |
| Max number of HARQ transmissions |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-0 |
| CSI request field |  |  | ‘10’ |
| trigger1 (Note 2) |  |  | 01000000 |
| trigger2 (Note 2) |  |  | 00000000 |
| Note 1: PCell is used for HARQ ACK/NACK feedback and aperiodic CSI triggering/reporting. PDSCH is not transmitted on PCell. Note 2: trigger1 and trigger2 are defined as TS 36.331 for aperiodicCSI-Trigger. They Indicate for which serving cell(s) the aperiodic CSI report is triggered when one or more SCells are configured. PDCCH DCI format 0 with a trigger for aperiodic CQI is transmitted periodically in subframe 3 and subframe 8 with 5ms periodicity. |  |  |  |

Table 9.2.6.2-2: PUSCH 3-0 static test on LAA Scell

| Parameter |  |  | Unit | Test 1 | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 |  |  |
| PDSCH transmission mode |  |  |  | 3 |  |  |
| Downlink power allocation |  |  | dB | -3 |  |  |
|  |  |  | dB | -3 |  |  |
|  |  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (2x2) |  |  |
| SNR in subframes with 6 dB power boost (Note 2) |  |  | dB | 9 |  | 10 |
| SNR in subframes with 0 dB power boost (Note 2) |  |  | dB | 3 |  | 4 |
| in subframes with 6 dB power |  |  | dB[mW/15kHz] | -89 |  | -88 |
| in subframes with 0 dB power |  |  | dB[mW/15kHz] | -95 |  | -94 |
|  |  |  | dB[mW/15kHz] | -98 |  |  |
| MBSFN subframe Configuration |  |  |  | Non-MBSFN |  |  |
| Cell Id |  |  |  | 0 |  |  |
| dmtc-Periodicity |  |  | ms | 80 |  |  |
| dmtc-Offset |  |  |  | 0 |  |  |
| Number of control OFDM symbols |  |  |  | 3 |  |  |
| Max number of HARQ transmissions |  |  |  | 1 |  |  |
| Reporting mode |  |  |  | PUSCH 3-0 |  |  |
| PDSCH transmission model | Basic model |  |  | As specified in Section B.7 |  |  |
|  | subframeStartPosition |  |  | s0 |  |  |
|  | Number of occupied symbols per subframe |  |  | 14 |  |  |
|  | The number of subframes set (S1) per burst |  |  | {3,8} |  |  |
|  | Power configuration for each burst |  |  | is randomly selected from 6 dB power boosting or 0 dB power boosting with equal probability |  |  |
| Note 1: Reference measurement channel RC.2A FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1, except for category 1 UE use RC.4A FDD with two sided dynamic OCNG Pattern OP.2 FDD as described in Annex A.5.1.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

### 9.2.7	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)

#### 9.2.7.1	Frame structure type 3 wth FDD Pcell

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.7.1-1, Table 9.2.7.1-2and using the downlink physical channels specified in tables C.3.6-1 and C.3.6-2, two sets of CQI reports are obtained for LAA Scell, The first one is obtained by reports whose reference resource is in the downlink subframes with 6 dB transmission power boost, i.e., high power subframes. The second one is obtained by reports whose reference resource is in the downlink subframe with 0 dB transmission power boost, i.e., low power subframe. In the test, PDSCH transport format in high power subframe is determined by first set of CQI reports and PDSCH transport format in low power subframe is determined by second set of CQI reports.

The reported CQI value in the first set of reports shall be in the range of ±1 of the reported median more than 90% of the first set of reports. The reported CQI value in the second set of reports shall be in the range of ±1 of the reported median more than 90% of the second set of reports.

If the PDSCH BLER in the high power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in high power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in high power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in high power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

If the PDSCH BLER in the low power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in low power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the low power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in low power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

The value of the wideband CQI for the first set of CQI report minus the wideband CQI median for second set of CQI shall be larger than or equal to 2 in Test 1 and Test 2.

Table 9.2.7.1-1: Parmeters for PUSCH 3-1 static test on FDD Pcell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| PDSCH transmission mode |  |  | 9 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |
| SNR |  | dB | 20 |
|  |  | dB[mW/15kHz] | -78 |
|  |  | dB[mW/15kHz] | -98 |
| Beamforming Model |  |  | As specified in Section B.4.3 |
| CRS reference signals |  |  | Antenna ports 0 |
| CSI reference signals |  |  | Antenna ports 15, 16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| CSI-RS reference signal configuration |  |  | 4 |
| CodeBookSubsetRestriction bitmap |  |  | 000001 |
| Number of control OFDM symbols |  |  | 3 |
| Max number of HARQ transmissions |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| CSI request field |  |  | ‘10’ |
| trigger1 (Note 2) |  |  | 01000000 |
| trigger2 (Note 2) |  |  | 00000000 |
| Note 1: PCell is used for HARQ ACK/NACK feedback and aperiodic CSI triggering/reporting. PDSCH is not transmitted on PCell. Note 2: trigger1 and trigger2 are defined as TS 36.331 for aperiodicCSI-Trigger. They Indicate for which serving cell(s) the aperiodic CSI report is triggered when one or more SCells are configured. PDCCH DCI format 0 with a trigger for aperiodic CQI is transmitted periodically in subframe 1 and subframe 6 with 5ms periodicity. |  |  |  |

Table 9.2.7.1-2: PUSCH 3-1 static test on LAA Scell

| Parameter |  |  | Unit | Test 1 |  | Test 2 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 MHz |  |  |
| Transmission mode |  |  |  | 9 |  |  |
| Downlink power allocation |  |  | dB | 0 |  |  |
|  |  |  | dB | 0 |  |  |
|  |  |  | dB | 0 |  |  |
|  |  |  | dB | 0 |  |  |
| SNR in subframes with 6 dB power boost (Note 3) |  |  | dB | 9 | 10 |  |
| SNR in subframes with 0 dB power boost (Note 3) |  |  |  | 3 | 4 |  |
| in subframes with 6 dB power |  |  |  | -89 | -88 |  |
| in subframes with 0 dB power |  |  | dB[mW/15kHz] | -95 | -94 |  |
|  |  |  | dB[mW/15kHz] | -98 |  | -98 |
| MBSFN subframe Configuration |  |  |  | Non-MBSFN |  |  |
| Cell Id |  |  |  | 0 |  |  |
| dmtc-Periodicity |  |  | ms | 80 |  |  |
| dmtc-Offset |  |  |  | 0 |  |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (2x2) |  |  |
| Beamforming Model |  |  |  | As specified in Section B.4.3 |  |  |
| CRS reference signals |  |  |  | Antenna ports 0 |  |  |
| CSI reference signals |  |  |  | Antenna ports 15, 16 |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  |  | 5/ 1 |  |  |
| CSI-RS reference signal configuration |  |  |  | 4 |  |  |
| CodeBookSubsetRestriction bitmap |  |  |  | 000001 |  |  |
| Number of control OFDM symbols |  |  |  | 3 |  |  |
| Max number of HARQ transmissions |  |  |  | 1 |  |  |
| Reporting mode |  |  |  | PUSCH 3-1 |  |  |
| PDSCH transmission model | Basic model |  |  | As specified in Section B.7 |  |  |
|  | subframeStartPosition |  |  | s0 |  |  |
|  | Number of occupied symbols per subframe |  |  | 14 |  |  |
|  | The number of subframes set (S1) per burst |  |  | {3, 8} |  |  |
|  | Power configuration for each burst |  |  | is randomly selected from 6 dB power boosting or 0 dB power boosting with equal probability |  |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.9A FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

#### 9.2.7.2	Frame structure type 3 wth TDD Pcell

The following requirements apply to UE Category ≥1. For the parameters specified in Table 9.2.7.2-1, Table 9.2.7.2-2and using the downlink physical channels specified in tables C.3.6-1 and C.3.6-2, two sets of CQI reports are obtained for LAA Scell, The first one is obtained by reports whose reference resource is in the downlink subframes with 6 dB transmission power boost, i.e., high power subframes. The second one is obtained by reports whose reference resource is in the downlink subframe with 0 dB transmission power boost, i.e., low power subframe. In the test, PDSCH transport format in high power subframe is determined by first set of CQI reports and PDSCH transport format in low power subframe is determined by second set of CQI reports.

The reported CQI value in the first set of reports shall be in the range of ±1 of the reported median more than 90% of the first set of reports. The reported CQI value in the second set of reports shall be in the range of ±1 of the reported median more than 90% of the second set of reports.

If the PDSCH BLER in the high power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in high power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in high power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in high power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

If the PDSCH BLER in the low power subframes using the transport format indicated by wideband CQI median is less than or equal to 0.1, the BLER in low power subframes using the transport format indicated by the (wideband CQI median + 1) shall be greater than 0.1. If the PDSCH BLER in the low power subframes using the transport format indicated by the wideband CQI median is greater than 0.1, the BLER in low power subframes using transport format indicated by (wideband CQI median – 1) shall be less than or equal to 0.1.

The value of the wideband CQI for the first set of CQI report minus the wideband CQI median for second set of CQI shall be larger than or equal to 2 in Test 1 and Test 2.

Table 9.2.7.2-1: Parmeters for PUSCH 3-1 static test on TDD Pcell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |
| PDSCH transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 2 |
| Special subframe configuration |  |  | 4 |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
|  |  | dB | 0 |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |
| SNR |  | dB | 20 |
|  |  | dB[mW/15kHz] | -78 |
|  |  | dB[mW/15kHz] | -98 |
| Beamforming Model |  |  | As specified in Section B.4.3 |
| CRS reference signals |  |  | Antenna ports 0 |
| CSI reference signals |  |  | Antenna ports 15, 16 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |
| CSI-RS reference signal configuration |  |  | 4 |
| CodeBookSubsetRestriction bitmap |  |  | 000001 |
| Number of control OFDM symbols |  |  | 3 |
| Max number of HARQ transmissions |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| CSI request field |  |  | ‘10’ |
| trigger1 (Note 2) |  |  | 01000000 |
| trigger2 (Note 2) |  |  | 00000000 |
| Note 1: PCell is used for HARQ ACK/NACK feedback and aperiodic CSI triggering/reporting. PDSCH is not transmitted on PCell. Note 2: trigger1 and trigger2 are defined as TS 36.331 for aperiodicCSI-Trigger. They Indicate for which serving cell(s) the aperiodic CSI report is triggered when one or more SCells are configured. PDCCH DCI format 0 with a trigger for aperiodic CQI is transmitted periodically in subframe 3 and subframe 8 with 5ms periodicity. |  |  |  |

Table 9.2.7.2-2: PUSCH 3-1 static test on LAA Scell

| Parameter |  |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 MHz |  |
| Transmission mode |  |  |  | 9 |  |
| Downlink power allocation |  |  | dB | 0 |  |
|  |  |  | dB | 0 |  |
|  |  |  | dB | 0 |  |
|  |  |  | dB | 0 |  |
| SNR in subframes with 6 dB power boost (Note 3) |  |  | dB | 9 | 10 |
| SNR in subframes with 0 dB power boost (Note 3) |  |  |  | 3 | 4 |
| in subframes with 6 dB power |  |  |  | -89 | -88 |
| in subframes with 0 dB power |  |  | dB[mW/15kHz] | -95 | -94 |
|  |  |  | dB[mW/15kHz] | -98 | -98 |
| MBSFN subframe Configuration |  |  |  | Non-MBSFN |  |
| Cell Id |  |  |  | 0 |  |
| dmtc-Periodicity |  |  | ms | 80 |  |
| dmtc-Offset |  |  |  | 0 |  |
| Propagation condition and antenna configuration |  |  |  | Clause B.1 (2x2) |  |
| Beamforming Model |  |  |  | As specified in Section B.4.3 |  |
| CRS reference signals |  |  |  | Antenna ports 0 |  |
| CSI reference signals |  |  |  | Antenna ports 15, 16 |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  |  | 5/ 3 |  |
| CSI-RS reference signal configuration |  |  |  | 4 |  |
| CodeBookSubsetRestriction bitmap |  |  |  | 000001 |  |
| Number of control OFDM symbols |  |  |  | 3 |  |
| Max number of HARQ transmissions |  |  |  | 1 |  |
| Reporting mode |  |  |  | PUSCH 3-1 |  |
| PDSCH transmission model | Basic model |  |  | As specified in Section B.7 |  |
|  | subframeStartPosition |  |  | s0 |  |
|  | Number of occupied symbols per subframe |  |  | 14 |  |
|  | The number of subframes set (S1) per burst |  |  | {3, 8} |  |
|  | Power configuration for each burst |  |  | is randomly selected from 6 dB power boosting or 0 dB power boosting with equal probability |  |
| Note 1: If the UE reports in an available uplink reporting instance at subframe SF#n based on CQI estimation at a downlink subframe not later than SF#(n-4), this reported subband or wideband CQI cannot be applied at the eNB downlink before SF#(n+4) Note 2: Reference measurement channel RC.9A FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.1/2 FDD as described in Annex A.5.1.1/2. Note 3: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |

-----------------------------------------------------------unchanged section is skiped----------------------------------------------------

# A.4 CSI reference measurement channels

This section defines the DL signal applicable to the reporting of channel status information (Clause 9.2, 9.3 and 9.5).

In Table A.4-1 are specified the reference channels. Table A.4-13 specifies the mapping of CQI index to modulation coding scheme, which complies with the CQI definition specified in Section 7.2.3 of [6].

Table A.4-0: Void

Table A.4-1: CSI reference measurement channels

| RMC Name | Duplex | CH-BW | Alloc. RB-s | UL/DL Config | Alloc. SF-s | MCS Scheme | Nr. HARQ Proc. | Max. nr HARQ Trans. | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 CRS Port |  |  |  |  |  |  |  |  |  |
| RC.1 FDD | FDD | 10 | 50 | - |  | MCS.1 | 8 | 1 |  |
| RC.1A FDD | FDD | 10 | 50 |  |  | MCS.1A | 8 | 1 |  |
| RC.1 TDD | TDD | 10 | 50 | Note 3 |  | MCS.1 | 10 | 1 |  |
| RC.1A TDD | TDD | 20 | 100 | Note 3 |  | MCS.1B | 10 | 1 |  |
| RC.3 FDD | FDD | 10 | 6 | - |  | MCS.10 | 8 | 1 |  |
| RC.3 TDD | TDD | 10 | 6 | Note 3 |  | MCS.10 | 10 or 7 (Note 9) | 1 |  |
| RC.4 FDD | FDD | 10 | 15 | - |  | MCS.15 | 8 | 1 | Note 6 |
| RC.4 TDD | TDD | 10 | 15 | Note 3 |  | MCS.15 | 10 | 1 | Note 6 |
| RC.5 FDD | FDD | 10 | 3 | - |  | MCS.17 | 8 | 1 |  |
| RC.5 TDD | TDD | 10 | 3 | Note 3 |  | MCS.17 | 10 | 1 |  |
| RC.14 FDD | FDD | 5 | 25 | - |  | MCS.14 | 8 | 1 |  |
| RC.15 FDD | FDD | 5 | 15 | - |  | MCS.15 | 8 | 1 | Note 6 |
| RC.16 FDD | FDD/HD-FDD | 10 | 2 |  |  | MCS.20 | 8 | 1 | Note 8,10 |
| RC.16 TDD | TDD | 10 | 2 | Note 3 |  | MCS.20 | 10 | 1 | Note 8 |
| RC.23FDD | FDD/HD-FDD | 10 | 3 |  |  | MCS.28 | 8 | 1 | Note 12, 13 |
| RC.23 TDD | TDD | 10 | 3 |  |  | MCS.28 | 10 | 1 | Note 12 |
| RC.25 FDD | FDD/HD-FDD | 10 | 3 |  |  | MCS.28 | 8 | 1 | Note 12, 14 |
| RC.25 TDD | TDD | 10 | 3 |  |  | MCS.28 | 10 | 1 | Note 12, 15 |
| 2 CRS Ports |  |  |  |  |  |  |  |  |  |
| RC.2 FDD | FDD | 10 | 50 | - |  | MCS.2 | 8 | 1 |  |
| RC.2A FDD | FDD | 20 | 100 |  |  | MCS.2A | 8 | 1 |  |
| RC.2 TDD | TDD | 10 | 50 | Note 3 |  | MCS.2 | 10 or 7 (Note 9) | 1 |  |
| RC.4A FDD | FDD | 20 | 15 | - |  | MCS.16 | 8 | 1 | Note 6 |
| RC.6 FDD | FDD | 10 | 15 | - |  | MCS.16 | 8 | 1 | Note 6 |
| RC.6 TDD | TDD | 10 | 15 | Note 3 |  | MCS.16 | 7 | 1 | Note 6 |
| 4 CRS Ports |  |  |  |  |  |  |  |  |  |
| RC.17 FDD | FDD | 10 | 50 | - |  | MCS.18 | 8 | 1 |  |
| RC.17 TDD | TDD | 10 | 50 | Note 3 |  | MCS.18 | 7 | 1 |  |
| RC.21 FDD | FDD | 10 | 50 | - |  | MCS.26 | 8 | 1 |  |
| RC.21 TDD | TDD | 10 | 50 | Note 3 |  | MCS.26 | 7 | 1 |  |
| 1 CRS Port + CSI-RS |  |  |  |  |  |  |  |  |  |
| RC.8 FDD | FDD | 10 | 6 | - | Non CSI-RS | MCS.11 | 8 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.12 |  |  |  |
| RC.8A FDD | FDD | 10 | 6 | - | Non CSI-RS | MCS.11A | 8 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.12A |  |  |  |
| RC.8 TDD | TDD | 10 | 6 | Note 3 | Non CSI-RS | MCS.11 | 10 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.12 |  |  |  |
| RC.8A TDD | TDD | 20 | 8 | Note 3 | Non CSI-RS | MCS.11B | 10 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.12B |  |  |  |
| RC.9 FDD | FDD | 10 | 50 | - | Non CSI-RS | MCS.3 | 8 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.4 |  |  |  |
| RC.9A FDD | FDD | 20 | 100 | - | Non CSI-RS | MCS.3A | 8 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.4A |  |  |  |
| RC.9 TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS | MCS.3 | 7 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.4 |  |  |  |
| 2 CRS Port + CSI-RS |  |  |  |  |  |  |  |  |  |
| RC.7 FDD | FDD | 10 | 50 | - | Non CSI-RS | MCS.5 | 8 | 1 |  |
|  |  |  |  |  | 4 CSI-RS | MCS.7 |  |  |  |
| RC.7 TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS | MCS.5 | 10 | 1 |  |
|  |  |  |  |  | 8 CSI-RS | MCS.8 |  |  |  |
| RC.11 FDD | FDD | 10 | 50 | - | Non CSI-RS | MCS.5 | 8 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.6 |  |  |  |
| RC.11 TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS | MCS.5 | 10 | 1 |  |
|  |  |  |  |  | 2 CSI-RS | MCS.6 |  |  |  |
| RC.18 FDD | FDD | 10 | 6 | - | Non CSI-RS | MCS.13 | 8 | 1 |  |
|  |  |  |  |  | 4 CSI-RS | MCS.19 |  |  |  |
| RC.18 TDD | TDD | 10 | 6 | Note 3 | Non CSI-RS | MCS.13 | 7 | 1 |  |
|  |  |  |  |  | 4 CSI-RS | MCS.19 |  |  |  |
| RC.17 TDD | TDD | 10 | 6 | Note 3 | 4 ZP-CSI-RS | MCS.21 | 10 | 1 |  |
| RC.18 TDD | TDD | 10 | 6 | Note 3 | 4 ZP-CSI-RS | MCS.22 | 10 | 1 |  |
| RC.19 TDD | TDD | 10 | 41 | Note3 | 4 ZP-CSI-RS | MCS.23 | 10 | 1 | Note 11 |
| RC.20 TDD | TDD | 10 | 50 | Note3 | Non CSI-RS | MCS.24 | 10 | 1 |  |
|  |  |  |  |  | 2 CSI-RS, 4 ZP-CSI-RS | MCS.25 |  |  |  |
| RC.22 FDD | FDD | 10 | 50 | - | Non CSI-RS | MCS.5 | 8 | 1 |  |
|  |  |  |  |  | 4 CSI-RS | MCS.27 |  |  |  |
| RC.22 TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS | MCS.5 | 10 | 1 |  |
|  |  |  |  |  | 4 CSI-RS | MCS.27 |  |  |  |
| 1 CRS Port + CSI-RS + CSI-IM |  |  |  |  |  |  |  |  |  |
| RC.13 FDD | FDD | 10 | 50 | - | Non CSI-RS/IM | MCS.3 | 8 | 1 |  |
|  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |
| RC.13 TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS/IM | MCS.3 | 10 | 1 |  |
|  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |
| 2 CRS Port + CSI-RS + CSI-IM |  |  |  |  |  |  |  |  |  |
| RC.10 FDD | FDD | 10 | 50 | - | Non CSI-RS | MCS.5 | 8 | 1 |  |
|  |  |  |  |  | 4 CSI-RS, 1 CSI process | MCS.8 |  |  |  |
| RC.10 TDD | TDD | 10 | 50 | Note 3 | Non CSI-RS | MCS.5 | 10 | 1 |  |
|  |  |  |  |  | 8 CSI-RS, 1 CSI process | MCS.9 |  |  |  |
| RC.12 FDD | FDD | 10 | 6 | - | Non CSI-RS/IM | MCS.13 | 8 | 1 |  |
|  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |
| RC.12 TDD | TDD | 10 | 6 | Note 3 | Non CSI-RS/IM | MCS.13 | 10 | 1 |  |
|  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |
| Note 1: 3 symbols allocated to PDCCH. Note 2: For FDD only subframes 1, 2, 3, 4, 6, 7, 8 and 9 are allocated to avoid PBCH and synchronization signal overhead. Note 3: TDD UL-DL configuration as specified in the individual tests. Note 4: For TDD when UL-DL configuration 1 is used only subframes 4 and 9 are allocated to avoide PBCH and synchronizaiton signal overhead. Note 5: For TDD when UL-DL configuration 2 is used only subframes 3, 4, 8, and 9 are allocated to avoid PBCH and synchronization signal overhead. Note 6: Centered within the Transmission Bandwidth Configuration (Figure 5.6-1). Note 7: Only subframes 2, 3, 4, 7, 8 and 9 are allocated to avoid PBCH and synchronization signal overhead. Note 8: Allocate PDSCH on 5th and 6th PRBs within a subband. Note 9: The number of HARQ processes is 10 for TDD UL/DL configuration 2 and 7 for TDD UL/DL configuration 1. Note 10: The downlink subframes are scheduled at the 1st, 2nd, 8th, 9th, 16th, 17th, 18th, 24th, 26th, 32nd, 33rd, 34th subframes every 40ms. Information bit payload is available if downlink subframe is scheduled.(starting from 0th subframe) Note 11: 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in subframe 0 and 5 in RC.19 TDD. Note 12: Allocate PDSCH on 3th, 4th and 5th PRBs within a narrowband. Allocate MPDCCH on the 0th and 1st PRBs within a narrowband. Note 13: The downlink subframes are scheduled at the 0th and 1st subframes every 10ms. Information bit payload is available if downlink subframe is scheduled (starting from 0th subframe). Note 14: The downlink subframes are scheduled at the 0th to 4th subframes every 10ms. Information bit payload is scheduled at the 4th subframe (starting from 0th subframe). Note 15: Information bit payload is scheduled at the 8th subframe every 20ms (starting from 0th subframe). |  |  |  |  |  |  |  |  |  |

Table A.4-1a: Void

Table A.4-1b: Void

Table A.4-1c: Void

Table A.4-1d: Void

Table A.4-1e: Void

Table A.4-2: Void

Table A.4-2a: Void

Table A.4-2b: Void

Table A.4-2c: Void

Table A.4-2d: Void

Table A.4-2e: Void

Table A.4-3: Void

Table A.4-3a: Void

Table A.4-3b: Void

Table A.4-3c: Void

Table A.4-3d: Void

Table A.4-3e: Void

Table A.4-3f: Void

Table A.4-3g: Void

Table A.4-3h: Void

Table A.4-3i: Void

Table A.4-3j: Void

Table A.4-3k: Void

Table A.4-3l: Void

Table A.4-3m: Void

Table A.4-4: Void

Table A.4-4a: Void

Table A.4-4b: Void

Table A.4-5: Void

Table A.4-5a: Void

Table A.4-5b: Void

Table A.4-6: Void

Table A.4-6a: Void

Table A.4-6b: Void

Table A.4-6c: Void

Table A.4-6d: Void

Table A.4-6e: Void

Table A.4-6f: Void

Table A.4-7: Void

Table A.4-8: Void

Table A.4-9: Void

Table A.4-10: Void

Table A.4-11: Void

Table A.4-12: Void

Table A.4-13: Mapping of CQI Index to Modulation coding scheme (MCS)

| CQI Index |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Coding Rate |  |  |  | OOR | 0.0762 | 0.1172 | 0.1885 | 0.3008 | 0.4385 | 0.5879 | 0.3691 | 0.4785 | 0.6016 | 0.4551 | 0.5537 | 0.6504 | 0.7539 | 0.8525 | 0.9258 |  |
| Modulation |  |  |  | OOR | QPSK |  |  |  |  |  | 16QAM |  |  | 64QAM |  |  |  |  |  |  |
| MCS Scheme |  | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.1 |  | 50 | 6300 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 18 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.2 |  | 50 | 6000 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 |  |
| MCS.2A |  | 100 | 12000 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 |  |
| MCS.3 |  | 50 | 5700 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 10 | 13 | 15 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.3A |  | 100 | 11400 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.4 |  | 50 | 5600 | DTX | 0 | 0 | 2 | 4 | 6 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.4A |  | 100 | 11200 | DTX | 0 | 0 | 2 | 4 | 6 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.5 |  | 50 | 5400 | DTX | 0 | 0 | 2 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 23 | 24 | 25 |  |
| MCS.6 |  | 50 | 5300 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 22 | 24 | 25 |  |
| MCS.7 |  | 50 | 5200 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 18 | 20 | 22 | 24 | 25 |  |
| MCS.8 |  | 50 | 5000 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 13 | 17 | 18 | 20 | 22 | 23 | 24 |  |
| MCS.9 |  | 50 | 4800 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 13 | 17 | 18 | 20 | 22 | 23 | 24 |  |
| MCS.10 |  | 6 | 756 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 19 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.11 |  | 6 | 684 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 14 | 17 | 20 | 21 | 23 | 25 | 27 |  |
| MCS.12 |  | 6 | 672 | DTX | 0 | 0 | 1 | 4 | 6 | 8 | 10 | 12 | 14 | 17 | 19 | 21 | 23 | 25 | 26 |  |
| MCS.13 |  | 6 | 648 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 19 | 21 | 22 | 24 | 25 |  |
| MCS.14 |  | 25 | 3150 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 18 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.15 |  | 15 | 1890 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 18 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.16 |  | 15 | 1800 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 |  |
| MCS.17 |  | 3 | 378 | DTX | 0 | 1 | 2 | 5 | 7 | 9 | 12 | 13 | 16 | 19 | 21 | 23 | 25 | 27 | 27 |  |
| MCS.18 |  | 50 | 5800 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 17 | 20 | 22 | 23 | 26 | 27 |  |
| MCS.19 |  | 6 | 624 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 17 | 18 | 20 | 22 | 24 | 25 |  |
| MCS.20 |  | 2 | 252 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 16 | 19 | 21 | 23 | 23 | 23 | 23 |  |
| MCS.21 |  | 6 | 696 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 21 | 24 | 25 | 27 |  |
| MCS.22 |  | 6 | 624 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 15 | 19 | 20 | 22 | 24 | 24 |  |
| MCS.23 |  | 41 | 4264 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 10 | 12 | 14 | 15 | 18 | 20 | 22 | 24 | 24 |  |
| MCS.24 |  | 50 | 5400 | DTX | 0 | 0 | 2 | 3 | 5 | 7 | 10 | 12 | 14 | 15 | 19 | 21 | 23 | 24 | 25 |  |
| MCS.25 |  | 50 | 5100 | DTX | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 12 | 13 | 15 | 18 | 20 | 22 | 23 | 24 |  |
| MCS 26 |  | 50 | 5800 | DTX | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | 18 | 20 | 22 | 24 | 26 | 27 |  |
| MCS.27 | CW0 | 50 | 4600 | DTX | 0 | 0 | 1 | 3 | 5 | 6 | 10 | 11 | 13 | 17 | 18 | 19 | 21 | 23 | 23 |  |
|  | CW1 | 50 | 4600 | DTX | 0 | 0 | 1 | 3 | 5 | 6 | 10 | 11 | 13 | 17 | 18 | 19 | 21 | 22 | 23 |  |
| Note 1: Mapping between Imcs and TBS according to Tables 7.1.7.1-1 and 7.1.7.2.1-1 in TS 36.213 [6]. Note 2: 3 symbols allocated to PDCCH. Note 3: Sub-frame#0 and #5 are not used for the corresponding requirement except for [MCS.23]. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-14: Mapping of CQI Index to Modulation coding scheme (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Spectral Efficiency |  |  | OOR | 0.1523 | 0.3770 | 0.8770 | 1.4766 | 1.9141 | 2.4063 | 2.7305 | 3.3223 | 3.9023 | 4.5234 | 5.1152 | 5.5547 | 6.2266 | 6.9141 | 7.4063 |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.1A | 50 | 6300 | DTX | 0 | 1 | 3 | 5 | 7 | 10 | 11 | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 26 |  |
| MCS.1B | 100 | 12600 | DTX | 0 | 1 | 3 | 5 | 7 | 10 | 11 | 14 | 15 | 18 | 20 | 22 | 24 | 26 | 26 |  |
| Note 1: Mapping between Imcs and CQI Index according to Tables 7.1.7.1-1A, 7.1.7.2.1-1 and 7.2.3-2 in TS 36.213 [6]. Note 2: 3 symbols allocated to PDCCH. Note 3: Sub-frame#0 and #5 are not used for the corresponding requirement. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-15: Mapping of CQI Index to Modulation coding scheme (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Spectral Efficiency |  |  | OOR | 0.1523 | 0.3770 | 0.8770 | 1.4766 | 1.9141 | 2.4063 | 2.7305 | 3.3223 | 3.9023 | 4.5234 | 5.1152 | 5.5547 | 6.2266 | 6.9141 | 7.4063 |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.11A | 6 | 684 | DTX | 0 | 1 | 3 | 5 | 7 | 8 | 10 | 13 | 14 | 16 | 18 | 20 | 22 | 24 | 25 |  |
| MCS.12A | 6 | 672 | DTX | 0 | 1 | 3 | 5 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 25 |  |
| MCS.11B | 8 | 912 | DTX | 0 | 1 | 3 | 5 | 7 | 9 | 10 | 13 | 14 | 16 | 18 | 19 | 22 | 24 | 26 |  |
| MCS.12B | 8 | 896 | DTX | 0 | 1 | 3 | 5 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 19 | 22 | 24 | 25 |  |
| Note 1: Mapping between Imcs and CQI Index according to Tables 7.1.7.1-1A, 7.1.7.2.1-1 and 7.2.3-2 in TS 36.213 [6]. Note 2: 3 symbols allocated to PDCCH. Note 3: Sub-frame#0 and #5 are not used for the corresponding requirement. The next subframe (i.e. sub-frame#1 or #6) shall be used for potential retransmissions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-16: Mapping of CQI Index to Modulation coding scheme (Modulation and TBS indx Table 3)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Coding Rate |  |  | OOR | 0.0391 | 0.0762 | 0.1172 | 0.1885 | 0.3008 | 0.4385 | 0.5879 | 0.3691 | 0.4785 | 0.6015 | Reserved | Reserved | Reserved | Reserved | Reserved |  |
| Modulation |  |  | OOR | QPSK |  |  |  |  |  |  | 16QAM |  |  |  |  |  |  |  |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.28 | 3 | 378 | DTX | 0 | 0 | 0 | 2 | 4 | 6 | 8 | 11 | 13 | 15 | N/A | N/A | N/A | N/A | N/A |  |
| Note 1: Mapping between Imcs and TBS according to Tables 7.1.7.1-1 and 7.1.7.2.1-1 in TS 36.213 [6]. Note 2: startSymbolBR = 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------------End of Change ----------------------------------------------------------