3GPP TSG-RAN WG4 Meeting #84bis                                        R4-1710237

Dubrovnik, Croatia, 9 -13 October, 2017

Source: 	CMCC

Title: 	SUL band combinations

Agenda Item:	9.2.2.9

Document for:	Approval

Introduction

WF on SUL band definition was agreed in [1]. And in RAN4#84 meeting, the proposed notation of SUL and NR-LTE coexistence was agreed in [2]. This contribution provides consideration on how to capture the SUL related band combinations into the TR.

Discussion

So far, several SUL bands are captured in the NR UE TR. The SUL bands are cited in Table 1 below.

Table 1: New NR SUL bands in FR1

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n80 | 1710 – 1785 MHz | N/A | SUL |
| n81 | 880 – 915 MHz | N/A | SUL |
| n82 | 832 – 862 MHz | N/A | SUL |
| n83 | 703 – 748 MHz | N/A | SUL |
| n84 | 1920 – 1980 MHz | N/A | SUL |

On top of these SUL bands, we are interested in SUL band combinations as below:

SUL band combinations:

- SUL Band n80 and Band n78

- SUL band n81 and Band n78

LTE NR DC band combinations with SUL bands:

- LTE Band 3 + SUL Band n80 and Band n78

- LTE Band 8 + SUL band n81 and Band n78

Currently, though there is an agenda for SUL and LTE-NR co-existence band combinations, there is no appropriate TR to capture the technical analysis of SUL and LTE-NR co-existence band combinations. Therefore, we propose to create a separate TR to capture the SUL related content.

Proposal 1: It is proposed to create a separate TR to capture the study outcome of SUL and LTE-NR co-existence band combinations

The notation for SUL and NR-LTE coexistence can be found in [2]:

Table 2 notation for SUL and NR-LTE coexistence

| Representation | Corresponding functionality |
| --- | --- |
| SUL_n78-n81 | Band combination of NR band n78 and band n81(SUL) for NR operation. |
| DC_3-SUL_n78-n80 | LTE-NR DC between LTE Band 3, and NR bands n80 (SUL) and n78 including NR-LTE coexistence with UL sharing. |

With these notations, we should capture the SUL band combinations which we are interested in the corresponding TR.

Table 3 SUL and NR-LTE coexistence combinations

| SUL combinations | Note |
| --- | --- |
| SUL_n78-n80 | Band combination of NR band n78 and band n80 (SUL) for NR operation. |
| SUL_n78-n81 | Band combination of NR band n78 and band n81 (SUL) for NR operation. |
| DC_3-SUL_n78-n80 | LTE-NR DC between LTE Band 3, and NR bands n80 (SUL) and n78 including NR-LTE coexistence with UL sharing. |
| DC_8-SUL_n78-n81 | LTE-NR DC between LTE Band 8, and NR bands n81 (SUL) and n78 including NR-LTE coexistence with UL sharing. |

Proposal 2: It is proposed capture the SUL band combinations in Table 3 to the corresponding TR.

Conclusion

Consideration on capturing the SUL band combinations in TR is provided in this contribution.

Proposal 1: It is proposed to create a separate TR to capture the study outcome of SUL and LTE-NR co-existence band combinations

Proposal 2: It is proposed capture the SUL band combinations in Table below to the corresponding TR.

| SUL combinations | Note |
| --- | --- |
| SUL_n78-n80 | Band combination of NR band n78 and band n80 (SUL) for NR operation. |
| SUL_n78-n81 | Band combination of NR band n78 and band n81 (SUL) for NR operation. |
| DC_3-SUL_n78-n80 | LTE-NR DC between LTE Band 3, and NR bands n80 (SUL) and n78 including NR-LTE coexistence with UL sharing. |
| DC_8-SUL_n78-n81 | LTE-NR DC between LTE Band 8, and NR bands n81 (SUL) and n78 including NR-LTE coexistence with UL sharing. |

- References

- R4-1706966, CMCC, “WF on SUL band definition”, Jun. 2017

- R4-1709181, NTT DOCOMO, “WF on band numbering”, Aug. 2017