3GPP TSG-RAN WG4 Meeting #84bis	R4-1710405

Dubrovnik, Croatia, 09 - 13 October 2017

Agenda item:	6.2.4.2

Title: 	Simulation results for MPDCCH performance for non BL/CE UE

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4# 83 it was agreed that new requirements shall be defined for UE supporting CE with 2Rx and 4Rx antennas. In RAN4#84 companies provided ideal simulation results for performance alignment to define requirements for the following tests for 2Rx and 4Rx antennas.

| Test cases | Reference in TS36.101 |
| --- | --- |
| MPDCCH CE Mode A | 8.11.2.1.1 / 8.11.2.2.1 |
| MPDCCH CE Mode B | 8.11.2.1.2 / 8.11.2.2.2 |
| PDSCH TM9 | 8.11.1.1.2 / 8.11.1.2.2 |
| PDSCH TM2 | 8.11.1.1.3 / 8.11.1.2.3 |

During the RAN4#84 meeting the simulation results from different companies were discussed and the repetition levels for 2Rx and 4Rx tests were agreed [2] as summarized below:

Repetition levels for MPDCCH tests:

- FDD

  - 2Rx

    - Mode A: Repetition number=8

    - Mode B: Repetition number=32

  - 4Rx

    - Mode A: Repetition number=2

    - Mode B: Repetition number=8

- TDD

  - 2Rx

    - Mode A: Repetition number=4

    - Mode B: Repetition number=16

  - 4Rx

    - Mode A: Repetition number=2

    - Mode B: Repetition number=4

Repetition levels for PDSCH tests:

- TM2 Mode B (FDD/TDD)

  - 2Rx

    - Repetition number=32

  - 4Rx

    - Repetition number=16

- TM9 Mode A (FDD/TDD)

  - 2Rx

    - Repetition number=4

  - 4Rx

    - Repetition number=2

With the introduction of smaller repetition levels for 2Rx and 4Rx tests, the frequency hopping interval also should to be reduced accordingly compared to 1 Rx tests. In this contribution we present the simulation results for MPDCCH performance with 2Rx and 4Rx with the repetition levels and frequency hopping intervals agreed upon in [2] and [4].

# Simulation Results

The agreed simulation assumptions for MPDCCH tests from are also listed in Appendix A.

  - CE Mode A

The performance results for MPDCCH in CE Mode A in FDD and TDD modes are shown in Figure 1 below.

Figure 1: MPDCCH Performance for CE Mode A

Based on the ideal simulation results we have the following observations for CE Mode A for Test2 and Test3 in FDD and TDD Modes:

Observation #1: For CE Mode A Test2 in FDD, the SNR corresponding to 1% missed downlink scheduling grant is at -8.7 dB

Observation #2: For CE Mode A Test3 in FDD, the SNR corresponding to 1% missed downlink scheduling grant is at -8.8 dB

Observation #3: For CE Mode A Test2 in TDD, the SNR corresponding to 1% missed downlink scheduling grant is at -7.8 dB

Observation #4: For CE Mode A Test3 in TDD, the SNR corresponding to 1% missed downlink scheduling grant is at -9.4 dB

The SNR points for the 1% missed downlink scheduling grant with no impairments for CE Mode A are summarized in Table 1.

Table 1: Simulation Results for MPDCCH - CE Mode A

| Duplex Type | BW | Aggregation level | Repetition Level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| FDD | 10 MHz | 16 ECCE | 8 | R.81 FDD | EPA5 | 2 x 2 Low | 1 | -8.7 |
|  |  |  | 2 | R.81 FDD | EPA5 | 2 x 4 Low | 1 | -8.8 |
| TDD | 10 MHZ | 16 ECCE | 4 | R.81 TDD | EPA5 | 2 x 2 Low | 1 | -7.8 |
|  |  |  | 2 | R.81 TDD | EPA5 | 2 x 4 Low | 1 | -9.4 |

  - CE Mode B

In RAN4 #84 the frequency hopping interval for CE Mode B tests were discussed and it was agreed that the 2Rx and 4Rx test cases will have lower frequency hopping interval than 1Rx test due to reduced repetition levels for MPDCCH compared to 1Rx. However, based on the simulation assumption of Number of frequency hopping narrowbands=4 for these tests, the agreed frequency hopping interval in [4] would only transmit on 2 of the 4 narrowbands during the repetition period. In order to transmit on all narrowbands, the frequency hopping interval could be further reduced. By setting the frequency hopping intervals to NumRep/4 all narrowbands will be used during the repetition period. For CE Mode B the MPDCCH performance is presented for the frequency hopping intervals agreed upon in [4] and also a lower interval in Figure 2.

Figure 2: MPDCCH Performance for CE Mode B

Based on the ideal simulation results we have the following observations for CE Mode B for Test2 and Test3 in FDD and TDD Modes:

Observation #5: For CE Mode B Test2 in FDD, the SNR corresponding to 1% missed downlink scheduling grant is at -16.8 dB with frequency hopping interval of 16

Observation #6: For CE Mode B Test2 in FDD, the SNR corresponding to 1% missed downlink scheduling grant is at -17.1 dB with frequency hopping interval of 8

Observation #7: For CE Mode B Test3 in FDD, the SNR corresponding to 1% missed downlink scheduling grant is at -15.8 dB with frequency hopping interval of 4

Observation #8: For CE Mode B Test3 in FDD, the SNR corresponding to 1% missed downlink scheduling grant is at -16.1 dB with frequency hopping interval of 2

Observation #9: For CE Mode B Test2 in TDD, the SNR corresponding to 1% missed downlink scheduling grant is at -14.7 dB with frequency hopping interval of 10

Observation #10: For CE Mode B Test2 in TDD, the SNR corresponding to 1% missed downlink scheduling grant is at -15.3 dB with frequency hopping interval of 5

Observation #11: For CE Mode B Test3 in TDD, the SNR corresponding to 1% missed downlink scheduling grant is at -16.1 dB with frequency hopping interval of 5

Based on the simulation results, for FDD mode there is not significant performance difference between using 2 narrowbands vs 4 narrowbands, and we could use the frequency hopping interval agreed upon in [4]. For TDD we could use lower frequency hopping interval.

Proposal #1: For CE Mode B Test2 in FDD set the frequency hopping interval to 16

Proposal #2: For CE Mode B Test3 in FDD set the frequency hopping interval to 4

Proposal #3: For CE Mode B Test2 in TDD set the frequency hopping interval to 5

The SNR points for the 1% missed downlink scheduling grant with no impairments for CE Mode B are summarized in Table 2.

Table 2: Simulation Results for MPDCCH - CE Mode B

| Duplex Type | BW | Aggregation level | Repetition Level | Freq Hop Interval | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| FDD | 10 MHz | 24 ECCE | 32 | 16 | R.81 FDD | ETU1 | 2 x 2 Low | 1 | -16.8 |
|  |  |  | 32 | 8 | R.81 FDD | ETU1 | 2 x 2 Low | 1 | -17.1 |
|  |  |  | 8 | 4 | R.81 FDD | ETU1 | 2 x 4 Low | 1 | -15.8 |
|  |  |  | 8 | 2 | R.81 FDD | ETU1 | 2 x 4 Low | 1 | -16.1 |
| TDD | 10 MHZ | 24 ECCE | 16 | 10 | R.81 TDD | ETU1 | 2 x 2 Low | 1 | -14.7 |
|  |  |  | 16 | 5 | R.81 TDD | ETU1 | 2 x 2 Low | 1 | -15.3 |
|  |  |  | 8 | 5 | R.81 TDD | ETU1 | 2 x 4 Low | 1 | -16.1 |

# Conclusions

In this contribution we have presented the ideal simulation results for MPDCCH for performance alignment for non-BL/CE UE requirements. Our observations and proposals are summarized below:

Observation #1: For CE Mode A Test2 in FDD, the SNR corresponding to 1% missed downlink scheduling grant is at -8.7 dB

Observation #2: For CE Mode A Test3 in FDD, the SNR corresponding to 1% missed downlink scheduling grant is at -8.8 dB

Observation #3: For CE Mode A Test2 in TDD, the SNR corresponding to 1% missed downlink scheduling grant is at -7.8 dB

Observation #4: For CE Mode A Test3 in TDD, the SNR corresponding to 1% missed downlink scheduling grant is at -9.4 dB

Observation #5: For CE Mode B Test2 in FDD, the SNR corresponding to 1% missed downlink scheduling grant is at -16.8 dB with frequency hopping interval of 16

Observation #6: For CE Mode B Test2 in FDD, the SNR corresponding to 1% missed downlink scheduling grant is at -17.1 dB with frequency hopping interval of 8

Observation #7: For CE Mode B Test3 in FDD, the SNR corresponding to 1% missed downlink scheduling grant is at -15.8 dB with frequency hopping interval of 4

Observation #8: For CE Mode B Test3 in FDD, the SNR corresponding to 1% missed downlink scheduling grant is at -16.1 dB with frequency hopping interval of 2

Observation #9: For CE Mode B Test2 in TDD, the SNR corresponding to 1% missed downlink scheduling grant is at -14.7 dB with frequency hopping interval of 10

Observation #10: For CE Mode B Test2 in TDD, the SNR corresponding to 1% missed downlink scheduling grant is at -15.3 dB with frequency hopping interval of 5

Observation #11: For CE Mode B Test3 in TDD, the SNR corresponding to 1% missed downlink scheduling grant is at -16.1 dB with frequency hopping interval of 5

Proposal #1: For CE Mode B Test2 in FDD set the frequency hopping interval to 16

Proposal #2: For CE Mode B Test3 in FDD set the frequency hopping interval to 4

Proposal #3: For CE Mode B Test2 in TDD set the frequency hopping interval to 5

# References

- R4-1706196,  “WF on requirements for Rel-13 Non-BL UE”, Huawei, HiSilicon, Ericsson, Intel Corporation, Qualcomm

- R4-1708690, “Way forward on R13 non-BL UE demodulation requirements”, Huawei, HiSilicon

- R4-1709104, “Requirements alignment for  eMTC Non-BL UE demodulation”, Huawei, HiSilicon

- R4-1709040, “CR for requirements of MPDCCH with 2Rx and 4Rx”, Huawei, HiSilicon

# Appendix A

The simulation assumptions for MPDCCH captured in [4] for FDD and half-duplex FDD are listed in the table below:

Table 1: Test Parameters for MPDCCH (Category ≥1)

| Parameter |  | Unit | CE Mode A (Test 2) | CE Mode A (Test 3) | CE Mode B (Test 2) | CE Mode B (Test 3) |
| --- | --- | --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 | 2 | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 | 0 | 0 |
| Downlink power allocation |  | dB | -3 | 0 | 0 | 0 |
|  |  | dB | -3 | 0 | 0 | 0 |
|  |  | dB | 0 | -3 | -3 | -3 |
|  | δ | dB | 3 | 0 | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 | 1 | 1 | 1 |
|  |  | ms | 4(Note 2) | 4(Note 2) | 16 (Note 2) | 16 (Note 2) |
| Beamforming Pre-Coder |  |  | Annex B.4.4 | Annex B.4.4 | Annex B.4.4 | Annex B.4.4 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 | Port 0 and 1 | Port 0 and 1 | Port 0 and 1 |
| Number of PRB per MPDCCH Set |  |  | 4 | 4 | 2+4 | 2+4 |
| Transmission type |  |  | Distributed | Distributed | Localized | Localized |
| Frequency hopping |  |  | Disabled | Disabled | Enabled | Enabled |
| Number of frequency hopping narrowbands |  |  | N/A | N/A | 4 | 4 |
| Frequency hopping offset |  |  | N/A | N/A | 1 | 1 |
| Frequency hopping interval |  | ms | [4] | 1 | [16 / 8] | [4 / 2] |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) Note 3 |  |  | 1.5 | 1.5 | 1.5 | 1.5 |
| Maximum number of repetitions |  |  | [8] | [2] | [32] | [8] |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 | 1 | 7 | 7 |
| PDSCH TM |  |  | TM2 | TM2 | TM2 | TM2 |
| Reference channel for PDSCH transmission |  |  | R.81 FDD | R.81 FDD | R.81 FDD | R.81 FDD |
| DCI Format |  |  | 6-1A | 6-1A | 6-1B | 6-1B |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1111111111 | 1111111111 | 1111111111 | 1111111111 |
| Note 1: For each test, DC subcarrier puncturing shall be considered. Note 2: Same precoding matrix is used for a PRB across subframes during the frequency hopping interval Note 3: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. |  |  |  |  |  |  |

The simulation assumptions for MPDCCH captured in [4] for TDD are listed in the table below:

Table 2: Test Parameters for MPDCCH (Category ≥1)

| Parameter |  | Unit | CE Mode A (Test 2) | CE Mode A (Test 3) | CE Mode B (Test 2) | CE Mode B (Test 3) |
| --- | --- | --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 | 2 | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 | 0 | 0 |
| Downlink power allocation |  | -3 | -3 | -3 | 0 | 0 |
|  |  | -3 | -3 | -3 | 0 | 0 |
|  |  | 0 | 0 | 0 | -3 | -3 |
|  | δ | 3 | 3 | 3 | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 | 1 | 1 | 1 |
|  |  | ms | 5 (Note 2) | 5 (Note 2) | 20 (Note 2) | 20 (Note 2) |
| Beamforming Pre-Coder |  |  | Annex B.4.4 | Annex B.4.4 | Annex B.4.4 | Annex B.4.4 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 | Port 0 and 1 | Port 0 and 1 | Port 0 and 1 |
| Number of PRB per MPDCCH Set |  |  | 4 | 4 | 2+4 | 2+4 |
| Transmission type |  |  | Distributed | Distributed | Localized | Localized |
| Frequency hopping |  |  | Disabled | Disabled | Enabled | Enabled |
| Number of frequency hopping narrowbands |  |  | N/A | N/A | 4 | 4 |
| Frequency hopping offset |  |  | N/A | N/A | 1 | 1 |
| Frequency hopping interval |  | ms | 1 | 1 | [10 / 5] | [5] |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) Note 3 |  |  | 5 | 5 | 5 | 5 |
| Maximum number of repetitions |  |  | [4] | [2] | [16] | [8] |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 | 1 | 7 | 7 |
| PDSCH TM |  |  | TM2 | TM2 | TM2 | TM2 |
| Reference channel for PDSCH transmission |  |  | R.81 TDD | R.81 TDD | R.81 TDD | R.81 TDD |
| DCI Format |  |  | 6-1A | 6-1A | 6-1B | 6-1B |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1000010000 | 1000010000 | 1000010000 | 1000010000 |
| Note 1: For each test, DC subcarrier puncturing shall be considered. Note 2: Same precoding matrix is used for a PRB across subframes during the frequency hopping interval Note 3: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. |  |  |  |  |  |  |