3GPP TSG-RAN WG4 Meeting #84	R4-1710658

Berlin, Germany, 21 – 25 Aug, 2017

Source:	MediaTek Inc

Title:	Correction TP for TR36.715-05-01: the support of CA_1A-3C-7A-20A_1UL_BCS0

Agenda item:	8.5.2

Document for:	Approval

1.  Introduction

CA_1A-3C-7A-20A_1UL_BCS0 was approved in 5DL/1UL basket WID in the RAN-P#76 meeting [1]. TP for completion of the CA has been provided[2]. This paper is to correct error in tables in [2] for Table 5.x.1-1 Maximum aggregated BW.

2.  Reference

[1]      RP-171466 LTE Advanced inter-band CA Rel-15 for 5DL/1UL, RAN #76.

[2]      R4-1707250  TP_1A-3C-7A-20A_1UL_BCS0, RAN4#84

Annex: TP for TR36. 715-05-01 (va.b.c)

[Unchanged portions skipped]

CA_1A-3C-7A-20A_1UL_BCS0

5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-3C-7A-20A | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  | 3 | See CA_3C in Table 5.6A.1-1 of 36.101 Bandwidth combination set 0 |  |  |  |  |  |  |  |
|  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"