3GPP TSG-RAN WG4 Meeting #84-Bis	R4-1710659

Dubrovnik, HR, 9 – 13 Oct, 2017

Source:	MediaTek Inc.

Title:	TP for TR36.715-02-01: the support of CA_4A-71A_1UL_BCS0

Agenda item:	8.2.2

Document for:	Approval

1.  Introduction

CA_4A-71A_1UL was approved in 2DL/1UL basket WID in the RAN-P#77 meeting [1]. This paper is to provide TP for the completion of this combination.

2.  Reference

- RP-171790   LTE Advanced inter-band CA Rel-15 for 2DL/1UL

Annex: TP for TR36. 715-02-01 (va.b.c)

[Unchanged portions skipped]

CA_4A-71A_1UL_BCS0

5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_4A-71A | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 71 |  |  | Yes | Yes | Yes | Yes |  |  |

5.x.2	Co-existence studies

The 2nd and 3rd order harmonics caused in the UE by transmitting of Band 4 and Band 71 UL carriers can be calculated as shown in Table 5.x.2-1 below. None of the products falls into the B4 or B71 UE receiving band. For UE coexistence the single UL is already considered for these bands in TS 36.101.

Table 5.x.2-1: Band 4 and Band 71 UL harmonics

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 4 | 1710 | 1755 | 2110 | 2155 | 3420 | 3510 | 5130 | 5265 |
| 71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 |

### 5.x.3	ΔTIB and ΔRIB values

CA_4A-71A is formed by diplexing a carrier in Band 4 with a carrier in Band 71.  No harmonic terms overlap with downlink in Band 4 from uplink in Band 71.  The diplexer relaxations TIB = 0.3 dB, RIB = 0 dB apply with no MSD or reference sensitivity exceptions.

Table 5.x.3-1: ΔTIB,c (two bands)

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_4A-71A | 4 | 0.3 |
|  | 71 | 0.3 |

Table 5.x.3-2: ΔRIB,c (two bands)

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_4A-71A | 4 | 0 |
|  | 71 | 0 |

5.x.4	REFSENS requirements

REFSENS remain unchanged.

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"