3GPP TSG-RAN WG4 Meeting #84bis	R4-1710787

Dubrovnik, Croatia, 9 – 13 October 2017

Agenda Item:	5.4.5.2

Source:	Ericsson

Title:	Simulation results of MPDCCH for non-BL CE UE

Document for:	Discussion

# 1	Introduction

RAN4#84 agreed with parameter setting of MPDCCH for Rel-13 Non-BL CE UEs.

This contribution presents our simulation results for MPDCCH considering 2Rx/4Rx UE supporting coverage enhancements.

# 2	Simulation results

## 2.1	CE Mode A

Table 1	Parameters for CE Mode A test.

| Test case | Duplex | Rx | Repetition | Interval | Note |
| --- | --- | --- | --- | --- | --- |
| CEModeA | FDD | 2 | 8 | 4 |  |
| CEModeA | FDD | 4 | 2 | 1 |  |
| CEModeA | TDD | 2 | 4 | 1 |  |
| CEModeA | TDD | 4 | 2 | 1 |  |

Figure 1 shows the simulation results of MPDSCH CE Mode A for FDD/TDD according to the parameters [1]. Table 1 shows the summary of the parameters. Table 2 is the required SNR values to achieve 1% MPDCCH BLER.

Figure 1	Simulation results of MPDCCH CE Mode A for 2Rx and 4Rx.

Table 2	Required SNR [dB] for 1% MPDCCH BLER.

| Duplex mode | Number of receive antennas | Ideal results | Impairment results |
| --- | --- | --- | --- |
| FDD | 2Rx | -8.8 | -6.8 |
|  | 4Rx | -9.4 | -7.4 |
| TDD | 2Rx | -7.3 | -5.3 |
|  | 4Rx | -9.4 | -7.4 |

## 2.2	CE Mode B

Table 3	Parameters for CE Mode B test.

| Test case | Duplex | Rx | Repetition | Interval | Note |
| --- | --- | --- | --- | --- | --- |
| CEModeB | FDD | 2 | 32 | 16 => 8 |  |
| CEModeB | FDD | 4 | 8 | 4 => 2 |  |
| CEModeB | TDD | 2 | 16 | 10 => 5 |  |
| CEModeB | TDD | 4 | 8 | 5 | 5 is the minimum number for TDD CEModeB. |

Figure 2 shows the simulation results of MPDSCH CE Mode A for FDD/TDD according to the parameters [1]. Table 3 shows the summary of parameters. Note we change the frequency hopping interval parameters in the simulation. This is because the interval parameter agreed in [1], can uses only 2 narrowbands although the number of narrowbands for CE Mode B uses 4. Table 4 is the required SNR values to achieve 1% MPDCCH BLER.

Figure 2	Simulation results of MPDCCH CE Mode B for 2Rx and 4Rx.

Table 4	Required SNR [dB] for 1% MPDCCH BLER.

| Duplex mode | Number of receive antennas | Ideal results | Impairment results |
| --- | --- | --- | --- |
| FDD | 2Rx | -16.8 | -14.8 |
|  | 4Rx | -16.0 | -14.0 |
| TDD | 2Rx | -15.3 | -13.3 |
|  | 4Rx | -16.0 | -14.0 |

# 3	Conclusion

Proposal 1: 	Modify the interval parameters for CE Mode B as follows:

| Test case | Duplex | Rx | Repetition | Interval | Note |
| --- | --- | --- | --- | --- | --- |
| CEModeB | FDD | 2 | 32 | 8 |  |
| CEModeB | FDD | 4 | 8 | 2 |  |
| CEModeB | TDD | 2 | 16 | 5 |  |
| CEModeB | TDD | 4 | 8 | 5 | 5 is the minimum number for TDD CEModeB. |

# 4	References

R4-1709040, “CR for requirements of MPDCCH with 2Rx and 4Rx”, Huawei, HiSilicon.