3GPP TSG-RAN WG4 Meeting #84	R4-1710801

Dubrovnik, Croatia, 9 – 13 October 2017

Agenda Item:	6.4.2.3

Source:	Ericsson

Title:	Simulation results for Advanced CSI PMI/RPI test for eFD-MIMO

Document for:	Discussion

# 1	Introduction

RAN4#84 discussed the test setup and metric for the PMI test for eFD-MIMO advanced CSI codebook [1] and agreed with the following way forward:

- Introduce advanced Codebook PMI test case with below configuration:

  - PUSCH 1-2 feedback mode, 16 CSI-RS ports with (N1, N2) = (2,4), (O1, O2) = (8, 4), CDM4, CSS configuration = 1, 2, 3, 4, EVA5Hz

- Test metric:

  - Option 1: Throughput ratio between the followed Advanced PMI/RPI and followed Advanced PMI with fixed RPI=0.

  - Option 2: Throughput ratio between the followed Advanced PMI/RPI and randomized {i1,3, i2, RPI} with the Advanced CSI. (Baseline)

- MIMO correlation matrix:

  - Option 1: High correlation as existing in specs

  - Option 2: Medium spatial correlation

|  | α1 | α2 | β | γ |
| --- | --- | --- | --- | --- |
| High spatial correlation | 0.9 | 0.9 | 0.9 | 0.3 |
| Medium spatial correlation | 0.3 | 0.3 | 0.6 | 0.2 |

- MCS and Rank:

  - Option 1: 16QAM 1/2 Rank2 (Baseline)

  - Option 2: 64QAM 1/2 Rank1

- Beam steering model: Dual cluster beam with independent MIMO channels

  - Relative power ratio: 1 (p=1)

We continue the discussion on the test setup and metric based on our simulation result.

# 2	Discussion

## 2.1	Followed-PMI/RPI simulation result

Figure 1 shows the simulation results for different conditions:

- EVA5 medium channel correlation or EVA high channel correlation

- 16QAM 1/2 rank 2 or 64QAM 1/2 rank 1

For each figure, we show the simulation results of:

- Followed Class A PMI (this means UE only report the first W1, W2 only)

- Followed Advanced CSI PMI/RPI

- Followed the first W1, randomized {i1,3, i2, RPI}

- Followed the first W1, i1,3, i2. Fixed RPI=0.

Note we set O1 is 4 because it is not possible to set O1=8 for advanced CSI precoder.

Figure 1	Simulation results of the followed Class A PMI, followed Advanced CSI PMI/PRI, followed Advanced CSI with RPI=0, and followed Advanced CSI with randomized {i1,3, i2, RPI}.

## 2.2	Metric for PMI/RPI test

Figure 2	Throughput ratios of 1) the followed Advanced CSI PMI/RPI over followed Advanced CSI with randomized {i1,3, i2, RPI} (Option 2, baseline), 2) the followed Advanced CSI PMI/RPI over followed Advanced CSI with RPI=0 (Option 1), and the followed Class A PMI over followed Advanced CSI with randomized {i1,3, i2, RPI}.

Figure 2 compares the throughput ratios:

- the followed Advanced CSI PMI/RPI over followed Advanced CSI with randomized {i1,3, i2, RPI} (Option 2, Baseline)

- the followed Advanced CSI PMI/RPI over followed Advanced CSI with RPI=0 (Option 1)

- the followed Class A PMI over followed Advanced CSI with randomized {i1,3, i2, RPI}

The first ratio and second ratio is the possible metrics in the way forward. The third one is the ratio when UE only reports Class A codebook for reference.

When comparing 64QAM and 16QAM, we observe from Figure 1 that the performance gain of the Followed Advanced CSI PMI/RPI with 64QAM is not so large compared with Class A PMI. This means UE supporting only Class A may pass the Advanced CSI PMI/PRI test if 1) is chosen as a metric. Even if we choose 2) as the metric, the throughput ratio is small (<1.2).

For 16QAM case, it is observed that the performance gain of the Followed Advanced CSI PMI/PRI over Followed Class A PMI for medium channel correlation is larger than that for high correlation matrix. We therefore propose to choose 16QAM rank 2 and medium channel correlation matrix.

Proposal 1: For Advanced CSI PMI/RPI test, use 16QAM 1/2 rank 2 for MCS.

Proposal 2: For Advanced CSI PMI/RPI test, use the medium channel correlation matrix.

For metric, if we look 16QAM with medium channel correlation, the throughput ratio at SNR=6dB (around 80% of maximum throughput with followed advanced PMI/PMI) is about 1.8 for metric 1), 1.35 for metric 2), and 1.4 for metric 3). If we adopt the 1) as metric, we should set the throughput ratio to be higher than the throughput ratio given by 3) to avoid UE supporting only Class A PMI pass the test, e.g., 1.6, according to our simulation. Otherwise, we can choose 2) for metric because this metric can secure the test for the advanced CSI PMI/RPI reporting. Our preference is still option 1 because it can avoid our concern.

Proposal 3: For the metrics (throughput ratios) of the Advanced PMI/RPI reporting test, adopt the throughput ratio between the followed Advanced PMI/RPI and followed Advanced PMI with fixed RPI=0.

# 3	Conclusion

Proposal 1: For Advanced CSI PMI/RPI test, use 16QAM 1/2 rank 2 for MCS.

Proposal 2: For Advanced CSI PMI/RPI test, use the medium channel correlation matrix.

Proposal 3: For the metrics (throughput ratios) of the Advanced PMI/RPI reporting test, adopt the throughput ratio between the followed Advanced PMI/RPI and followed Advanced PMI with fixed RPI=0.

# 4	References

R4-1708725, “Way forward on eFD-MIMO performance requirements”, Samsung, CATT, Qualcomm, Intel, Ericsson.

# Appendix

## Simulation setup (based on [1])

| ## Parameter | ## Unit | ## Test 1 (Multiple PMI test) -16ports |
| --- | --- | --- |
| ## Bandwidth | ## MHz | ## 10 |
| ## Transmission mode |  | ## 9 |
| ## Propagation channel |  | ## EVA5 |
| ## Precoding granularity ## (only for reporting and following PMI) |  | ## 6 |
| ## Correlation and antenna configuration |  | ## 2D XP 16 x 2 (N1,N2,P) =(2,4,2) ## High or Medium |
| ## Beamforming model |  | ## [Annex B.4.3] |
| ## Cell-specific reference signals |  | ## Antenna ports 0,1 |
| ## CSI reference signals |  | ## Antenna ports |
|  |  | ## 15,…,30 |
| ## Number of CSI-RS ports |  | ## 16 |
| ## CDM Type |  | ## CDM4 |
| ## CSI-RS periodicity and subframe offset TCSI-RS / ICSI-RS |  | ## 5/1 |
| ## NZP-CSI-RS-Configuration-List |  | ## {0,1,2,3} |
| ## FrequencyDensityNonPrecoded |  | ## 1 |
| ## NZP-TransmissionCombNonprecoded |  | ## 1 |
| ## eMIMO-Type |  | ## Class A |
| ## advancedCodebookEnabled |  | ## True |
| ## codebookConfig-N1 |  | ## 2 |
| ## codebookConfig-N2 |  | ## 4 |
| ## codebook-Over-Sampling-RateConfig-O1 |  | ## 4 |
| ## codebook-Over-Sampling-RateConfig-O2 |  | ## 4 |
| ## codebookSubsetRestriction-1 |  | ## 0x02/01 ## FFFF FFFF FFFF FFFF ## FFFF FFFF FFFF FFFF ## FFFF FFFF FFFF FFFF ## FFFF FFFF FFFF FFFF |
| ## codebookSubsetRestriction-2 |  | ## Codebook-Config 1: ## 0000 1111 0000 ## Codebook-Config 2,3,4: ## 0x 00 000000 FFFF 0000 |
| ## Reporting mode |  | ## PUSCH 1-2 |
| ## Reporting interval | ## ms | ## 5 |
| ## PMI delay | ## ms | ## 8 |
| ## MCS &Rank |  | ## 16QAM 1/2, Rank2 or ## 64QAM 1/2, Rank1 |
| ## Max number of HARQ transmissions |  | ## 4 |
| ## Redundancy version coding sequence |  | ## {0,1,2,3} |