3GPP TSG-RAN WG4 Meeting #84Bis	R4-1711154

Dubrovnik, Croatia, 9 – 13 October 2017

Agenda Item:	9.5.4.2.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on below 6GHz NR BS ACS

Document for:	Approval

# 1.	Introduction

Below 6GHz NR BS ACS requirements were further discussed in RAN4-NR#3, and the way forward was agreed in [1], where certain aspects are FFS.

This contribution provides a proposal to conclude on the FFS aspects to specify the below 6GHz NR BS ACS conducted requirements in the RAN4 specifications.

# 2.	Discussion

The following aspect for NR BS ACS requirements was FFS in the agreed way forward in [1]:

- For NR channel bandwidths less than or equal to 20 MHz, to specify 5 MHz as the interfering carrier bandwidth for ACS requirements.

- For NR channel bandwidths wider than 20 MHz, to specify [5 or 20] MHz as the interfering carrier bandwidth for ACS requirements.

Specifying 5 MHz as the interfering carrier bandwidth for ACS requirements for NR channel bandwidths wider than 20 MHz would reduce the number of FRCs required to be defined for below 6GHz NR BS receiver requirements. On the other hand, specifying 20 MHz as the interfering carrier bandwidth for ACS requirements for NR channel bandwidths wider than 20 MHz would better represent real-life deployment scenarios, where larger (both wanted and adjacent) channel bandwidth should be expected in wider operating bands.

In view of the above reasonings, each option has its pro and con, and the delta is minor in term of the required radio performance to fulfil the requirement. To complete the RAN4 NR work as schedule, it is proposed to decide on one option as the interfering carrier bandwidth for ACS requirements for NR channel bandwidths wider than 20 MHz based on the majority view in RAN4#84Bis.

# 3.	Conclusion and proposals

This contribution has provided proposal to conclude on the FFS aspects to specify the below 6GHz NR BS ACS conducted requirements in the RAN4 specifications.

Proposal:

To decide on one option as the interfering carrier bandwidth for ACS requirements for NR channel bandwidths wider than 20 MHz based on the majority view in RAN4#84Bis.

# References

[1]	R4-1710037, “WF on BS ACS for sub 6GHz”, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, ZTE, NEC, NTT DoCoMo.