3GPP TSG-RAN4 Meeting #84bis	R4-1711615

# Dubrovnik, HR, 9th – 13th October 2017

Title:	Draft response LS for time mask for sTTI

Response to:	Reply LS on implication of sTTI operation on UL ON/OFF time mask

Release:	Rel-15

Work Item:  	LTE_sTTIandPT-core

Source:	RAN4

To:	RAN1

CC:

Contact Person:

# Name:	Prashanth Akula

# E-mail Address:	pakula@qti.qualcomm.com

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on implication of sTTI operation on UL ON/OFF time mask. RAN4 has discussed the issues mentioned in the LS, and have following feedbacks:

- Regarding the duration of the transient period the agreements in RAN4 are:

  - For ON-OFF mask for sTTI:

    - For Tx BW>1.4 MHz, duration of transient period=10 usec

    - For Tx BW=1.4 MHz, duration of transient period =20 usec

    - The transient period shall be outside the TTI duration

  - For consecutive sTTI symbols:

    - Transient periods =10+10 usec.

    - The location for the transient period is dependent on the contents of the TTI

- There are multiple cases for consecutive TTIs as listed below:

Table 1: List of all possible cases for transient periods between consecutive symbols

Table 2: Logic to decide the location for the transient periods for each of the above cases

| Case | If condition | then |
| --- | --- | --- |
| 4 | If the last symbol of current TTI is SRS and if the first symbol on the right sTTI is ‘R’ | Place the entire transient period in ‘R’ |
|  | Else if the last symbol of current TTI is SRS and if the first symbol on the right sTTI is ‘D’ | Share the transient period symmetrically between the two symbols/TTIs |
|  | Else | Place the entire transient period in current TTI |
| 5 | For all data patterns | Treat this case similar to that of legacy 1msec+1msec TTI |

2. Actions:

ACTION: 	RAN4 kindly asks RAN1 to take above information in the work for shortened TTI and processing time for LTE.

3. Date of Next 3GPP TSG-RAN4 Meetings:

# 3GPP TSG-RAN WG4 Meeting #85		27 Nov - 1 Dec 2017		Reno, Nevada, US

# 3GPP TSG-RAN WG4 Meeting #86		26 Feb - 2 Mar 2018		Athens, GR