3GPP TSG-RAN WG4 Meeting #84bis	R4-1711761

# Dubrovnik, Croatia, 9 - 13 October, 2017

Source: 	Huawei

Title:	TP to draft TS38.104: Radiated NR BS transmit power; FR1 (9.2)

Agenda Item:	9.5.3.8.1 (Other Tx requirements, FR1)

Document for:	Approval

# 1 Introduction

In this contribution we provide TP to draft TS 38.104 [1] on the radiated NR BS transmit power for FR1 in section 9.2.

The following modifications were introduced:

- 3.1: new definitions directly related to the radiated NR BS transmit power requirement for FR1

  - New definition added for the rated beam EIRP

- 9.2: Radiated transmit power

  - Text based on the TS 37.105, section 9.2,

  - It shall be noted that the NR BS transmit power for FR2 was not discussed so far

In separate contribution in [2], a TP to TR 38.817-2 (BS RF) is provided, for the text alignment purposes among draft TS and TR.

# 2 Conclusion

Proposal 1: approve the attached TP to draft TS 38.104 [1] on the radiated NR BS transmit power requirement for FR1.

# 3 References

Draft TS 38.104, v. 0.2.0

R4-1711328		TP to TR38.xxx (BS RF): Radiated NR BS transmit power, FR1, RAN4#84bis

# TP to the draft TS 38.104

In this section, TP to the TS 38.104 [1] is proposed for approval.

- ------------------------------ Modified section ------------------------------

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

NOTE:	For certain AAS BS antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

beam peak direction: direction where the maximum EIRP is found

beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

EIRP accuracy directions set: beam peak directions for which the EIRP accuracy requirement is intended to be met

NOTE:	The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

rated beam EIRP: For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the base station is declared to radiate at the associated beam peak direction during the transmitter ON period

reference beam direction pair: declared beam direction pair, including reference beam centre direction and reference beam peak direction where the reference beam peak direction is the direction for the intended maximum EIRP within the EIRP accuracy compliance directions set

- ------------------------------ Next modified section ------------------------------

# 9		Radiated transmitter characteristics

## 9.1	General

Radiated transmitter characteristics requirements apply on the AAS BS including all its functional components active and for all foreseen modes of operation of the AAS BS unless otherwise stated.

## 9.2	Radiated transmit power

### 9.2.1	General

1-O [and 2-O] type BS is declared to support one or more beams. Radiated transmit power is defined as the EIRP level for a declared beam at a specific beam peak direction.

For each beam, the requirement is based on declaration of a beam identity, reference beam direction pair, beamwidth, rated beam EIRP, EIRP accuracy directions set, the beam direction pairs at the maximum steering directions and their associated rated beam EIRP and beamwidth(s).

For a declared beam and beam direction pair, the rated beam EIRP level is the maximum power that the base station is declared to radiate at the associated beam peak direction during the transmitter ON period.

For each beam peak direction associated with a beam direction pair within the EIRP accuracy directions set, a specific rated beam EIRP level may be claimed. Any claimed value shall be met within the accuracy requirement as described below. Rated beam EIRP is only required to be declared for the beam direction pairs subject to conformance testing as detailed in TS 38.141-2 [x].

NOTE 1:	EIRP accuracy directions set is set of beam peak directions for which the EIRP accuracy requirement is intended to be met. The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

NOTE 2: 	A beam direction pair is data set consisting of the beam centre direction and the related beam peak direction.

NOTE 3:	A declared EIRP value is a value provided by the manufacturer for verification according to the conformance specification declaration requirements, whereas a claimed EIRP value is provided by the manufacturer to the equipment user for normal operation of the equipment and is not subject to formal conformance testing.

### 9.2.2	Minimum requirement for BS type 1-H and 1-O

For each declared beam, in normal conditions, for any specific beam peak direction associated with a beam direction pair within the EIRP accuracy directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ±2.2 dB of the claimed value.

For each declared beam, in extreme conditions, for any specific beam peak direction associated with a beam direction pair within the EIRP accuracy directions set, a manufacturer claimed EIRP level in the corresponding beam peak direction shall be achievable to within ±2.7 dB of the claimed value.

Normal and extreme conditions are defined in TS 38.141-2, annex B [x].

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

### 9.2.3	Minimum requirement for BS type 2-O

FFS

## 9.3	OTA Base station output power

Detailed structure of the subclause is TBD.

------------------------------ End of modified sections ------------------------------