3GPP TSG-RAN WG4 Meeting #85                                                                  R4-1712243

Reno, NV, USA, 27 Nov – 1 Dec, 2017

Source:	Samsung, KT, [Verizon]

Title:		TP for RF requirements of TR 38.815

Agenda Item:	   9.2.1.4

Document for:	Approval

- Intoriduction

This paper summarized the status of RF requirements for FR2, the common RF requirements for 24.25-29.5GHz range and band specific requirements for Band n257 and n258. The draft TP is provided based on this summary according to current RAN4 agreement [1, 2].

- Discussion

In current RAN4 study, only 24.25-29.5GHz and 37-42.5GHz are under discussion for FR2. So Table 2-1 and Table 2-2 summarize the current status for FR2 RF requirements with categories as band agnositice requirements, common requirements for n257/n258 and band specific requirement for n257 and n258.

Table 2-1 UE RF requirements list for FR2

| Subclause in TS 38.101-2 | Requirements | Status | Band agnostic requirements for FR2 | Common requirements for Band n257 and n258 | n257 specific | n258 specific |
| --- | --- | --- | --- | --- | --- | --- |
| 6 | Transmitter characteristics |  |  |  |  |  |
| 6.1 | General |  |  |  |  |  |
| 6.2 | Transmitter power | Open |  |  | Yes | Yes |
| 6.3 | Output power dynamics |  |  |  |  |  |
| 6.3.1 | Minimum output power | Agreed | Yes |  |  |  |
| 6.4 | Transmit signal quality | Open | Yes |  |  |  |
| 6.5 | Output RF spectrum emissions |  |  |  |  |  |
| 6.5.1 | Occupied bandwidth | Agreed | Yes |  |  |  |
| 6.5.2 | Out of band emission |  |  |  |  |  |
| 6.5.2.1 | Spectrum emission mask | Agreed | Yes |  |  |  |
| 6.5.2.2 | Additional spectrum emission mask | Open |  |  | Yes | Yes |
| 6.5.2.3 | Adjacent channel leakage ratio | Agreed |  | Yes |  |  |
| 6.5.2.4 | Additional ACLR | Open |  |  |  |  |
| 7 | Receiver characteristics |  |  |  |  |  |
| 7.1 | General |  |  |  |  |  |
| 7.2 | Diversity characteristics | Open |  |  |  |  |
| 7.3 | Reference sensitivity power level | Open |  |  | Yes | Yes |
| 7.4 | Maximum input level | Agreed | Yes |  |  |  |
| 7.5 | Adjacent channel Selectivity | Agreed |  | Yes |  |  |
| 7.6 | Blocking characteristics | Open |  |  |  |  |

Table 2-1 BS RF requirements list for NR

| Subclause in TS 38.104 | Requirements | Status | Band agnostic requirements for FR2 | Common requirements for Band n257 and n258 | n257 specific | n258 specific |
| --- | --- | --- | --- | --- | --- | --- |
| 9 | Radiated transmitter characteristics |  |  |  |  |  |
| 9.1 | General |  |  |  |  |  |
| 9.2 | Radiated transmit power | Open | Yes |  |  |  |
| 9.3 | OTA Base station output power | Open | Yes |  |  |  |
| 9.4 | OTA Output power dynamics | Open | Yes |  |  |  |
| 9.5 | OTA Transmit ON/OFF power | Agreed | Yes |  |  |  |
| 9.6 | OTA Transmitted signal quality | Open | Yes |  |  |  |
| 9.7 | OTA Unwanted emissions |  |  |  |  |  |
| 9.7.1 | General |  |  |  |  |  |
| 9.7.2 | OTA Occupied bandwidth | Agreed | Yes |  |  |  |
| 9.7.3 | OTA Adjacent Channel Leakage Power Ratio (ACLR) | Agreed |  | Yes |  |  |
| 9.7.4 | OTA out of band emissions |  |  |  |  |  |
| 9.7.4.3 | Spectrum emission mask (FR2) | Agreed |  | Yes |  |  |
| 9.7.5 | OTA Transmitter spurious emissions | Agreed | Yes for basic requirement |  | FFS on additional spurious emission for EESS protection | FFS on additional spurious emission for EESS protection |
| 9.8 | OTA Transmitter intermodulation | Agreed no need for RF2 | Yes |  |  |  |
| 10 | Radiated receiver characteristics |  |  |  |  |  |
| 10.1 | General |  |  |  |  |  |
| 10.2 | OTA sensitivity | Open | Yes |  |  |  |
| 10.3 | OTA Reference sensitivity level | Open on necessity for FR2 |  |  |  |  |
| 10.4 | OTA Dynamic range | Agreed no need for RF2 | Yes |  |  |  |
| 10.5 | OTA In-band selectivity and blocking | Open |  | ACS |  |  |
| 10.6 | OTA Out-of-band blocking | Open |  |  |  |  |
| 10.7 | OTA Receiver spurious emissions | Open | Yes |  |  |  |
| 10.8 | OTA Receiver intermodulation | Agreed no need for RF2 | Yes |  |  |  |
| 10.9 | OTA In-channel selectivity | Open | Yes |  |  |  |

- Reference

[1] R4-1711049, “TR 38.817-1 v0.1.0: General aspects for UE RF for NR”, DOCOMO.

[2] R4-1711981, “TR 38.817-2 v0.3.0: General aspects for BS RF for NR”, Ericsson.

[3] R4-1708842,	“Skeleton TR 38.815”, KT.

Text proposal for TR 38.815

<Start of TP>

- Common RF requirements for band n257 and n258

According to current study in RAN4 at least RF requirements summarized in Table 7-1 should be the band agnostic RF requirements for FR2 which are applicable for band n257 and n258 as well. Besides that, the RF requirements which are band specific but common between band n257 and n258 are gathered in this section.

Table 7-1: Summary on band agnostic of RF requirements for FR2

| BS TX side capture in TS 38.104 | BS RX side capture in TS 38.104 | UE Tx side capture in TS 38.101-2 | UE Rx side capture in TS 38.101-2 |
| --- | --- | --- | --- |
| 9.2 Radiated transmit power | 10.2 OTA sensitivity | 6.3.1 Minimum output power | 7.4 Maximum input level |
| 9.3 OTA Base station output power | 10.5 OTA In-band selectivity and blocking | 6.4 Transmit signal quality |  |
| 9.4 OTA Output power dynamics | 10.7 OTA Receiver spurious emissions | 6.5.1 Occupied bandwidth |  |
| 9.5 OTA Transmit ON/OFF power | 10.9 OTA In-channel selectivity | 6.5.2.1 Spectrum emission mask:General |  |
| 9.6 OTA Transmitted signal quality |  |  |  |
| 9.7.2 OTA Occupied bandwidth |  |  |  |
| 9.7.5 OTA Transmitter spurious emissions |  |  |  |

The common RF requirements for Band n257 and n258 are listed in following sections with expected changes in TS 38.101-2 and 38.104 for UE and BS respectively.

## 7.1       UE specific

7.1.1      Transmitter characteristics

7.1.1.1     Adjacent Channel Leakage Ratio (ACLR)

For Band n257 and Band n258, and the general ACLR requirements is 17dB and more test parameters are shown in table 7.1.1.1-1:

Table 7.1.1.1-1: General ACLR requirements for Band n257 and Band n258

|  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n257, n258 | 17 dB | 17 dB | 17 dB | 17 dB |
| NR channel Measurement bandwidth | [50] MHz | [100] MHz | [200] MHz | [400] MHz |
| Adjacent channel centre frequency offset [MHz] | +50 / -50 | +100.0 / -100.0 | +200 / -200 | +400 / -400 |

7.1.2      Receiver characteristics

7.1.2.1     Adjacent Channel Selectivity (ACS)

For Band n257 and Band n258, the ACS requirements is 23dB and more parameter are shown in table 7.1.2.1-1, Table 7.1.2.1-2 and Table 7.1.2.1-3:

Table 7.1.2.1-1: Adjacent channel selectivity

|  |  | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| ACS for band n257, n258 | dB | 23 | 23 | 23 | 23 |

Table 7.1.2.1-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |
| PInterferer for band n257, n258 | dBm | REFSENS + [35.5] dB | REFSENS +[35.5]dB | REFSENS +[35.5]dB | REFSENS +[35.5]dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 | 100 / -100 | 200 / -200 | 400 / -400 |
| NOTE 1: [The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5.] NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.X.X with one sided dynamic OCNG Pattern as described in Annex A.X.X.X and set-up according to Annex C.3.1. NOTE 3: The REFSENS power level is specified in Table [7.3.1-1]. |  |  |  |  |  |

Table 7.1.2.1-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for band n257, n258 | dBm | [-46.5] | [-46.5] | [-46.5] | [-46.5] |
| PInterferer | dBm | [-25] |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 | 100 / -100 | 200 / -200 | 400 / -400 |
| NOTE 1: [The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5]. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex X.X with one sided dynamic OCNG Pattern TDD as described in Annex A.X.X.X and set-up according to Annex C.X.X. |  |  |  |  |  |

## 7.2	BS specific

7.2.1      Transmitter characteristics

7.2.1.1     Adjacent Channel Leakage Ratio (ACLR)

The BS OTA ACLR limit for Band n257 and n258 is defined based on TRP and specified ACLR1 in table 7.2.1.1-1. The OTA ACLR2 limit is not specified since the emission level at the ACLR2 range will be close to noise floor.

Table 7.2.1.1-1: Base Station OTA ACLR

| NR Channel bandwidth | BS adjacent channel centre frequency offset below the lower or above the upper Base Station RF bandwidth edge | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| [50, 100, 200, 400, …] | BWChannel | NR of same BW | NR of same BW with SCS that provides largest transmission bandwidth configuration (BWConfig) | 28 (Note 1) |
| Note 1: Applicable to bands defined within the frequency spectrum range of 24.24 – 33.4 GHz |  |  |  |  |

7.2.1.2     Spectrum emission mask

BS unwanted emissions for Band n257 and n258 shall not exceed the maximum levels specified in table 7.2.1.2-1 and table 7.2.1.2-2.

Table 7.2.1.2-1: SEM applicable for PTx ≥ 35 dBm in the frequency range 24.24 – 33.4 GHz

| Frequency offset from “edge of transmission” Δf | Limit | Measurement bandwidth |
| --- | --- | --- |
| 0 f < 10% of the total transmission bandwidth | -5 dBm | 1 MHz |
| 10% of the total transmission bandwidth f < OOB boundary | -13 dBm | 1 MHz |

Table 7.2.1.2-2: SEM applicable for PTx < 35 dBm in the frequency range 24.24 – 33.4 GHz

| Frequency offset from “edge of transmission” Δf | Emission limit | Measurement bandwidth |
| --- | --- | --- |
| 0 f < 10% of the total transmission bandwidth | Max(PTx – 40 dB, -12 dBm) | 1 MHz |
| 10% of the total transmission bandwidth f < OOB boundary | Max(PTx – 48 dB, -20 dBm) | 1 MHz |
| f ≥ OOB boundary | Spurious domain limits |  |

7.2.2      Receiver characteristics

7.2.2.1     Adjacent Channel Selectivity (ACS)

BS ACS level for Band n257 and n258 is agreed to be 24dBc.

<Editor’s note: the detailed requirement is to be updated according to RAN4 agreement.>

# 8	NR band n257 specific issues

This section captures band specific RF requirements for n257.

## 8.1	UE specific

### 8.1.1	Transmitter characteristics

8.1.1.1	Transmitter power

8.1.1.1.1            Maximum output power

<The tolerance of maximum output power will be added>

8.1.1.1.2           Maximum output power with additional requirements

<The A-MPR will be added >

8.1.1.2	Additional spectrum emission mask

<The text will be added >

8.1.1.3	Spurious emission

8.1.1.3.1            Additional spurious emission

<The requirements limits will be added to protect EESS>

8.1.1.3.2            Spurious emission band UE co-existence

<The requirements limits will be added>

### 8.1.2	Receiver characteristics

8.1.2.1	       Reference sensitivity power level

<The REFSENS value will be added>

8.1.2.2	       Out of band blocking

<The requirements limits will be added >

## 8.2	BS specific

### 8.2.1	Radiated transmitter characteristics

<Text to be added>

### 8.2.2	Radiated receiver characteristic

<Text to be added>

# 9	NR band n258 specific issues

This section captures band specific RF requirement for n258.

## 9.1	UE specific

### 9.1.1	Transmitter characteristics

9.1.1.1	Transmitter power

9.1.1.1.1            Maximum output power

<The tolerance of maximum output power will be added>

9.1.1.1.2           Maximum output power with additional requirements

<The A-MPR will be added >

9.1.1.2	Additional spectrum emission mask

<The text will be added >

9.1.1.3	Spurious emission

9.1.1.3.1            Additional spurious emission

<The requirements limits will be added to protect EESS>

9.1.1.3.2            Spurious emission band UE co-existence

<The requirements limits will be added>

### 9.1.2	Receiver characteristics

9.1.2.1	       Reference sensitivity power level

<The REFSENS value will be added>

9.1.2.2	       Out of band blocking

<The requirements limits will be added >

## 9.2	BS specific

### 9.2.1	Radiated transmitter characteristics

<Text to be added>

### 9.2.2	Radiated receiver characteristic

<Text to be added>

<End of TP>