3GPP TSG RAN WG4 Meeting #85	R4- 1712298

Reno, US, 27 November - 1 December 2017

Agenda item:	9.7.10.1

Title: 	Simulation results for Cell Detection in NR

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

# In RAN4#84 simulation assumptions for PSS/SSS detection in NR were approved [1]. In this paper we present simulation results for cell detection in NR.

# Simulation Results

The cell detection in NR includes PSS/SSS detection and SS-Block time index detection. The cell identification requirements in RAN4 can be based on PSS/SSS alone or along with corresponding SS-block index.  The SS-Block index in sub-6GHz is determined by the DMRS time index and in case of > 6GHz on DMRS time index and PBCH payload. The cell identification requirements for PSS/SSS and SS-Block index detection should include results from the same simulation assumptions. In this paper we present the cell detection delay that includes PSS/SSS detection and SS-Block index detection for sub-6GHz.

## 2.1 Simulation Assumptions

The simulation assumptions used to determine the detection delay is based on [1] and also provided below in Table 1. In this contribution we provide results for sub-6 GHz with 15 KHz and 30 KHz sub-carrier spacing.

Table 1: Simulation Assumptions for NR Cell Detection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| Carrier frequency |  | 4GHz; 30GHz | The same frequency as for Cell 1 is used |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 4GHz; | The same values as for Cell 1 are used |
| RB Utilization | % | 100 | 100 |
| Power boosting for PSS and SSS | dB | 0 | 0 |
| Power boosting for Data and Control | dB | 0 | 0 |
| Data Modulation | - | QPSK | QPSK |
| CP Length | - | Normal | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 | 1 |
| SS burst set periodicity | ms | 5 | 5 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | CP/2 |
| Relative Delay of 1st Path (asynchronous) | ms | 0 | 1.5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Es/Noc for K=1 | dB | 6.4 | 1.25 (SINR: -6dB) |
| Number of Tx antennas | - | 1 | 1 |
| (,) | - | (112, 1) | Case1: (224, 2) Case2: (224, 1) |
| Propagation Condition | - | AWGN, EPA5, ETU30 TDL-A/B/C with 100ns for 4 GHz Note1 |  |
| Noc Model | - | AWGN noise |  |
| NOTE 1: The channel models of CDL and TDL are used for simplified link level evaluations, which are defined in TR 38.901. |  |  |  |

Table 2: UE-specific Parameters for Cell Detection

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 and Cell 2 by the UE | Yes |
| False detect threshold | Required as in a real UE implementation |
| UE having apriority knowledge of system being synchronous or asynchronous | No |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence IDs. |
| CP length detection | Not simulated |
| Receive antennas | 2 (uncorrelated, Without receiver beamforming) |

Table 3: Cell ID Combinations simulated

| Case # | Cell 1 (Interferer Cell) |  | Cell 2 (Desired Cell) |  | Scenario |
| --- | --- | --- | --- | --- | --- |
| 1 | psc1 | ssc1 | psc2 | ssc2 | Synchronous |
| 2 | psc1 | ssc1 | psc1 | ssc3 | Synchronous |

Table 4: Example PSS, SSS indices simulated

| PSS Indices |  | SSS Indices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Label | Code index | Label | Code index (m0, m1) | Cell ID | (,) |
| psc1 | 43 | ssc1 | (20,0) | 337 | (112, 1) |
| psc2 | 86 | ssc2 | (40, 0) | 674 | (224, 2) |
|  |  | ssc3 | (35,0) | 673 | (224, 1) |

## 2.2 PSS/SSS Detection

For NR-PSS/SSS acquisition time, the detection is claimed to be successful only when both the PSS and SSS are correctly detected and the timing offset is within half CP length. The CDF curves for joint PSS/SSS detection delay in terms of number of SS-Blocks are shown below.

Figure 1: PSS/SSS Detection for 15 KHz SCS

Figure 2: PSS/SSS Detection for 30 KHz SCS

Table 5 captures the detection time for 90th percentile successful identification of the desired cell with 2Rx antenna.

Table 5: PSS/SSS detection time

| SCS |  | AWGN | EPA5 | ETU30 | TDL-C |
| --- | --- | --- | --- | --- | --- |
| 15KHz | Case 1 | 1 | 2 | 2 | 2 |
|  | Case 2 | 1 | 2 | 3 | 2 |
| 30KHz | Case 1 | 1 | 2 | 2 | 2 |
|  | Case 2 | 1 | 2 | 2 | 2 |

## 2.3 SS-Block Time Index Detection

The SSB Index detection for sub-6 GHz is based only on the DMRS time index detection. The SSB Index detection results are assuming perfect PSS/SSS detection. The plots below show the SSB index detection rate for different cases.

Figure 3: DMRS Time Index Detection for 15 KHz SCS

Figure 4: DMRS Time Index Detection for 30 KHz SCS

Table 6 below captures the number of SS-Blocks required to detect DMRS time index with 99% accuracy with 2Rx antenna.

Table 6: SSB Index detection time

| SCS |  | AWGN | EPA5 | ETU30 | TDL-C |
| --- | --- | --- | --- | --- | --- |
| 15KHz | Case 1 | 1 | 2 | 2 | 2 |
|  | Case 2 | 1 | 2 | 2 | 2 |
| 30KHz | Case 1 | 1 | 2 | 2 | 2 |
|  | Case 2 | 1 | 2 | 2 | 2 |

# Observations

PSS/SSS Detection: For the baseline case of 2 Rx antenna, up to 3 SS-Blocks are required to detect PSS/SSS with 90% success rate under all scenarios for 4GHz.  Also, the performance with different SCS for 4GHz is comparable.

SSB Index Detection: For the baseline case of 2 Rx antenna, up to 2 SS-Blocks are required to detect SSB Index with 99% accuracy under all scenarios for 4GHz. Also, the performance with different SCS for 4GHz is comparable.

# Conclusion

In this contribution the cell detection time for PSS/SSS detection and SSB Index detection are presented under the same conditions. We suggest that the detection latency presented here are considered in defining cell identification requirements in NR.

# References

[1]	R4-1708698, Updated link level simulation assumption of cell detection in NR, Huawei, HiSilicon, NTT DOCOMO, INC.