3GPP TSG-RAN WG4 85	R4-1712339

Reno, Nevada, 27 November – 1 December, 2017

# Source:	Intel Corporation

# Title:	TP for TS 38.101-2: channel bandwidth

# Agenda Item:	9.3.1

# Document for:	Approval

# Background

This contribution provides the text proposal for channel bandwidth and subcarrier spacing requirement for TS 38.101-2.

Based on the current agreement [1], channel bandwidth with subcarrier spacing could be agreeable. Depending on RAN4 progress, it should be added other supported channel bandwidth.

# References

[1] R4-1706982, “WF on band specific UE channel bandwidth”, NTT Docomo, RAN4-NR#2, Qingdao, June 2017

# Text Proposal

<Start of Text Poposal for TR 38.101-2>

<Start of Change 1>

## 5.3	Channel bandwidth

In FR2, 60 kHz and 120 kHz SCS for Data shall be supported. With 60 kHz SCS, up to 100 MHz channel bandwidth shall be supported and with 120 kHz SCS, up to 400 MHz channel bandwidth shall be supported. For both 60 kHz and 120 kHz SCS, 50 MHz channel bandwidth shall be supported as minimum channel bandwidth. Supported channel bandwidth for each SCS is band specific, and detailed information for each of bands is specified the Table 5.3-1.

Table 5.3-1: Supported channel bandwidth

| Operating band | Data SCS (kHz) / Channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Data SCS = 60 kHz |  | Data SCS = 120 kHz |  |  |
| n257 | 50 | 100 | 50 | 100 | 400 |
| n258 | 50 | 100 | 50 | 100 | 400 |
| n260 | 50 | 100 | 50 | 100 | 400 |

<End of Change 1>

<Start of Change 2>

Table 6.3.2-1: Transmit OFF power

| Operating band | Transmit OFF power (dBm) in measurement bandwidth / Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz |  |  | 400 MHz |
| nX, nY | [-25] |  |  |  |  |
|  | [45 MHz] |  |  |  |  |

<End of Change 2>

<Start of Change 3>

Table 6.6.1-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz |  | 400 MHz |  |
| Channel bandwidth (MHz) | 50 | 100 |  | 400 |

<End of Change 3>

<Start of Change 4>

Table 6.6.2.1.1-1: General NR spectrum emission mask for Range 2.

| ΔfOOB (MHz) | 50 MHz | 100 MHz |  | 400 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
| 0-5 | -5 | -5 |  | -5 | 1 MHz |
| 5-10 | -13 | -5 |  | -5 | 1 MHz |
| 10-20 | -13 | -13 |  | -5 | 1 MHz |
| 20-40 | -13 | -13 |  | -5 | 1 MHz |
| 40-125 | -13 | -13 |  | -13 | 1 MHz |
| 125-250 |  | -13 |  | -13 | 1 MHz |
| 250-500 |  |  |  | -13 | 1 MHz |
| 500-1000 |  |  |  | -13 | 1 MHz |

<End of Change 4>

<Start of Change 5>

Table 6.6.2.3.1-1: General requirements for NRACLR

|  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz |  | 400 MHz |
| NRACLR for band n257, n258 | 17 dB | 17 dB |  | 17 dB |
| NRACLR for band n260 | 16 dB | 16 dB |  | 16 dB |
| NR channel Measurement bandwidth | TBD MHz | TBD MHz |  | TBD MHz |
| Adjacent channel centre frequency offset [MHz] | +50 / -50 | +100.0 / -100.0 |  | +400 / -400 |

<End of Change 5>

<Start of Change 6>

Table 7.3.1-1: Reference sensitivity

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz |  |  | 400 MHz |
| nX |  |  |  |  |  |

<End of Change 6>

<Start of Change 7>

Table 7.4.1-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz |  | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -25 (NOTE 2) |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.X-X with PCMAX_L as defined in subclause X.X.X. NOTE 2: Reference measurement channel is specified in Annex A.X.X: [QPSK, R=X/X] variant with one sided dynamic OCNG Pattern as described in Annex XXXX. |  |  |  |  |  |

<End of Change 7>

<Start of Change 8>

Table 7.5.1-1: Adjacent channel selectivity

|  |  | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 50 MHz | 100 MHz |  | 400 MHz |
| ACS for band n257, n258 | dB | 23 | 23 |  | 23 |
| ACS for band n260 | dB | 22 | 22 |  | 22 |

Table 7.5.1-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz |  | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |
| PInterferer for band n257, n258 | dBm | REFSENS + [35.5] dB | REFSENS +[35.5]dB |  | REFSENS +[35.5]dB |
| PInterferer for band n260 | dBm | REFSENS + [34.5] dB | REFSENS +[34.5]dB |  | REFSENS +[34.5]dB |
| BWInterferer | MHz | 50 | 100 |  | 400 |
| FInterferer (offset) | MHz | 50 / -50 | 100 / -100 |  | 400 / -400 |
| NOTE 1: [The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5.] NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.X.X with one sided dynamic OCNG Pattern as described in Annex A.X.X.X and set-up according to Annex C.3.1. NOTE 3: The REFSENS power level is specified in Table [7.3.1-1]. |  |  |  |  |  |

Table 7.5.1-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz |  | 400 MHz |
| Power in Transmission Bandwidth Configuration for band n257, n258 | dBm | [-46.5] | [-46.5] |  | [-46.5] |
| Power in Transmission Bandwidth Configuration for band n260 | dBm | [-45.5] | [-45.5] |  | [-45.5] |
| PInterferer | dBm | [-25] |  |  |  |
| BWInterferer | MHz | 50 | 100 |  | 400 |
| FInterferer (offset) | MHz | 50 / -50 | 100 / -100 |  | 400 / -400 |
| NOTE 1: [The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5]. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex X.X with one sided dynamic OCNG Pattern TDD as described in Annex A.X.X.X and set-up according to Annex C.X.X. |  |  |  |  |  |

<End of Change 8>

<Start of Change 9>

Table 7.6.1.1-1: In band blocking minimum requirements

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz |  | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14dB |  |  |  |
| BWInterferer | MHz | 50 | 100 |  | 400 |
| PInterferer for bands n257, n258 | dBm | REFSENS + 42 | REFSENS + 42 |  | REFSENS + 42 |
| PInterferer for bands n260 | dBm | REFSENS + 41 | REFSENS + 41 |  | REFSENS + 41 |
| FIoffset | MHz | 100 / -100 | 200 / -200 |  | 800 / -800 |
| FInterferer | MHz | FDL_low – 75 to FDL_low + 75 | FDL_low – 150 to FDL_low + 150 |  | FDL_low – 600 to FDL_low + 600 |
| NOTE 1: [The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5.] NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.X.X with one sided dynamic OCNG Pattern as described in Annex A.X.X.X and set-up according to Annex C.X.X. NOTE 3: The REFSENS power level is specified in Table 7.X.X.X. NOTE 4: The wanted signal consists of the reference measurement channel specified in Annex A.X.X (QPSK, R=X/X) with one sided dynamic OCNG pattern as described in Annex A.X.X.X and set-up according to Annex C.X.X |  |  |  |  |  |

<End of Change 9>

<End of Text Poposal>