3GPP TSG-RAN WG4 Meeting #84-bis	R4-1712369

# Dubrovnik, Croatia, 9 - 13 October, 2017

Source:	KDDI Corporation

Title:	TP for TR36.715-04-04: General and BS requirements for UL CA more than 2CCs

Agenda item:	8.15.1 – UE RF

Document for:	Approval

Introduction

During TSG RAN#77, new work item for up to 4UL CA with Band 41 and Band 42 was approved [1].  TR number for this work item was assigned as TR36.715-04-04 and ver 0.0.1 of draft TR skeleton was approved in RAN4#84-bis [2].  This contribution intends to capture UE RF requirements for UL CA more than 2CCs into TR.

Proposal

It is proposed to approve TP described in section 4.

Reference

- [1]	RP-172030, “New WID on LTE Advanced Inter-band Carrier Aggregation (4DL/4UL) of Band 41C and Band 42C”, KDDI, et al.

- [2] R4-1710274, “Draft skeleton: TR36.715-04-04”, KDDI

TP for TR36.715-04-04

<Start of TP>

# 6	Inter-band carrier aggregation up to 4 component carriers with Band 41 and Band 42

## 6.1	General

## 6.1.1	Operating bands for CA

# Table 6.1.1-1: Inter-band CA operating bands for CA_41-42

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_41-42 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

6.1.2	Channel bandwidth per operating band for CA

In this technical report, three CA configurations are handled as described in sub-clause 5.1.  Table 6.1.2-1 shows CA combinations and channel bandwidth combination set for each CA configuration.  For band width combination set for CA_41C or CA_42C, configurations in Table 6.1.2-2 apply.

# Table 6.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_41A-42C, CA_41C-42A and CA_41C-42C

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_41A-42C | CA_41A-42A, CA_41A-42C | 41 | See CA_41C Bandwidth Combination Set 0 in Table 6.1.2-2 |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |
| CA_41C-42A | CA_41A-42A, CA_41C-42A | 41 |  |  |  | Yes | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 6.1.2-2 |  |  |  |  |  |  |  |
| CA_41C-42C | CA_41A-42A, CA_41C, CA_42C, CA_41C-42C | 41 | See CA_41C Bandwidth Combination Set 0 in Table 6.1.2-2 |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 6.1.2-2 |  |  |  |  |  |  |  |

Table 6.1.2-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_41C and CA_42C

| E-UTRA CA Configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_41C | CA_41C | 10 | 20 | 40 | 0 |
|  |  | 15 | 15, 20 |  |  |
|  |  | 20 | 10, 15, 20 |  |  |
|  |  | 5, 10 | 20 | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |
|  |  | 10 | 15, 20 | 40 | 2 |
|  |  | 15 | 10, 15, 20 |  |  |
|  |  | 20 | 10, 15, 20 |  |  |
|  |  | 10 | 20 | 40 | 3 |
|  |  | 20 | 20 |  |  |
| CA_42C | CA_42C | 5, 10, 15, 20 | 20 | 40 | 0 |
|  |  | 20 | 5, 10, 15 |  |  |
|  |  | 10, 15, 20 | 20 | 40 | 1 |
|  |  | 20 | 10, 15 |  |  |

## 6.2	BS characteristic

## 6.2.1	Co-existence studies

Since both Band 41 and Band 42 are operating bands with unpaired spectra, it is expected that results for co-existence studies of BS transmitter would be completely identical with those of UE transmitter, which is described in sub-clause 6.3.1 of this TR.  Hence, it is recommended to refer the sub-clause.

## 6.2.2	BS RF impact

Since current version of TS36.104 has already specified requirements for inter-band carrier aggregation of Band 41 and Band 42, it is possible to interpret that 3UL or 4UL inter-band CA by operating E-UTRA bands 41 and 42 is supported from the specification point of view.  Hence, it is expected that no modification is necessary into TS36.104 for supporting feature which is reported in this technical report.

<End of TP>