3GPP TSG-RAN WG4 meeting #85	R4-1712402
Reno, NV, USA, 27 Nov – 1 Dec, 2017

Source:	T-Mobile USA

Title:	TP to 38.101-1: N71 specific changes to section 7

Agenda item:	9.4.1.1

Document for:	Approval

## Background

This contribution contains the N71 specific changes for 38.101-1 section 7.  Except for the REFSENS requirements the N71 changes in this section mirror the band 71 requirements found in TS36.101.

## 2. Proposal

It is proposed that the attached text proposal is included in TS 38.101-1.

## References

- RP-172115 Revision of WI: New Radio Access Technology

TEXT PROPOSAL:

< start of text proposal >

## 7.2	Diversity characteristics

Detailed structure of the subclause is TBD.

## 7.3	Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3-1 and Table 7.3-2

Table 7.3-1: Reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n71 | 15 | [-97.2] | [ -94.2] | [-92.0] | [-87.5] | n/a | n/a | n/a | - |  |  | FDD |
|  | 30 | FFS | FFS | FFS | FFS | n/a | n/a | n/a | n/a | n/a | n/a |  |
|  | 60 | - | FFS | FFS | FFS | n/a | n/a | n/a | n/a | n/a | n/a |  |
|  | 15 |  |  |  |  |  |  |  | - |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 | - |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  |  |  |  | - |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 | - |  |  |  |  |  |  |  |  |  |  |
| [NOTE 1: Four Rx antenna ports shall be the baseline for this operating band.] NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n71 | 15 | 25 | 251 | 201 | 201 | n/a | n/a | n/a | - |  |  |  |
|  |  | 30 |  | FFS |  | FFS |  | FFS |  | FFS |  | n/a |  | n/a | n/a |  | n/a |  | n/a |  | n/a |  |  |  |
|  |  | 60 |  | - |  | FFS |  | FFS |  | FFS |  | n/a |  | n/a | n/a |  | n/a |  | n/a |  | n/a |  |  |  |
|  | 15 |  |  |  |  |  |  |  | - |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 | - |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  |  |  |  | - |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 | - |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |  |  |  |  |  |  |  |

Unless given by Table 7.3-4, the minimum requirements specified in Tables 7.3-1, 7.3-2 and 7.3-3 shall be verified with the network signalling value NS_01 (Table 6.2.3-1) configured.

Table 7.3-4: Network signaling value for reference sensitivity

| Operating band | Network Signalling value |
| --- | --- |
| 71 | 35 |

For the UE which supports inter-band carrier aggregation, the minimum requirement for reference sensitivity in Table 7.3-1 and Table 7.3-2 shall be increased by the amount given in ΔRIB,c defined in subclause 7.3.1 for the applicable operating bands.

## << Unchanged sections omitted >>

## 7.6	Blocking characteristics

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

### 7.6.1	Inband Blocking

In-band blocking (IBB)is defined for an unwanted interfering signal falling into the UE receive band or into an offset region (defined in Table 7.6.1-2 and Table 7.6.2-2) below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.  The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.1-1 and 7.6.1-2. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.1-1: Inband Blocking Parameters

| NR band | Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| n1, n3, n7, n8, n20, n28, n41, n70,n71 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |
|  |  |  | 6 | 6 | 7 | 9 | 10 | 12 | 13 | 14 |  |  |
|  | BWInterferer | MHz | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |
|  | FIoffset, case 1 SCS=15kHz |  | 7.5+ 0.0025 | 7.5+ 0.0025 | 7.5+ 0.0075 | 7.5+ 0.0025 | 7.5+ 0.0025 | 7.5+ 0.0025 | NA | NA |  |  |
|  | FIoffset, case 1 SCS=30kHz |  | NA | 7.5+ 0.005 | 7.5+ 0.015 | 7.5+ 0.005 | 7.5+ 0.005 | 7.5+ 0.005 | 7.5+ 0.005 | 7.5+ 0.015 |  |  |
|  | FIoffset, case 1 SCS=60kHz |  | NA | 7.5+ 0.010 | 7.5+ 0.030 | 7.5+ 0.010 | 7.5+ 0.010 | 7.5+ 0.010 | 7.5+ 0.010 | 7.5+ 0.030 |  |  |
|  | FIoffset, case 2 SCS=15kHz |  | 12.5+ 0.0075 | 12.5+ 0.0025 | 12.5+ 0.0025 | 12.5+ 0.0075 | 12.5+ 0.0025 | 12.5+ 0.0025 | NA | NA |  |  |
|  | FIoffset, case 2 SCS=30kHz |  | NA | 12.5+ 0.005 | 12.5+ 0.005 | 12.5+ 0.015 | 12.5+ 0.005 | 12.5+ 0.005 | 12.5+ 0.015 | 12.5+ 0.005 |  |  |
|  | FIoffset, case 2 SCS=60kHz |  | NA | 12.5+ 0.010 | 12.5+ 0.010 | 12.5+ 0.030 | 12.5+ 0.010 | 12.5+ 0.010 | 12.5+ 0.030 | 12.5+ 0.010 |  |  |
|  | Case 3-6 if needed |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in TBD with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in TBD NOTE 3: The REFSENS power level is specified in TBD for two and four antenna ports, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.6.1-2: Inband Blocking

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 | Case 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |  |  | TDB |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |  |  |  |
| n1, n3, n7, n8, n20, n28, n41, n70 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |  |  |  |
| 71 | FInterferer | MHz | (Note 2) | FDL_low – 12 to FDL_high + 15 |  |  | FDL-low - 12 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

### 7.6.3	Narrow Band Blocking

This requirement is measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The relative throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.3.1-1. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.3.2-1: Narrow Band Blocking

| NR band | Parameter | Unit | Channel Bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| 71 | Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 | 16 | 16 | 16 | 16 |  |  |
|  | Puw (CW) | dBm | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 |  |  |
|  | Fuw (offset SCS= 15 kHz) | MHz | 2.7075 | 5.2125 | 7.7025 | 10.2075 | TBD | TDB | NA | NA |  |  |
|  | Fuw (offset SCS= 30 kHz) | MHz | NA | NA | NA | NA |  |  | TBD | TBD |  |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.1-1 and Table 7.3.1-1a for two and four antenna ports, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |

< end of text proposal >