3GPP TSG-RAN WG4 Meeting #85	R4-1712427

Reno, Nevada, USA, 27 November – 1 December 2017

Source:	Ericsson

Title:	TP to TR 36.715-03-01: CA_3DL_3C-38A_1UL_BCS0_1UL_BCS0

Agenda item:	8.3.2

Document for:	Approval

# 1. Introduction

This contribution provides a TP for TR 36.715-03-01 to include the requirements for the 3DL/1UL CA combination CA_3DL_3C-38A_1UL_BCS0_1UL_BCS0 as defined in RP-171749 [1].

# 2. Text Proposal for TR 36.715-03-01

##### ---Start of changes---

## 5.x	CA_3DL_3C-38A_1UL_BCS0_1UL_BCS0

### 5.x.1	Operating bands for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3-3-38 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

### 5.x.2	Channel bandwidths per operating band for CA

Table 5.x.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_3C-38A | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  | 60 | 0 |
|  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.x.3	Co-existence studies

Table 5.x.3-1 gives the harmonic frequency limits for Band 3 and Band 38 3DL CA. As shown in the table, UL carrier of CA_3C-38A _BCS0 is not located at 2nd order harmonics or 3rd order harmonics of DL carriers.

# Table 5.x.3-1: DL harmonics frequency limits for CA of Band 3 and Band 38

| DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1805 | 1880 | 2570 | 2620 |
| 2nd order DL harmonics frequency range (MHz) | 3610 to 3760 |  | 5140 to 5240 |  |
| 3rd order DL harmonics frequency range (MHz) | 5415 to 5640 |  | 7710 to 7860 |  |

As shown in Table 5.x.3-2, the harmonic frequencies of Band 3 and Band 38 in UL are away from the receive bands of interest in the DL and we can conclude that there is no issue on UL harmonic interference.

Table 5.x.3-2: UL harmonics frequency limits for CA of Band 3 and Band 38

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 |
| 38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 |

### 5.x.4	ΔTIB,c and ΔRIB,c values

Table 5.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3C-38A | 3 | 0.5 |
|  | 38 | 0.5 |

Table 5.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3C-38A | 3 | 0 |
|  | 38 | 0 |

### 5.x.5	 MSD

No MSD values need to be defined for this combination.

---End of changes---

# Reference

[1]	RP-171749, “Revised WID: LTE Advanced inter-band CA Rel-15 for 3DL/1UL”, RAN#77, Huawei, HiSilicon